3GPP TSG-SA3 Meeting #108e-AdHoc 	draft_S3-222626-r2

e-meeting, 10 - 14 October 2022

Title:	LS on the data and analytics exchange between two NWDAFs in different PLMNs

Response to:	LS S2-2207142 on LS OUT to GSMA on the data and analytics exchange between two NWDAFs in different PLMNs from 3GPP SA WG2

Release:	18

Work Item:	FS_eNA_Ph3

Source:	SA3 Meeting #108e-AdHoc

To:	3GPP SA WG2

Cc:	GSMA

Contact person:	German Peinado

german.peinado@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 would like to thank SA2 for their LS S2-2207142 on data and analytics exchange between two NWDAFs in different PLMNs in the context of the Rel-18 TR 23.700-81.

SA3's TR 33.738 addresses this issue in KI#1. The study has not been concluded yet.

Question 1:  Does SA3 have feedback to the security aspects outlined in the key issue description? For example, are there any privacy and security protection for data and/or analytics exchange between PLMNs (VPLMN and HPLMN) already defined in SA3 or are there any privacy and security issues foreseen when exchange data and/or analytics between PLMNs?

SA3 response: KI#1 of TR 33.738 describes the identified threats and potential security requirements with respect to the key issue description of SA2. SA3 will inform SA2 when the conclusions of KI#1 are agreed on.

Question 2: Whether it is necessary to verify user consent for UE-specific data and/or analytics by the VPLMN? If so, what scenario required user consent verification? How to verify user consent in roaming scenario in VPLMN?

SA3 response:

Please note that SA3 has initiated a UC phase 2 study (TR 33.896) where the user consent in roaming scenarios is studied, including eNA (Key Issue #1 "User consent for roaming case in eNA"). SA3 will provide feedback to SA2 once the study has concluded.

More specifically, for data that requires user consent, SA3 needs to have the following questions answered:

- Is there any legitimate interest for the data or analytics? Some examples of these are:

- service or feature provisioning, performance improvements including network optimization,

- performance of subscriber contract (to which the data subject is a party),

- general network maintenance and optimization,

- emergency scenarios,

- etc.

- Does the data or analytics contain any Personal Identifiable Information? If yes, would it be possible to de-identify the data and/or analytics?

- Will the data be disclosed to third parties?

- For the purpose of anomaly and fault detection an impact assessment should be carried out to understand the consequences of consent being withheld or withdrawn.

# 2	Actions

To SA2

ACTION: 	Take the above information in consideration, and kindly answer the questions above. More detailed information will be provided once the corresponding studies in SA3 (TR 33.738, TR 33.896) are concluded.

# 3	Dates of next TSG SA WG 3 meetings

SA3#109	14 - 18 November 2022	Toulouse (France)

SA3#110e	16 - 20 January 2023	Online (electronic meeting)