# 3GPP TSG-SA3 Meeting #108Adhoc-e	draft_S3-222833-r2

# e-meeting, 10th – 14th October, 2022

# Source:	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon

# Title:	add new key issue for path switching between two indirect network communication paths

# Document for:	Approval

Agenda Item:	5.3

1	Decision/action requested

Approve the KI added to TR33.740

2	References

[1]	3GPP TR 23.700-33 "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2"

[2]	3GPP TR 33.740 "Study on security aspects of Proximity Based Services (ProSe) in 5G System (5GS) phase 2"

3	Rationale

The contribution proposes a new KI on security aspects for path switching between two indirect network communication paths on UE-to-Network Relaying (KI#2 of TR 23.700-33).

Merger of S3-222480.

4	Detailed proposal

***************  Start of 1st change  ************

5.X	Key Issue #X: security consideration for path switching between two indirect network communication paths for UE-to-Network Relaying

5.X.1	Key issue details

There are three path switching scenarios are studied in TR 23.700-33, which including:

- Key Issue #2: Support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration.

- Key Issue #3: Support direct communication path switching between PC5 and Uu (i.e., non-relay case).

- Key Issue #4: Support of path switching between direct network communication path and indirect network communication path for Layer-2 UE-to-Network Relay with session continuity consideration.

.

For KI#2, there're different options:

-	Layer-3 UE-to-Network Relay with N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-2 UE-to-Network Relay.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay with N3IWF.

SA2 is studying how to select a UE-to-Network Relay for path switching and identify the path switch procedure with service continuity consideration. Similarly, security capabilities and configurations of target UE-to-Network Relay   should be considered when identify and select a UE-to-Network Relay for path switching. In addition, the potential security procedure after switching should be considered to avoid impacting service continuity.

5.X.2	Security threats

Security of the relayed traffics may be compromised after a ProSe remote UE switched to another UE-to-Network Relay.

Service continuity of a ProSe relay session may be impacted because of security procedure triggered after a ProSe remote UE switched to another UE-to-Network Relay.

If security and/or privacy measures are not considered and applied during and/or after the path switching procedures, an attacker can track the remote UE by linking the two indirect communications.

5.X.3	Potential security requirements

The 5G System shall provide a means to maintain the same level of security after a ProSe remote UE switched to another UE-to-Network Relay.

The 5G System shall provide a means to avoid service impact caused by security procedure after a ProSe remote UE switched to another UE-to-Network Relay.

The 5G system shall protect the privacy of the Remote UE during and/or after the path switching between two indirect network communication paths for UE-to-Network Relaying.

*************    End of the changes	*********