# 3GPP TSG-SA3 Meeting #108Adhoc-e	draft_S3-222833-r5

# e-meeting, 10th – 14th October, 2022

# Source:	Nokia, Nokia Shanghai Bell, Huawei, HiSilicon

# Title:	add new key issue for path switching between two indirect network communication paths

# Document for:	Approval

Agenda Item:	5.3

1	Decision/action requested

Approve the KI added to TR33.740

2	References

[1]	3GPP TR 23.700-33 "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2"

[2]	3GPP TR 33.740 "Study on security aspects of Proximity Based Services (ProSe) in 5G System (5GS) phase 2"

3	Rationale

The contribution proposes a new KI on security aspects for path switching between two indirect network communication paths on UE-to-Network Relaying (KI#2 of TR 23.700-33).

Merger of S3-222480.

4	Detailed proposal

***************  Start of 1st change  ************

5.X	Key Issue #X: security consideration for path switching between two indirect network communication paths for UE-to-Network Relaying

5.X.1	Key issue details

Support of path switching between two indirect network communication paths for UE-to-Network Relaying is studied in TR 23.700-33 as KI#2, there're different options:

-	Layer-3 UE-to-Network Relay with N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-2 UE-to-Network Relay.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay with N3IWF.

Editor's note: RAN conclusions needs to be taken into account.

SA2 is studying how to select a UE-to-Network Relay for path switching and identify the path switch procedure with service continuity consideration. Similarly, security capabilities and configurations of target UE-to-Network Relay   should be considered when identify and select a UE-to-Network Relay for path switching, especially for path switching across U2N relays with different RSCs.

5.X.2	Security threats

Security of the relayed traffics may be compromised after a ProSe remote UE switched to another UE-to-Network Relay, esepcially after switched to an UE-to-Network relays with differnet RSC.

For example, the PC5 identifier (e.g. L2 ID, CP-PRUK ID, UP-PRUK ID) used for two indirect links may not change over the links. In this case a third-party observer can link two links into the same remote UE if security and/or privacy measures are not considered and applied during and/or after the path switching procedures.

5.X.3	Potential security requirements

The 5G System shall provide a means to protect traffic of ProSe remote UE via UE-to-Network Relay with similar security policies after the remote UE switched to another UE-to-Network Relay.

The 5G system shall protect the privacy of the Remote UE during and/or after the path switching between two indirect network communication paths for UE-to-Network Relaying.

*************    End of the changes	*********