3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222853-r2

# e-meeting, 10 - 14 October 2022

# Source:	Samsung, Intel, Apple, Deutsche Telekom

# Title:	Conclusion for key issue#2

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Proposed to approve the conclusion for key issue#2 in 5GFBS TR 33.809

# 2	References

Null

# 3	Rationale

This pCR proposes to add the conclusion for key issue#2.

# 4	Detailed proposal

****START OF CHANGES ***

## 7.2	Conclusions on Key Issue #2

Following conclusions are made on Key Issue #2 "Security protection of system information":

It is concluded that Digital Signatures are used for protection of system information

Signature-based solutions (#7, #11, #12, #20, #21, and #27) are taken as the basis for the normative work.

It is concluded that both Elliptic Curve-based Certificateless Signatures for Identity-based Encryption (ECCSI) [9] and Certificate based Public Key Signature Schemes are supported.

UE supports both the schemes

Network supports at least Certificate based Public Key Signature Schemes

Solutions #7 and #12 to be considered as baseline for normative work for ECCSI and Solutions #21 and #27 to be considered as baseline for normative work for Certificate based Public Key Signature Scheme

It is concluded that Elliptic Curve Digital Signature Algorithm (ECDSA) is the signature algorithm to be used

Support for further signature algorithms to be decided during normative phase

RAN WGs based on the inputs from SA3 during study phase (Rel-18) will decide details on the inclusion of the digital signature in the SI framework and broadcasting periodicity.

Following conclusions are made on the normative work timeline:

Alternative 1: Normative work for the above conclusions to be done in Rel-18.

Alternative 2: Normative work for the above conclusions to be done in Rel-19.

****END OF CHANGES ***