3GPP TSG-SA3 Meeting #108Adhoc-e 	S3-222903

# e-meeting, 10-14 October 2022

Title:	draft reply LS on protection of the URSP rules from HPLMN

Response to:	LS S2-2207501/S3-222460 on LS on protection of the URSP rules from HPLMN from SA2

Release:	Rel-18

Work Item:	FS_eUEPO

Source:	SA3

To:	SA2

Cc:	CT1

Contact person:	Helena Vahidi

helena dot vahidi dot mazinani at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA3 would like to thank SA2 for the LS on protection of the URSP rules from HPLMN.

As mentioned in the LS, there is no mechanism in Rel-15 to Rel-17 for the UE to verify that URSP rules are provided from HPLMN and that it has not been intercepted and modified by the VPLMN. Instead, in these releases the trust between HN and SN has been seen as enough. There is not consensus in SA3 whether protection is required or not. Some members believe it is required and other believe it is not needed of that it needs further study.

No direct threats have been identified which indicates that the protection for previous releases is adequate. If SA2 believes that protection could add value in Rel-18, SA3 can study the topic further.

SA3 would like to inform SA2 that, if needed, similar security mechanisms as being used for protection of SoR and UPU could be reused to provide integrity protection for URSP rules, enabling the UE to verify that the information originates from the HPLMN and that it has not been altered by the VPLMN.

In case such a mechanism is introduced in Rel-18, it will impact UE and HPLMN. Both need to support the mechanism in order for it to work. Hence, there may also be need for a UE capability transfer from UE to HN. Since transport of URSP rules is transparent for AMF and V-PCF, there should be no impact on the serving network.

# 2	Actions

To SA2

ACTION: 	SA3 asks SA2 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#109	14 - 18 November 2022	Toulouse (France)

SA3#110e	16 - 20 January 2023	Online (electronic meeting)