| 3GPP TR 33.875 V1.4.0 (2022-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on enhanced security aspects of the 5G Service Based Architecture (SBA); (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Trust model	10

4.0 	General	10

4.1	 Actors	10

4.2	 Deployment options	11

4.3	 Description of the trust assumptions	11

4.3.0	General	11

4.3.1	Trust within one PLMN	11

4.3.2	Trust in Inter-PLMN communication	12

5	Key issues	13

5.1	Key issue #1: Authentication of NRF and NF Service Producer by the NF Service Consumer in indirect communication	13

5.1.1	Key issue details	13

5.1.2	Security threats	13

5.1.3	Potential security requirements	13

5.2	Key issue #2: Need for additional security at operational level among SCP domains	14

5.2.1	Key issue details	14

5.2.2	Security threats	15

5.2.3	Potential security requirements	15

5.3	Key Issue #3: Service access authorization in the "Subscribe-Notify" scenarios	15

5.3.1	Key issue details	15

5.3.2	Security threats	16

5.3.3	Potential security requirements	16

5.4	 Key issue #4: Authorization of SCP to act on behalf of an NF or another SCP	16

5.4.1	Key issue details	16

5.4.2	Security threats	16

5.4.3	Potential security requirements	17

5.5	 Key issue #5: End-to-end integrity protection of HTTP messages	17

5.5.1	Key issue details	17

5.5.2	Security threats	17

5.5.3	Potential security requirements	17

5.6	Key issue #6: Access token usage by all consumer NFs of an NF Set	17

5.6.1	Key issue details	17

5.6.2	Security threats	18

5.6.3	Potential security requirements	18

5.7	Key issue #7: Authorization mechanism determination	19

5.7.1	Key issue details	19

5.7.2	Security threats	19

5.7.3	Potential security requirements	19

5.8	Key issue #8: Service access authorization requirements in intra-PLMN scenarios for PLMN deploying multiple NRFs (in OAuth2.0 AS role)	19

5.8.1	Key issue details	19

5.8.1.1	Introduction	19

5.8.1.2	Hierarchical NRFs / Deployment model with local NRFs	20

5.8.1.3	Deployment model with NF Service Consumer directly accessing the NRF where the NF Service Producer is registered	20

5.8.2	Security threats	21

5.8.3	Potential security requirements	21

5.9	Key issue #9: Authorization for Inter-Slice Access	22

5.9.1	Key issue details	22

5.9.2	Security threats	22

5.9.3	Potential security requirements	22

5.10	Key issue #10: N32 security in mediated roaming scenarios	22

5.10.1	Introduction	22

5.10.2	Key issue details	23

5.10.3	Security requirements	23

5.11	Key issue #11: NRF validation of NFc for access token requests	24

5.11.1	Key issue details	24

5.11.1.0	General	24

5.11.1.1	Problem 1a	24

5.11.1.2	Problem 1b	24

5.11.1.3	Questions from Problems 1a and 1b:	24

5.11.1.4	Problem 2	24

5.11.2	Security threats	24

5.11.3	Potential security requirements	25

5.12	Key issue #12: security in Hosted SEPP scenarios	25

5.12.1	Introduction	25

5.12.2	Key issue details	25

5.12.3	Security requirements	25

6	Solutions	26

6.0	Mapping of solutions to key issues	26

6.1	Solution #1: Verification of the entity sending the service response in indirect communication without delegated discovery	27

6.1.1	Introduction	27

6.1.2	Solution details	28

6.1.3	Evaluation	29

6.2	Solution #2: Authorization between NFs and SCP	29

6.2.1	Introduction	29

6.2.2	Solution details	29

6.2.3	Evaluation	31

6.3	Solution #3: Using existing procedures for authorization of SCP to act on behalf of an NF Consumer	31

6.3.1	Introduction	31

6.3.2	Solution details	31

6.3.2.1	Request of access token on behalf of the consumer	31

6.3.2.2	Service request on behalf of the consumer	32

6.3.2.4	 Protection of the NF consumer's CCA	33

6.3.3	Evaluation	34

6.4	Solution #4: Service request authenticity verification in indirect communication	34

6.4.1	Introduction	34

6.4.2	Solution details	34

6.4.3	Evaluation	35

6.5	Solution #5: End-to-end integrity protection of HTTP body and method	35

6.5.1   	Introduction	35

6.5.2 	Solution details	36

6.5.3	Evaluation	37

6.6	Solution #6: Verification of Service Response from a NF Service Producer at the expected NF Set	38

6.6.1	Introduction	38

6.6.2 	Solution details	38

6.6.2.1 	For indirect communication without delegated discovery procedure	38

6.6.2.2 	For indirect communication with delegated discovery	39

6.6.2.3 	Client credentials assertion of NF Service Producer	40

6.6.3 	Evaluation	41

6.7	Solution #7: Access token request for NF Set	41

6.7.1	Introduction	41

6.7.2	Solution details	42

6.7.3	Evaluation	43

6.8	Solution #8: Integrity protection of HTTP message in consideration of update by SCP	43

6.8.1	Introduction	43

6.8.2	Solution Details	44

6.8.3 	Evaluation	44

6.9	Solution #9: Authorization mechanism negotiation	45

6.9.1	Introduction	45

6.9.2	Solution details	45

6.9.3	Evaluation	45

6.10	Solution #10: NRF deployment clarifications	46

6.10.1	Introduction	46

6.10.2	Solution details	46

6.10.3	Evaluation	46

6.11	Solution #11: Registered NF Profile changes for Inter-Slice Access	46

6.11.1	Introduction	46

6.11.1	Solution Details	47

6.11.3	Evaluation	47

6.12	Solution #12: Authorization of notification endpoint in “Subscribe-Notify” scenarios	47

6.12.1	Introduction	47

6.12.2 	Solution details	48

6.12.3 	Evaluation	49

6.13	Solution #13: Authentication of NF Service Producer in Indirect Communication	49

6.13.1	Introduction	49

6.13.2	Solution details	49

6.13.3	Evaluation	50

6.14	Solution #14: SCP trust domain or technical domain grouping	50

6.14.1	Introduction	50

6.14.2	Solution details	51

6.14.3	Evaluation	52

6.15	 Solution #15: Authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenario.	52

6.15.1 	Introduction	52

6.15.2 	Solution details	53

6.15.3 	Evaluation	54

6.16	Solution #16: Selective End of End Protection of HTTP Request and Response in Indirect Communication	54

6.16.1	Introduction	54

6.16.2	Solution details	54

6.16.3	Evaluation	55

6.17	Solution #17: Authorization mechanism negotiation using existing methods	55

6.17.1	Introduction	55

6.17.2	Solution details	55

6.17.3	Evaluation	56

6.18	Solution #18: TBD	56

6.18.1	Introduction	56

6.18.2 	Solution details	56

6.19	Solution #19: Hosted SEPP requirements	57

6.19.1	Introduction	57

6.19.2	Solution details	57

6.19.3	Evaluation	57

6.20	 Solution #20: PRINS for Roaming Hubs	58

6.20.1	 Introduction	58

6.20.2	 Solution details	58

6.20.3	 Evaluation	61

6.21	Solution #21: Certificate solution for NRF validation of NFc for access token requests	61

6.21.1	Introduction	61

6.21.2	Solution details	62

6.21.2.1	NF consumer information to validate at Service Request Authorization	62

6.21.2.2	Certificates	62

6.21.2.3	NRF validation solution	62

6.21.3	Evaluation	62

6.22	Solution #22: Combined certificate and profile solution for NRF validation of NFc for access token requests	63

6.22.1	Introduction	63

6.22.2	Solution details	63

6.22.2.1	NF consumer information to validate at Service Request Authorization	63

6.22.2.2	O&M Provisioning solution	63

6.22.2.3	Certificates	63

6.22.2.4	NRF validation solution	63

6.22.3	Evaluation	64

7	Conclusions	64

7.1	KI#1: Authentication of NRF and NF Service Producer in indirect communication	64

7.1.1	Analysis	64

7.1.2	Conclusion	64

7.2	KI#2: Need for additional security at operational level among SCP domains	64

7.2.1	Analysis	64

7.2.2	Conclusion	64

7.3	KI#3: Service access authorization in the "Subscribe-Notify" scenarios	65

7.3.1	Analysis	65

7.3.2	Conclusion	65

7.4	KI#4: Authorization of SCP to act on behalf of an NF or another SCP	65

7.4.1	Analysis	65

7.4.2	Conclusion	65

7.5	KI #5: End-to-end integrity protection of HTTP messages	66

7.5.1	Analysis	66

7.5.2	Conclusion	66

7.6	KI#6: Access token usage by all NFs of an NF set	66

7.6.1	Analysis	66

7.6.2	Conclusion	66

7.7	KI#7: Authorization mechanism determination	66

7.7.1	Analysis	66

7.7.2	Conclusion	66

7.8	KI#8: Service access authorization requirements in intra-PLMN scenarios for PLMN deploying multiple NRFs (in OAuth2.0 AS role)	67

7.8.1	Analysis	67

7.8.2	Conclusion	67

7.9	KI #9: Authorization for Inter-Slice Access	67

7.9.1	Analysis	67

7.9.2	Conclusion	67

7.10	KI #10: N32 security in mediated roaming scenarios	68

7.10.1	Analysis	68

7.10.2	Conclusion	68

7.11	KI #11: NRF validation of NFc for access token	68

7.11.1	Analysis	68

7.11.2	Conclusion	68

Annex A (informative): Change history	69

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

Should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

Can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The 5G core network introduced a Service-Based Architecture (the so-called SBA). This brought fundamental impacts on the way new services are created and how the individual Network Functions (NF) communicate. A more open and adaptable system design necessitated to study different approaches to enforce the security requirements of 3GPP systems, whilst not impeding flexible service creation and future innovations. Along with these architectural challenges, SBA further introduced changes to the protocol stack and serialization format of the 5G core network.

The SBA was set on providing solutions for authentication and authorization in direct communication scenarios as well as the N32 security. Later on enhancements were introduced for indirect communication scenarios as well as the concept of Client Credential Assertion to allow NRF/NF Service Producer to directly authenticate a NF Service Consumer.

While the SBA provides a good level of security, several additional aspects have been identified that may bring new potential threats. This will be documented by the present document.

# 1	Scope

The present document studies enhanced security aspects of the 5G Service Based Architecture. It will analyse potential threats, study necessary security enhancements, and document decisions of solutions to be adopted or not adopted after evaluating the risks versus the complexity.

In particular, the following topics are addressed:

-	Need and mechanism of enabling end to end authentication in roaming case if no cross-certification between operators is enabled;

-	Need and mechanism of enabling NF Service Consumer authentication of NRF and the NF Service Producer;

-	Need for addressing potential security impact of different deployment scenarios including the several SCPs;

-	Verification of URI in subscription/notification;

-	Dynamic authorization between SCPs or NF and SCP;

-	End-to-End Critical HTTP headers/body parts integrity protection;

-	Security of NRF service management;

-	N32 roaming security considerations for deployment scenarios including roaming hub and hosted SEPP.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3"

[6]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[7]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 3".

[8]	GSMA NG.113: "5GS Roaming Guidelines"; https://www.gsma.com/newsroom/wp-content/uploads/NG.113-v6.0.pdf.

[9]	3GPP TS 29.573: "5G System; Public Land Mobile Network (PLMN) Interconnection; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

- traffic shaper: a network device that is used to limit the throughput for individual connections by delaying or dropping packets.

## 3.2	Symbols

No symbols are defined.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and 3GPP TS 33.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 33.501 [2].

NFc 	NF Service Consumer

NFp 	NF Service Producer

SRV	service request verification

HL	header list

# 4	Trust model

## 4.0 	General

With introduction of the service-based architecture and moving at the same time to cloud deployments, new attack vectors such as that for NFs deployed in clouds give ground to vulnerabilities and, thus, can impact the mobile operator domain. As more important it is to assure the trust also within one operator's network. For this, security concepts have been introduced.

A service request requires mutual authentication, thus, all network functions support mutually authenticated TLS and HTTPS. After registration and discovery, any service request of a network function needs also to be authorized by an authorization server (NRF) before a NF Service Consumer can consume the services of a NF Service Producer. For this 5G introduces the concept of authorization token utilizing the Oauth 2.0 authorization framework.

With Rel-16 indirect communication via a Service Communication Proxy (SCP) is possible. SCPs can be operated in a fully service-meshed environment or as standalone entity.

In the following the trust relationships between the entities of an operator network are described.

## 4.1		Actors

The following actors within one PLMN are considered: NF, NRF, SCP.

- NFs can provide services or consume services.

- NRF is a repository capturing NF profiles of NFs offering its services to other NFs. It receives discovery requests from NF instances, maintains NF profiles and acts as an authorization server. NRF responds to authorization requests by NF Service Consumers by providing Oauth 2.0 access tokens to authorize a NF Service Consumer for gaining access to a service from a NF Service Producer.

- SCP is a service communication proxy used in indirect communication to interact with NFs and other SCPs within the PLMN. SCP also communicates with the SEPP.

The following types of actors when requesting services from another PLMN are considered: SEPP.

- SEPP sits at the edge of one operator's network to allow for a secured communication with the other operator's network’s SEPP.

## 4.2		Deployment options

The following deployment options need to be considered:

- Direct communication within the same PLMN vs direct communication in Inter-PLMN scenario, i.e. without SCP behind SEPP.

- Indirect communication in the same PLMN vs indirect communication in Inter-PLMN scenario: For both, SCP standalone and service mesh need to be considered.

SEPP to SEPP communication is secured on N32-c via TLS and on N32-f via TLS (i.e. transport layer security) or PRINS (i.e. application layer security on top of NDS/IP or TLS). TLS provides for authentication between two entities. Thus, securing at transport layer provides hop-by-hop security between two SEPPs. PRINS provides end-to-end application layer security between two SEPPs.

Hop-by-hop security introduces the additional risk of allowing an entity on the path to gain full access to signalling messages exchanged. An intermediary node can read, hide, or modify the originator information.

## 4.3		Description of the trust assumptions

### 4.3.0	General

Trust among the entities within one PLMN is required whether the NF Service Consumer (NFc) and NF Service Producer (NFp) are within the same PLMN or not. The trust relationships described here can be replaced by security mechanisms.

### 4.3.1	Trust within one PLMN

This clause describes the existing trust relationships within one PLMN.

NOTE: Whether the list of existing trust relationships described below is complete, depends on deployment choices.

NRF is the core entity handling management, discovery and authorization requests by NFs or SCP. The operator needs to apply necessary security measures to secure these operations. It is assumed that there is only one NRF, or all NRFs are within the same trust domain, i.e. all NRFs are in the same security domain and the same entity(-ies) are responsible for all NRFs.

Registration Management:

An NF Service Provider needs to trust the NRF that no other NF can register with the identity of NFp.

If there is no direct communication between NF and NRF, an NF Service Provider needs to trust that the SCPs forward NFp profiles unmodified.

If there is no direct communication between NF and NRF, an NF Service Provider needs to trust the SCPs that no other NF can impersonate the identity of NFp towards the SCP, i.e. tempting the SCP to register an NF with the false identity.

Discovery:

An NF Service Consumer needs to trust NRF to provide profiles of authenticated NF Service Providers that offer their services to the requesting consumer.

An NF Service Consumer needs to trust SCP to correctly forward the profiles of authenticated NF Service Providers that offer their services to the requesting consumer.

Access token request:

Trust in direct communication between NFs, NFs and SCP/SEPP, as well as SCP and SEPP is assumed per 33.501 with mandatory mutual authentication using TLS.

An NF Service Provider needs to trust NRF to provide access tokens for consumption of its services only to those NF Service Consumers that have requested for it and only for those services that are allowed by the registered NRF policy and the registered NF Service Provider policy.

Authentication and confidentiality protection in indirect communication is only achieved between NF and SCP, (potentially between multiple SCPs), SCP and NRF as well as SCP and SEPP, but additional considerations are needed for achieving trust between NFs, NF and NRF, as well as NRF and SEPP, NF and SEPP, when an SCP is on the path. This is because all traffic in indirect communication passes through SCPs, and TLS terminates at SCPs.

Thus, the SCP needs to be trusted by NFc and NFp, to only forward authentication tokens or CCA with the original request, as well as to forward information only between the legitimate endpoints of the communication.

An NF Service Provider needs to trust NRF to provide access tokens for consumption of its services only to those SCPs that are authorized by the NF Service Consumers that have requested for it and only for those services that are allowed by the registered NRF policy and the registered NF Service Provider policy.

It also needs to be distinguished if SCP is collocated to NFs (service mesh) or standalone.

For both standalone and service-mesh, the NFs sending their service requests via an SCP need to trust the SCP to which they send their service requests.

If a SCP is co-located (e.g., a side-car proxy) with a NF, trust of the NF in the SCP is implicit by its co-location. This is, because this SCP is performing many of the functionalities on behalf of the consumer, which already indicates a certain level of trust between NFs with co-located SCPs. When the SCP is implemented as a service mesh, the security solution between the side-car proxies is out of scope of 3GPP and left to the SCP implementation.

A standalone SCP is serving many NFs, not necessarily in the same infrastructure.

### 4.3.2	Trust in Inter-PLMN communication

With 5G, a new element has been introduced to handle inter-PLMN communication. The SEPP, i.e., the Secure Edge Protection Proxy acting as perimeter of PLMN, is responsible to secure the signalling message exchange with the SEPP of another PLMN via the Internet.

Editor's Note: in case IPX and RHUB are considered in 3GPP specification, trust statement in this clause needs to be clarified.

The SEPP of the sending PLMN needs to trust the SEPP of the receiving PLMN that no other entity on the path has unauthorized access or can modify signalling messages if not permitted to do so by policy.

Discovery:

The NRF in the NF Service Consumer PLMN needs to trust the cSEPP to route the request to the pSEPP representing the target PLMN and apply the correct protection policies to the discovery request.

The NRF in the NF Service Provider PLMN needs to trust the pSEPP to authenticate the origin network of the discovery request and ensure that this origin network is correctly represented in the request arriving at the pNRF.

Access token request:

When requesting an access token from the NRF in another PLMN, there is always an indirect communication involving the cSEPP and pSEPP. In addition, SCPs can be involved in either network.

An NF Service Provider needs to trust pNRF to provide access tokens for consumption of its services only to those NF Service Consumers in another PLMN and only for those services that are allowed by the registered NRF policy and the registered NF Service Provider policy.

An NF Service Provider needs to trust cNRF to provide access tokens for consumption of its services only to those NF Service Consumers in another PLMN that have requested for it and only for those services that are allowed by the registered NRF policy and the registered NF Service Provider policy.

An NF Service Provider needs to trust SCP in the Service Consumer PLMN to only forward authentication tokens or CCA with the original request, as well as to forward information only between the legitimate endpoints of the communication.

Service request:

An NF Service Provider needs to trust pSEPP to authenticate and verify the NFc's PLMN included in the request in order to be able to perform dynamic authorization.

A pSEPP needs to trust that the cSEPP is not forwarding requests on behalf of foreign PLMNs.

# 5	Key issues

## 5.1	Key issue #1: Authentication of NRF and NF Service Producer by the NF Service Consumer in indirect communication

### 5.1.1	Key issue details

When SCP is present, the TLS between an NF Service Consumer and NRF/NF Service Producer can be split into at least two segments (NFc-SCP, SCP-NRF or SCP-NFp). In this case, the NF Service Consumer (NFc) and NRF or NFc and NF Service Producer (NFp) do not directly authenticate each other via TLS.

Client Credentials Assertion (CCA) has been specified to allow NRF or another NF to directly authenticate an NF Service Consumer in the presence of an SCP, but direct authentication of the NRF or NF Service Producer by the NF Service Consumer has not been addressed in indirect communication. The key issue will investigate solutions allowing the NF Service Consumer to directly authenticate the NRF/NF Service Producer in indirect communication.

### 5.1.2	Security threats

The request by the NF Service Consumer could be forwarded by the SCP to an unintended NF.

The request by the NF Service Consumer could be forwarded within the validity of an authorization token more than one request could be sent to the same NF, which could result in a deny of service attack.

In indirect communication scenarios an NF Service Consumer cannot verify whether the service response was sent by a legitimate NF or NRF. NFc can only authenticate the SCP, but not NFp or NRF. Thus, an NF Service Consumer could receive service responses from an unintended NF.

### 5.1.3	Potential security requirements

The 5GS should provide a mechanism that allows an NF Service Consumer to authenticate an NRF or an NF Service Producer during an indirect communication with them via an SCP.

NOTE: It needs to be taken into account that producer reselection by SCP can be a desired feature.

## 5.2	Key issue #2: Need for additional security at operational level among SCP domains

### 5.2.1	Key issue details

TS 23.501 [3] addresses the aspects of handling multiple SCPs in indirect communication without and with delegated discovery. With Rel-16 a SCP domain is introduced, which comprises a configured group of one or multiple SCPs that can reach certain NF instances or SCPs directly.

SCPs can register a SCP profile in NRF. 3GPP TS 23.502 [7] describes in the SCP profile SCP domain registration details about interconnected SCPs and also identifies SCPs that interconnect domains. Table 6.1.6.2.2-1 of 3GPP TS 29.510 [6] provides a detailed description. SCPs need to request NRF to discover the next hop SCP to route a service request from the NF Service Consumer to a NF Service Producer via multiple SCPs.

The primary purpose of SCP domains is to describe the connectivity topology within a network. All SCPs within an SCP domain can directly interconnect. One SCP can be part of multiple SCP domains. In fact, the primary purpose of intermediate SCPs in the path is to interconnect SCP domains, thus, there are boundaries between SCP domains at each SCP in the path.

PLMN-wide trust between NFs and SCPs is an option, but more restrictions could be desirable in complex networks with SCP domains, e.g., if SCPs are operated in different regions/provinces, the domain concept could be used for establishing operational restrictions by defining SCP domains. Or, there can be several technical SCP domains within a PLMN, where equipment with different capabilities is deployed and signalling also varies in some respects, e.g., if equipment upgrade is performed in a stepwise manner. Such technical domains can be defined based on computer centre boundaries, based on operators of subnetworks, based on regions/provinces, etc.

The figure below gives an idea of structuring or grouping a PLMN in several SCP domains, possibly interconnecting with other SCP domains via dedicated SCPs.

Figure 5.2.1-1: Illustration of SCP domains connecting via dedicated SPCs

This key issue is to study the concept of one or several SCP domains becoming regions of trust of finer granularity than PLMN and whether there is a necessity of trust and policing of communication within or among such domains, i.e., for the case that request messages traverse a boundary between trust domains.

### 5.2.2	Security threats

In large PLMNs operated by different organizations in different regions/provinces or by slices operated by a tenant, PLMN wide trust between NFs and SCPs could result in messages traversing over several SCP domain boundaries, even though this is not wanted. It could be not possible to shield, e.g., regions or organizations against each other, resulting in availability of information to unauthorized parts of the network.

If an access token has been requested by one SCP on behalf of the NF Service Consumer and granted by NRF, it is not guaranteed that the same SCP is forwarding the service request to the NF Service Producer. Instead, the SCP could forward the service request including the access token to another SCP first. If the new SCP is not be part of the same SCP domain, the trust in this SCP could be less than in SCPs within the same SCP domain.

### 5.2.3	Potential security requirements

N/A

NOTE: If it is decided to further develop the solutions of this key issue, potential security requirements can be discussed.

### 5.2.4	Potential architectural/operational security requirements

The 5G system shall allow restricting network topology so that requests between certain NFs or SCPs or SCP domains are not allowed.

## 5.3	Key Issue #3: Service access authorization in the "Subscribe-Notify" scenarios

### 5.3.1	Key issue details

"Subscribe-Notify" NF Service illustration 1 specified in TS 23.501, clause 7.1.2, allows one NF (e.g., NF_A) to subscribe to notifications of NF producer (e.g., NF_B). The subscription request includes the notification endpoint (e.g., the notification URL) of the NF Service Consumer. In this scenario, NF_A subscribes the service of NF_B for itself.

Figure 5.3.1-1: "Subscribe-Notify" NF Service illustration 1 (non-delegated scenario)

"Subscribe-Notify" NF Service illustration 2 specified in TS 23.501, clause 7.1.2, allows one NF (e.g., NF_A) to subscribe the service of NF producer (e.g., NF_B) on behalf of another NF (NF_C), in which the notification URI of NR_C is included. It means the NF_C will receive the notification message even though the subscribe request is sent by NF_A.

Figure 5.3.1-2: "Subscribe-Notify" NF Service illustration 2 (delegated scenario)

For instance, as defined in TS 23.502 clause 4.15.3.2.2, UDM could send subscribe request including the UDM URI and NEF URI to the AMF to subscribe service on behalf of the NEF, i.e., Namf_EventExposure_subscribe request. If the monitored event occurs, the AMF will send the event report to the associated notification URI endpoint of the NEF. Here the location report of the UE is one of the potential event reports, which can be provided by the AMF during in the above procedure. It means that the UE location report will be transmitted to the NF_C according to the subscribe request sent by NF_A.

The security issue of "Subscribe-Notify" NF Service illustration 1 and 2 is that NF_B may redirect the Notification message to an unauthorized NF if the Notification URI in the subscribe message is not authorized. The issue now also arises because of the subscribe notify use cases that have been defined with respect to DCCF and MFAF, wherein both the DCCF and the MFAF are only provided with the URI where the notification has to be sent, and therefore an unauthorized consumer can receive the notifications if the URI is not authorized.

This key issue seeks for solutions on how to assure that the notification messages could be only forwarded to an authorized NF by the NRF.

### 5.3.2	Security threats

When a malicious NF or a compromised NF tries to access an unauthorized service, in “Request-Response” scenario, NRF can verify and prevent it during access token process. But, in “Subscribe-Notify” scenario, a compromised NF can subscribe a notification service from a NF Service Producer to notify data to an unauthorized NF (possibly, a malicious NF) by setting address of notification endpoint (e.g., “Notification URI”) with address of the unauthorized NF. In this case, the NF Service Producer cannot ensure that the NF, whose URI is mentioned, is authorized to receive the notification. Thus, a malicious NF can force the NF Service Producer to send notifications to arbitrary consumers, which can, e.g., result in information leakage.

According to TS 23.501, “Subscribe-Notify” scenario are used not only for subscriber’s mobility, session and subscription related events but also for NF’s own event (e.g., AMF Status change), and this information can be leaked to an unauthorized NF, while according to TS 23.288 Clause 6.2.6 subscribe-notify is used in order to enable the data consumer to receive the data from DCCF and MFAF. On the other hand, the Notification message that may include the sensitive information (e.g., location report), may expose to an unauthorized network function routed by the URI in the subscribe request message.

### 5.3.3	Potential security requirements

It shall be possible for 5G system to ensure notification service is only provided to an authorized NF routed by the URI in the subscribe request message.

It shall be possible for 5G system to prevent information disclosure to an unauthorized NF routed by the URI in the subscribe request message.

Editor’s Note: It is ffs whether these are the correct requirements.

## 5.4		Key issue #4: Authorization of SCP to act on behalf of an NF or another SCP

### 5.4.1	Key issue details

This key issue is about authorization of SCP to request services on behalf of an NF or of another SCP and how this authorization is verified by the NRF or NF Service Producer.

### 5.4.2	Security threats

If the NRF cannot verify if the SCP has been authorized by the NF Service Consumer, the SCP can send a service request and receive a valid service response on behalf of NF Service Consumer, even though the NF Service Consumer has not authorized the SCP.

If the NF Service Producer cannot verify if the SCP has been authorized by the NF Service Consumer, the NF Service Producer can provide a service response to an unauthorized entity.

### 5.4.3	Potential security requirements

The 5GS should provide a mechanism for how an NRF or NF Service Producer can verify an SCP has been authorized by an NF Consumer to request access tokens or services on behalf of the consumer.

## 5.5		Key issue #5: End-to-end integrity protection of HTTP messages

### 5.5.1	Key issue details

Currently, in the case of indirect communication with an SCP in the path between an NF Service Consumer and an NF Service Producer, the integrity protection of the HTTP messages is provided by TLS for each hop but not end-to-end between the NF Service Consumer and the NF Service Producer. Since an SCP may need to change the content of an HTTP message, this KI is to investigate how end-to-end integrity protection of HTTP messages can be achieved while at the same time continue to allow the SCP to perform necessary mediation of HTTP messages.

NOTE: Potential issues with backwards compatibility with existing procedures are to be considered during the study.

### 5.5.2	Security threats

Critical elements of an HTTP message that are not end-to-end integrity protected could be modified by an attacker. In more detail, a service request in indirect communication could lead to attacks by Man in the Middle, which for instance can intercept the service request and try to modify the content of the message or HTTP (custom) header. This could cause communication failure, lead to DoS attacks.

### 5.5.3	Potential security requirements

In the case of indirect communication with an SCP in the path between an NF Service Consumer and an NF Service Producer, the 5GS should support end-to-end integrity protection of critical elements of an HTTP message while allowing the SCP to continue to perform necessary HTTP message mediation.

NOTE: Critical HTTP elements would need to be determined by stage 3.

The NF Service Producer should be able to verify that critical elements of a service request of the NF Service Consumer received via the SCP have not been modified.

## 5.6	Key issue #6: Access token usage by all consumer NFs of an NF Set

### 5.6.1	Key issue details

SBA introduces the concepts of NF Set and NF Service Set, i.e., sets of functionally equivalent and inter-changeable NFs or NF services. 5G SBA architecture design further allows for the concept of stateless NFs, whereby binding indication the NF Service Resource owner can indicate to the NF Service Consumer, for a particular resource, whether it is to an NF Service Instance, NF Instance, NF Service Set or NF Set.

Access token usage for NF Service Producer Set:

As specified in Rel-16, an access token can be provided by NRF for consuming a service from a dedicated producer with a distinct NF Instance Id or a specific NF type or a NF Set Id for a NF Set of NF Service Producer instances. Thus, if the NF Service Producer belongs to a NF Set, the access token can be consumed by a NF Service Consumer from any of the NF Service Producers within the set.

Stateless NFs:

NF Set concept supports stateless NF implementations i.e., an NF Service Producer or NF Service Consumer in a NF Set can take over at any time the control of respectively resource contexts (e.g., PDU session contexts) or session contexts to receive notifications. NFs typically produce and consume services (e.g., an SMF producing the PDUSession service to establish PDU session also needs to consume services to render its PDU session service, e.g., it consumes PCF and CHF services), taking over the control at any time allows for reliability of NF instances within the same NF Set (e.g., when an NF instance fails or is scaled-in).

If an access token is granted to a specific NF Service Consumer instance, other NF Service Consumer instances in the same NF Set currently need to request always a new access token, whenever a request is sent by a different NF Service Consumer instance.

For example, a connection is released since the NF Service Consumer is stateless, then another NF Service Consumer of the NF Set can be assigned to continue subsequent communication. This optimization is part of 23.501/29.500 specifications, but the related security aspects of using such optimization have not been addressed in 33.501 Rel-16. Thus, any NF in NF Set issuing a service request targeting an existing context need to request a new access token. Further, any subsequent request may be sent to any other NF than the initiator NF of the NF Set; and also in this case, a new access token is needed.

Examples:

The following examples show, why it is useful to have an access token also be valid/useable for any NF in the NF Set during its validity time.

1) A SMF instance can wish to remain the SMF (binding to itself), but at end of procedure, i.e., non-moving UEs anymore foreseen. Thus, this SMF gets stateless because it considers it is a long time before next SMF involvement. Thus, if another SMF than the service request originating SMF would get involved later, it would either need a new token or it could re-use the non-expired access token, the other SMF instance of the NF Set received earlier.

2) In stateless UDM, the binding within UDM set can be used. When UDM instance of UDM Set initially creates an AMF event subscription, it has to request an access token to be able to access the corresponding AMF service. However, the UDM instance that created the subscription may be a completely different UDM instance of the UDM Set that is later deleting the subscription. Thus, the same token within the NF Set should be useable for achieving this. Otherwise, it could end in massive access token requests that are used in the same context of service consumption.

If an access token cannot be used by any ND in the NF Set during its validity time, the need for access token requests is multiplied, because every time there is a different NF instance in the NF Set that is requesting from the existing resource would need a new access token, while this is not necessarily required.

Key issue scope:

This key issue proposes to study the advantages and disadvantages from security perspective that any NF in a NF Set targeting a service of an existing resource can use an access token provided to a NF Set.

If acceptable from security point of view, the benefit of this concept would be that it maps with the 5G SBA architecture design, the concept of stateless NF, and the binding level of NF Set, where any NF instance can serve subsequent request without each time requesting a new access token.

Thus, this key issue studies the security implications of a stateless NF Service Consumer belonging to a NF Set requesting an access token on behalf of and for usage by all NF instances of the NF Set.

### 5.6.2	Security threats

In general, the concept of access token is already in place. No new threats need to be listed.

### 5.6.3	Potential security requirements

All NF Service Consumers of an NF Set shall be authorized to use the access token requested by one NF Instance of the NF Set, if the access token is issued for NF Set.

The 5GS may provide means to authorize a NF Service Consumer of the NF Set to request and/or use an access token requested by another NF Service Consumer of the same NF Set.

If an access token provided for all NFs of the same NF Set is used, its usage shall be limited to this specific NF Set.

## 5.7	Key issue #7: Authorization mechanism determination

### 5.7.1	Key issue details

It is specified in TS 33.501 [2] clause 13.3.0 that static authorization can be used for authorization when token-based authorization is not used. However, two PLMNs may have a roaming issue if the authorization mechanism is not aligned between them. For example, when the NF Service Consumer (NFc) deployed in one PLMN only supports the usage of static authorization, and the NF Service Producer (NFp) deployed in the other PLMN only supports the usage of OAuth authorization, the NFp will reject the NF Service Consumer.

On the other hand, TS 29.510 [6] defined an oauth2Required indicating that OAuth authorization is required for the NFp service access, which will be sent back to the NFc via the discovery response. Accordingly, NFc should get the token before consuming the NFp services. Hence, NFc that only supports the usage of the static authorization will not be able to consume the service provided by the NFp. However, how to handle the failure issue when the NFc only supports the usage of static authorization is not clarified.

The key issue will investigate solutions allowing the two operators to handle the case that one operator uses token-based authorization, and its roaming partner uses static authorization.

### 5.7.2	Security threats

The SBA service authorization will fail in the roaming case if the authorization mechanism is not aligned between them.

### 5.7.3	Potential security requirements

The 5GS should provide mechanisms to handle the case that one operator uses token-based authorization, and its roaming partner uses static authorization.

## 5.8	Key issue #8: Service access authorization requirements in intra-PLMN scenarios for PLMN deploying multiple NRFs (in OAuth2.0 AS role)

### 5.8.1	Key issue details

#### 5.8.1.1	Introduction

Multiple NRFs can be deployed in a PLMN, optionally using a hierarchical structure whereby an NRF may redirect or forward service requests to another NRF. One (or more) NRF can serve the entire PLMN, a set of network slices, or a single network slice.

TS 23.501 states:

In the context of Network Slicing, based on network implementation, multiple NRFs can be deployed at different levels (see clause 5.15.5):

-	PLMN level (the NRF is configured with information for the whole PLMN),

-	shared-slice level (the NRF is configured with information belonging to a set of Network Slices),

-	slice-specific level (the NRF is configured with information belonging to an S-NSSAI).

One PLMN with several NRFs can be deployed in many ways: NRFs can have all the same data or could hold different subset of data. NRFs could all be OAuth 2.0 servers or only some of them, e.g., having one NRF being the central OAuth 2.0 server.

To receive an access token, the OAuth client need to be known to the NRF issuing the token. But looking at the different deployment options, the NRF knowing the client could be different from the NRF authorizing and issuing the access token. This raises the question, by which NRF an OAuth client needs to be authenticated and by which NRF an OAuth client gets the access token after authorization.

Only the NRF where the NF Service Producer has registered its services can act as the OAuth authorization server, i.e., to provide an access token. But the requesting NF Service Consumer is not necessarily known to this OAuth authorization server in deployment scenarios with multiple NRFs. For instance, an AMF may be registered in a PLMN-wide NRF while SMFs supporting specific network slice(s) may be registered in a slice(s) specific NRF. How does the AMF get an access token to access the SMF services in such deployment?

This key issue will clarify the service access authorization requirements and call flows, for the different NRF deployment models in case of multiple NRFs in the PLMN, including when the access token request is sent to a different NRF than the NRF where the NF Service Producer has registered its services.

TS 33.501 does only cover the inter-PLMN case, where vNRF authenticates the NF Service Consumer and hNRF provides the access token after the hNRF authorized the NF Service Consumer. How the trust between vNRF and hNRF is assured needs further clarification. For the intra-PLMN case, in particular slice specific authorization, such clause is missing.

Therefore, this key issue takes into account the different deployment models in intra-PLMN authorization requests.

#### 5.8.1.2	Hierarchical NRFs / Deployment model with local NRFs

This deployment model assumes that NFc needs to be registered at a local NRF or that NFc is known (as Oauth client) at a local NRF. It also assumes that one NRF is trusting the other NRF in the same PLMN.

When requesting an access token, NFc goes first to its local NRF, which authenticates NFc and then forwards or redirects the request to the target NRF, where a NFp has registered its services. In this case the local NRF authenticates the NFc and the target NRF (holding the policy for NFp services) provides the access token for NFp service.

Comment: This variant uses the model of inter-PLMN service access authorization also for intra-PLMN cases with multiple NRFs (with OAuth2 Authorization Server role), i.e., with an NFc registered as OAuth2 client to one NRF (local NRF) and with access token requests issued by this NFc always going through this specific/local NRF and being forwarded or redirected to the target NRF (with OAuth2 Authorization Server role) where the NFp has registered its services.

This deployment model can also apply to deployments where NFc is registered or known as Oauth client at a NRF that is not necessarily close to NFc, e.g., an AMF registered in a PLMN wide NRF.

#### 5.8.1.3	Deployment model with NF Service Consumer directly accessing the NRF where the NF Service Producer is registered

There can be centralized NRF(s) or distributed NRFs in OAuth2 Authorization Server role.  An NRF can be configured by OAM with OAuth clients/ access token policies enabling a consumer to get access tokens from different NRFs (in OAuth2 Authorization Server role).

A NF (e.g., AMF) can register and/or be known as OAuth 2.0 client to a PLMN-wide NRF, but can also address a specific NRF directly, e.g., AMF can be configured with or can retrieve from the NSSF the NRF Access Token URI to use for a specific network slice: AMF may retrieve from the NSSF the NRF Access Token URI it should use for a specific network slice (see 29.531).

| nrfAmfSetAccessTokenUri | Uri | O | 0..1 | When present, this IE shall contain the API URI of the NRF Access Token Service (see clause 6.3.2 of 3GPP TS 29.510 [13]). |
| --- | --- | --- | --- | --- |

Thus, an AMF can send the Access Token Request directly to the slice specific NRF, there is no need to go via a "local" NRF, where the AMF could be registered.

23.502, Figure 4.3.2.2.3.2-1: SMF selection for non-roaming and roaming with local breakout scenarios

This procedure may be skipped altogether if SMF information is available in the AMF by other means (e.g., locally configured); otherwise:

-	when the serving AMF is aware of the appropriate NRF to be used to select NFs/services within the corresponding Network Slice instance based on configuration or based on the Network Slice selection information received during Registration, only steps 3 and 4 in the following procedure are executed as described in Figure 4.3.2.2.3.2-1;

-	when the serving AMF is not aware of the appropriate NRF to be used to select NFs/services within the corresponding Network Slice instance, all steps in the following procedure are executed as described in Figure 4.3.2.2.3.2-1.

1.	The AMF invokes the Nnssf_NSSelection_Get service operation from the NSSF in serving PLMN with the S-NSSAI of the Serving PLMN from the Allowed NSSAI requested by the UE, PLMN ID of the SUPI, TAI of the UE and the indication that the request is within a procedure of PDU Session establishment in either the non-roaming or roaming with local breakout scenario.

2.	The NSSF in serving PLMN selects the Network Slice instance, determines and returns the appropriate NRF to be used to select NFs/services within the selected Network Slice instance, and optionally may return a NSI ID corresponding to the Network Slice instance.

See 29.531, 6.1.6.2.7 which includes

| nrfAccessTokenUri | Uri | O | 0..1 | When present, this IE shall contain the API URI of the NRF Access Token Service (see clause 6.3.2 of 3GPP TS 29.510 [13]). |
| --- | --- | --- | --- | --- |

Thus, based on configuration or based on the Network Slice selection information received during Registration or PDU session establishment, the AMF is aware of the appropriate NRF to be used to select NFs/services within the corresponding Network Slice instance. There is no way to pass this URI to a “local NRF” (where the AMF would be known as OAuth2 client), as opposed to the Inter-PLMN case, where the NRF Access Token Request supports the AMF providing the hnrfAccessTokenUri.

### 5.8.2	Security threats

Not applicable, since this key issue is for clarifying missing specification text.

### 5.8.3	Potential security requirements

Not applicable, since this key issue is for clarifying missing specification text.

## 5.9	Key issue #9: Authorization for Inter-Slice Access

### 5.9.1	Key issue details

GSMA LS S3-211383 on “Prevention of attacks on sliced core networks” identifies a number of issues related to SBA authorization framework. This key issue studies SBA related aspects of the attack papers mentioned in the GSMA LS.

In the current SBA authorization framework, the mechanisms specified to validate the S-NSSAI of NF Consumer are not fully specified and therefore, sometimes an NF Service Producer and/or NRF is dependent on local configuration or proprietary mechanisms to verify if the NF Service Consumer really belongs to the S-NSSAI it is claiming to belong to. While an NRF could validate the S-NSSAI by comparing the S-NSSAI provided in, e.g., Access-Token-Get request with the S-NSSAI registered in the NF-Profile; this is not always possible, as it is not necessary that every NF registers its profile with NRF, unless the information is available to NRF by other means (e.g., local configuration). This results in a NF Service Consumer gaining access to slices it should not have been allowed to access.

Additionally, currently there is no mechanisms specified whereby an NRF could validate if an NF Service Consumer is allowed to access resources belonging to a different S-NSSAI in the NF Service Producer. While an NRF could specify, in the Access Token Grant, the S-NSSAIs an NF Service Consumer is allowed to access in the NF Service producer, it is assumed that NRF uses local configurations while providing such information.

This Key Issue will study how to prevent any malicious entity (for instance a NF Service Consumer) from accessing a slice it is not authorized to access, or from requesting a service from a slice which it is not authorized to access.

NOTE: In the GSMA LS to SA3 (S3-211383), it is assumed that an NF within a 3GPP network can be fully compromised, which is a rather strong assumption. This Key Issue only aims to strengthen the authorization mechanism for granting access to an NF within SBA.

### 5.9.2	Security threats

A malicious entity (for instance a NF Service Consumer) can request an access token for the slice it is not authorized to access.

A malicious entity (for instance a NF Service Consumer) can request a service request for the slice which it is not authorized to access.

### 5.9.3	Potential security requirements

The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

The 5GS should provide a mechanism that allows NRF not to provide access tokens to NF consumers which are not authorized to access a slice.

## 5.10	Key issue #10: N32 security in mediated roaming scenarios

### 5.10.1	Introduction

In roaming scenarios, the communication between the visited network and the home network is typically mediated and routed through intermediaries. Two main types of intermediaries exist.

IPX providers: While, traditionally, for some roaming relations, they simply route the traffic, for others they actively modify certain messages in order to establish or enhance interoperability.

Roaming hubs: They offer a contractual and technical framework that enables operators to avoid entering a very large number of roaming agreements with individual other operators in order to achieve a large roaming coverage. By contracting the roaming hub, the operator obtains access to a large roaming footprint, without individually contracting the operators that are mediated through the hub. Apart from mediating the signalling, roaming hubs typically also mediate the billing. GSMA introduced the concept of roaming hubs to 3GPP in LS S3-213806.

The key issue looks at N32 security in intermediary scenarios, in particular roaming hub scenarios that have not been addressed in TS 33.501 so far.

### 5.10.2	Key issue details

The GSMA is working on best practice guidance and related specifications for how operators interact with both IPX and roaming hubs. However, the pre-5G architecture of these methods uses the 4G hop-by-hop security paradigm. In 5G, SEPP communication for roaming relies on end-to-end security by design as specified in TS 33.501, i.e. messages are authenticated and integrity-protected between the SEPP of the visited network and the SEPP of the home network. This creates the need to study in more detail how the migration away from hop-by-hop security can be achieved without loss of the benefits provided by IPX providers and roaming hubs while the 5G security requirements on N32 are met.

In certain scenarios, IPX providers and roaming hubs would need to modify certain IEs. This requires the setup of appropriate modification policies between the roaming partners. Given that, according to TS 33.501,

- all attributes transferred over the N32-f interface shall be integrity protected (clause 5.9.3.3),

- a default data-type encryption policy, which provides confidentiality protection for authentication vectors, cryptographic material and location data, and optionally for SUPI, applies (clause 5.9.3.3), and

- any roaming partner-specific data-type encryption policy takes precedence over the default policy (clause 13.2.3.6),

and further given that

- it is assumed that the set of IEs to be modified depends on the deployment scenario, and

- no default modification policy has been specified,

maintaining a large set of roaming partner-specific modification policies could cause avoidable complexities. It is, therefore, unclear whether the current specification needs to be extended in order to avoid such complexities.

Furthermore, a roaming hub may need to prevent the setup of N32-c between a home network SEPP and a visited network SEPP, e.g., in case the respective operators have not established a commercial roaming relation via the roaming hub.

### 5.10.3	Security requirements

Existing requirements in TS 33.501 on the SEPP should apply in the roaming intermediary scenario unless there are explicit exceptions.

Specifically, the requirements on edge protection of the PLMN should still apply, including end-to-end security for N32-c.

Existing NFs and SCPs should be impacted as least as possible.

For the scenarios involving intermediaries (IPX providers, roaming hubs), requirements are as follows.

- The intermediaries (IPX providers, roaming hubs) shall use their own unique credentials to authenticate themselves.

## 5.11	Key issue #11: NRF validation of NFc for access token requests

### 5.11.1	Key issue details

#### 5.11.1.0	General

There are several procedures for access token requests in TS 33.501 [2] clause 13.4.1, each for different scenarios such as direct or indirect communication. The description of how the NRF authorizes the access token request are more or less detailed. The most detailed description is in clause 13.4.1.1.2, Access token request for accessing services of NF Service Producers of a specific NF type: "The NRF may verify that the input parameters (e.g., NF type) in the access token request match with the corresponding ones in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s)." Other procedures either refer to clause 13.4.1.1.2 or contain even less detailed descriptions.

There are several issues with how authorization of access token requests is described in TS 33.501 [2].

#### 5.11.1.1	Problem 1a

Not all NF Service Consumers register an NF profile at the NRF. This results in no NF profile is available. Furthermore, the NF profile is provided by the NF itself, which makes it a less reliable source for deciding whether the NF is authorized.

#### 5.11.1.2	Problem 1b

It is not a mandatory requirement that all IEs defined in the SBA TLS certificate has been filled in with information presented in the TLS certificate. For example, NF Type is only "should". Other kind of information, like slice information, is not contained in the TLS certificate profile at all. So, there may be situations when certificates are available, but information may not be sufficient for NFc validation in NRF. It is not specified how the NRF shall validate the NFc request in all situations.

#### 5.11.1.3	Questions from Problems 1a and 1b:

The study needs to elaborate on the following questions:

1.	What IEs are required to validate as a minimum?

2.	What information shall be used in NRF to validate the NFc?

3.	How is this information provisioned in NRF?

#### 5.11.1.4	Problem 2

It is also possible that the NFc TLS certificate is available together with the NF profile registered in the NRF. In this case it is unclear what precedence the NRF shall use when performing the NFc validation. The information in the profile is provided by the NFc itself, but the information in the certificate is provided outside of the NFc, i.e., the CA.

The study needs to elaborate on the following questions:

- What information shall have precedence when NFc certificate is available as well as the NF profile?

- What information shall be cross-checked between the certificate and the profile?

### 5.11.2	Security threats

The security level may be NRF vendor specific if not specified.

If there is no standardized agreement of the solution to the Key Issue the interoperability between different vendors of NFc and NRF may result in being too allowing from a security perspective.

### 5.11.3	Potential security requirements

N/A.

This KI does not add any new security requirements but is focused on to develop solutions to the problems stated above.

## 5.12	Key issue #12: Security in Hosted SEPP scenarios

### 5.12.1	Introduction

In some scenarios, the operator may decide to outsource the operation of its SEPP to an external entity. This scenario is called the “Hosted SEPP” scenario.

The Hosted SEPP scenario introduces security requirements which must be fulfilled by N32 and by the connection between PLMN and Hosted SEPP. This key issue documents how they can be fulfilled.

### 5.12.2	Key issue details

The decision of an operator to outsource the operation of a SEPP to an external entity can be independent of whether and which IPX providers are used, and whether and which roaming hub(s) are used. See KI#10 for this topic. However, it is possible that an IPX provider, a roaming hub, or any other external entity takes the role of the Hosted SEPP provider.

In case an operator uses a Hosted SEPP, the security perimeter of the PLMN as described in TS 33.501, clause 4.2.1, extends to an entity external to the PLMN.

According to GSMA LS SA3-221737, a PLMN may use both Local SEPPs (managed by PLMN) and Hosted SEPPs, i.e., the PLMN has Local SEPPs each handling a set of roaming relations, while the Hosted SEPPs handle a different set of roaming relations.

In a scenario where an entity external to the PLMN operates all or some of the SEPPs of a PLMN new risks may arise. These include:

-	attacks on the traffic between PLMN and Hosted SEPP,

-	missing edge protection at the PLMN,

-	lack of a way to attribute the cause of a security issue to a specific actor (the operator or the Hosted SEPP provider),

-	an unauthorized actor claiming to be a Hosted SEPP provider of an operator towards its roaming partners, and

-	risks due to one actor operating Hosted SEPPs for different PLMNs.

### 5.12.3	Security requirements

Existing requirements in TS 33.501 on the SEPP shall apply in the Hosted SEPP scenario, unless there are explicit exceptions. Specifically, edge protection requirements can differ from the existing requirements since the hosted SEPP is not deployed at the edge of the PLMN.

Existing NFs and SCPs should be impacted as least as possible.

For the scenario of outsourcing a SEPP, the following requirements shall apply:

- The Hosted SEPP providers shall use their own unique credentials to authenticate themselves. Moreover, it shall be possible for operators that receive signalling from a Hosted SEPP provider pertaining to a roaming partner to verify that the Hosted SEPP provider has been authorised by the roaming partner.

- The connection between the PLMN and the Hosted SEPP provider shall be confidentiality, integrity, and replay protected.

- The solution should enable the Hosted SEPP providers to operate SEPPs for multiple PLMNs in a way that isolates the SEPP instances operated for different PLMNs from each other for security reasons. Specifically, if one of the instances gets corrupted or otherwise malfunctions, other instances should remain unaffected as much as possible.

# 6	Solutions

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | Key Issues |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 |
| #1: Service response verification in indirect communication without delegated discovery | X |  |  |  |  |  |  |  |  |  |  |  |
| #2: Authorization between NFs and SCP |  |  |  | X |  |  |  |  |  |  |  |  |
| #3: Using existing procedures for authorization of SCP to act on behalf of an NF Consumer |  |  |  | X |  |  |  |  |  |  |  |  |
| #4: Service request authenticity verification in indirect communication |  |  |  |  | X |  |  |  |  |  |  |  |
| #5: End-to-end integrity protection of HTTP body and method |  |  |  |  | X |  |  |  |  |  |  |  |
| #6: Verification of Service Response from a NF Service Producer at the expected NF Set | X |  |  |  |  |  |  |  |  |  |  |  |
| #7: Access token request for NF Set |  |  |  |  |  | X |  |  |  |  |  |  |
| #8: integrity protection of HTTP message in consideration of update by SCP |  |  |  |  | X |  |  |  |  |  |  |  |
| #9: Authorization mechanism negotiation |  |  |  |  |  |  | X |  |  |  |  |  |
| #10: NRF deployment clarifications |  |  |  |  |  |  |  | X |  |  |  |  |
| #11: Registered NF Profile changes for Inter-Slice Access |  |  |  |  |  |  |  |  | X |  |  |  |
| #12: Authorization of notification endpoint in “Subscribe-Notify” scenarios |  |  | X |  |  |  |  |  |  |  |  |  |
| #13: Authentication of NF Service Producer in Indirect Communication | X |  |  |  |  |  |  |  |  |  |  |  |
| #14: SCP trust domain or technical domain grouping |  | X |  |  |  |  |  |  |  |  |  |  |
| #15: Authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenario |  |  | X |  |  |  |  |  |  |  |  |  |
| #16: Selective End of End Protection of HTTP Request and Response in Indirect Communication |  |  |  |  | X |  |  |  |  |  |  |  |
| #17: Authorization mechanism negotiation using existing methods |  |  |  |  |  |  | X |  |  |  |  |  |
| #18: Avoiding slice isolation violation |  |  |  |  |  |  |  |  | X |  |  |  |
| #19: Hosted SEPP requirements |  |  |  |  |  |  |  |  |  |  |  | X |
| #20: PRINS for Roaming Hubs |  |  |  |  |  |  |  |  |  | X |  |  |
| #21: Certificate solution for NRF validation of NFc for access token requests |  |  |  |  |  |  |  |  |  |  | X |  |
| #22: Combined certificate and profile solution for NRF validation of NFc for access token requests |  |  |  |  |  |  |  |  |  |  | X |  |

## 6.1	Solution #1: Verification of the entity sending the service response in indirect communication without delegated discovery

### 6.1.1	Introduction

This solution is addressing KI#1.

This solution allows the NF Service Consumer (NFc) to verify the genuineness of the NF Service Producer (NFp), which is sending the response, when an SCP is used in between and the discovery of NFp has not been delegated to the SCP (see 3GPP TS 33.501 [X] Annex R, model C). I.e., The deployment scenario addressed is indirect communication (via SCP) without delegated discovery without re-selection.

The solution counters a malicious SCP or a Man in the Middle (MitM) that could forward the service request to a malicious or unauthorized NF Service Producer, i.e., a NFp that was not intended to provide a response. Especially where multiple SCPs are involved, and the NF Service Consumer does not know whether the right entity or some malicious entity is responding its request, this situation can occur.

Currently there is no means to prevent a malicious SCP or Man in the Middle to forward the service request to a different NF Service Producer.

The example shows a Service request with a token for service consumption from NFp that has been redirected by a malicious MitM to a different NF Service Producer, which can be a rogue NF cooperating with the MitM.

Figure 6.1.1-A: Example of a potential attacking scenario

This solution avoids that a service response is returned back to the NF Service Consumer by an unauthenticated and/or unauthorized MitM.

### 6.1.2	Solution details

NFc discovers NFp at NRF and requests an access token for a specific NFp Instance ID for consuming a service from NFp.

If indicated by NFc in the service request, the NFp provides back its CCA_NFp. Thus, the NFc can compare the NFp instance ID in the CCA_NFp with the selected NF instance ID when NFc requested the service. I.e., NFc can check if the NFp ID that the access token was provided for by NRF is matching the NFp ID present in the subject of CCA_NFp. Since NFp provides its CCA_NFp, this comparison is even possible if the response is sent via SCP.

If the NFp includes its own CCA_NFp in the service response, by this the NFc can verify that NFp, the sender of the service response, is the one that NFc's service request was sent to.

This allows authentication of NFp by NFc, i.e., by NFc verifying the CCA_NFp against the original NFp Instance ID, for which NRF provided the access token. NFp is authenticated, if the certificate NFp used to sign CCA has been verified by NFc. In case of failure, error messages can be triggered and reported to the operator.

In the following, the steps are described in detail.

Figure 6.1.2-1: Flow chart for allowing verification of NF sending the service response

Step1,2: NFc selects NFp to send a service request along with the token. To allow NFc to validate the service response, it will require validation of the producer's identity via CCA as part of the response.

Step 3: SCP intends to forward the service request to further SCPs. If SCP or some proxy is malicious (or MitM), it forwards the service request to a rouge NFp instead.

Step 4,5: A rouge NF can try to send the service response without performing the authorization. As the service request requires validation, the NFp has to add its CCA header, CCA_NFp.

Step 6: SCP will relay back the response to NFc including the CCA_NFp.

Step 7: NFc compare the NFp instance ID received and Set ID (if present) in the CCA_NFp with the one used for service request. If it is the same, then NFc is assured the service response is received from a genuine NFp.

Step 8: If it does not match, the NFc can also raise an alarm and revert the transaction at NFc.

### 6.1.3	Evaluation

This solution proposes an enhancement at the NF Service Producer to use the CCA as defined in TS 33.501. It provides an approach how an NF Service Consumer can authenticate NF Service Producer, from which NF Service Consumer received a service response, as intended NF for Service Response in indirect communication without delegated discovery.

This solution introduces Client credentials assertion of NF Service Producer which includes NFp Instance ID and signature using certificate of NFp. The NF Service Consumer can validate the CCA sent by the NF Service Producer and ensure that no rogue or malicious SCP or MitM has sent a service request to a malicious NF Service Producer.

This solution is only applicable in a very limited scope, it does not cover model D and the case when SCP reselects another NF as NF Service Producer which is different from the targeted NF Service Producer by NF Service Consumer. Therefore, it is possible for NF Service Consumer to reject the received service response from a legitimate NF Service Producer and may induce service unavailability. Further, the NF Service Producer cannot determine if the request coming from SCP which is using model D or SCP using model C or a re-selected by SCP, so the producer cannot determine, when to generate CCA_NFp.

## 6.2	Solution #2: Authorization between NFs and SCP

### 6.2.1	Introduction

This potential solution addresses KI#4.

### 6.2.2	Solution details

When sending the service request to SCP in delegated discovery, the NF Service Consumer must authorize the SCP to act on its behalf. Thus, NRF needs to be provided with evidence by NFc about the SCP instance ID.

NRF knows implicit the SCP instance ID because of direct TLS between SCP and NRF, when SCP ID would be added in a SCP TLS certificate. But this still does not assure that NFc as sent its request to this SCP. Thus, authorization between NF Service Consumer and SCP, when sending the service request to SCP in delegated discovery, has to be explicit. The solution proposes to do so by enhancing the CCA by inserting either the SCP Instance ID or the SCP Domain Info in CCA_NFc, and therefore the NF Service Consumer can authorize SCP.

NOTE: Since in model D the NF Service Consumer is delegating the discovery, as well as access token request, service request and receiving service response to SCP, the NF Service Consumer authorizes the SCP to perform these actions on its behalf.

The SCP also generates its own enhanced CCA_SCP including its Instance ID and/or its Domain Info and sends it along with access token request and the enhanced CCA_NFc as received from NF Service Consumer.

NFc

SCP

NRF

1. Service Request (optionally includes enhanced CCA')

* CCA' additionally includes Authorized SCP ID

2. Nnrf_AccessToken_Get_Request (includes CCA' and CCA,

CCA contains the SCP Instance ID in the subject parameter)

3. NRF analyzes the request, and authorizes the SCP

4. Nnrf_AccessToken_Get Response

(access token)

Figure 6.2.2-1: Authorization of SCP by NFc in indirect communication

The NRF verifies that the Target SCP Instance ID and/or SCP Domain info present in the CCA_NFc matches the Instance ID/Domain Info of SCP as also being part of the subject of the CCA_SCP. A successful verification of CCA(s) by NRF ensures that the SCP has been authorized by the NF Service Consumer.

Thus, the NRF needs to know the SCP Instance ID. One way for the NRF to learn the SCP instance ID is to use direct TLS between SCP and NRF, this requires SCP ID in the SCP TLS certificate which is currently not specified in TS 33.310. Another way for the NRF to learn the SCP instance ID is by CCA_SCP.

If authentication was successful and the NF Service Consumer is authorized based on the NRF policy and the SCP requesting the access token has been explicitly authorized by NF Service Consumer, the NRF issues an access token.

A similar solution is also applicable for authorizing SCP by NFc to request a service and receive a response from NFp on its behalf. The NFp then may perform similar verification and, in case of successful verification, can send the service response to SCP.

However, even if the TLS certificate of the NFc would mandate the usage of NFc Instance ID, another problem still needs to be solved in case the SCP selects another SCP. This is because if the NRF or the NF Service Producer do not know the SCP domain, to which the SCP belongs to, the SCP_CCA included by NFc does not help NRF. Thus, for this reason it is suggested that the TLS certificate needs in addition to SCP Instance Id also to hold the SCP domain identifier for allowing NRF to verify that NFc authorized one SCP of a SCP domain.

If the SCP, that NF Service Consumer delegated the authorization token request to, is not serving the NF Service Consumer request by itself, but demands another SCP' to do so, then the same procedure is needed between SCP and SCP'. Hence, SCP forward the service request to SCP' with its own CCA_SCP including into it the SCP' ID.

The verification of the CCA shall be performed by the receiving node as described in clause 13.3.8.3, but verifying that the SCP instance ID in the CCA is matching the SCP instance ID in the public key certificate used for signing the CCA. This is either done by an SCP, in case there are several SCPs in between, or by the NF Service Producer.

In practice, one would expect one to three SCPs between consumer and producer. But it needs to be noted, if the NF Service Producer wants to have verification of the full chain of trust via several SCPs, all CCAs and certificates from the NF Service Consumer and the intermediary SCPs need to be available to the NF Service Producer.

### 6.2.3	Evaluation

This solution fulfils requirement on KI#4. The SCP can be authorized to act on behalf of an NF Service Consumer and to request access tokens by NFc, because the NFc is including the SCP ID or SCP Domain ID into CCA_NFc. With NFc providing the SCP ID in the CCA, authorization of that particular SCP is given, because NRF or NF Service Producer can verify that the SCP, which provides CCA_NFc, is indeed the one SCP to which the NF Service Consumer sent its CCA and has authorized that SCP to request services and receive response on its behalf.

Thus, this solution counters a potential attack of SCP stealing a CCA and using it for requesting an access token without being requested by a NF Service Consumer.

The concept of CCAs is already known. In addition to verifying the NFc CCA the NF Service Producer needs to verify also the CCAs of one or several SCPs. In addition to the verification of the NF Service Consumer's CCA the NRF needs to verify also the CCAs of one or several SCPs. Further, SCPs need to be able to create their own CCAs.

## 6.3	Solution #3: Using existing procedures for authorization of SCP to act on behalf of an NF Consumer

### 6.3.1	Introduction

This solution addresses Key Issue #4 "Authorization of SCP to act on behalf of an NF or another SCP". It explains how token-based authorization and CCAs as currently specified in TS 33.501 [2] can be used to authorize the SCP to act on behalf of an NF Consumer, i.e., to request access tokens or services on behalf of the consumer. The solution relies on implicit authorization of the SCP using mutual TLS between NF Service Consumer and SCP.

### 6.3.2	Solution details

#### 6.3.2.1	Request of access token on behalf of the consumer

The SCP requests access tokens on behalf of the consumer in Scenario D (indirect communication with delegated discovery) and in Scenario C (indirect communication without delegated discovery) without mutual authentication between NF and NRF at the transport layer. The following procedure describes token requests for Scenario D, and particularly how CCAs are used to authorize the SCP to request access tokens on behalf of the NF Consumer. For Scenario C without mutual authentication between NF and NRF at the transport layer, the same principles hold.

Figure 6.3.2.1-1: Access token request of SCP on behalf of an NF Consumer

1.	The NF Service Consumer sends a service request to the SCP. The consumer includes a CCA signed by the consumer. The CCA includes the NF Instance ID of the consumer. The consumer's certificate used for signing the CCA also contains the consumer's NF Instance ID.

2.	The SCP sends an access token request to the NRF. The SCP includes the CCA received by the consumer in step 1.

3.	The NRF verifies the CCA as described in clause 13.3.8.3 of TS 33.501 [2] and thus obtains the NF Instance ID of the consumer that signed the CCA. Besides authentication of the consumer, the CCA also implicitly authorizes the SCP to act on behalf of the NF Service Consumer. 

The NRF authorizes the NF Service Consumer as described in TS 33.501 [2].

4.-8. The remaining steps of the access token request and service request procedure are exactly as described in TS 33.501 [2].

#### 6.3.2.2	Service request on behalf of the consumer

The SCP requests services on behalf of the consumer in all indirect communication scenarios. The following procedure describes access token and service requests for Scenario D, and particularly how CCAs and access tokens are used to authorize the SCP to request services on behalf of the NF Consumer. For Scenario C, the same principles hold.

Figure 6.3.2.2-1: Service request of SCP on behalf of an NF Consumer

1.-4. Service request and access token request and response are performed as described in the previous clause, clause 6.3.2.1.

5.	The SCP sends a service request to the NF Service Producer. The service request contains the access token and optionally the CCA received in step 1. The access token contains the NF instance ID of the NF Service Consumer.

6.	The NF Service Producer validates the access token as described in TS 33.501 [2]. Because the network implements the procedures described in the previous clause, clause 6.3.2.1, the NRF has already verified that the SCP was authorized to request the access token on behalf of the NF Service Consumer. Hence the access token does not only authorize the consumer, but also implicitly authorizes the SCP to act on behalf of the NF Service Consumer.

7.-8. The remaining steps of the access token request and service request procedure are exactly as described in TS 33.501 [2].

#### 6.3.2.4		Protection of the NF consumer's CCA

The CCA is protected in transport and storage by the following methods, partly in and partly out of 3GPP scope:

-	Transport protection: The CCA is protected in transport by TLS or other means, as specified in TS 33.501 [2], clause 13.1.0. Thus, it is protected between NF and SCP, and between SCP and NRF or NFp.

-	Storage protection: Although CCAs are expected to be short-lived, they could be cached for a short period of time at the NF Service Consumer. Similar as for other data handled at the NF Service Consumer, e.g., sensitive UE data, storage protection mechanisms outside of 3GPP scope need to be in place.

If used according to the procedure describes in clause 6.3.2.1, only the NF Service Consumer itself, the SCP and the NRF will obtain the CCA that allows to request access tokens on behalf of the NF Service Consumer. This solution assumes, that the SCP is authorized by the NF Service Consumer to request access tokens on behalf of it, the NF Service Consumer indicates that by sending the CCA to the SCP. The NRF is itself the entity that issues access tokens for the NF Service Consumer. Hence, if used according to the procedure described in clause 6.3.2.1, only entities that are authorized by the NF Service Consumer to request access tokens on behalf of it obtain the CCA.

### 6.3.3	Evaluation

The solution addresses the threats and requirements of Key issue #4: Authorization of SCP to act on behalf of an NF or another SCP.

The solution relies on token-based authorization and CCAs as currently specified in TS 33.501 [2].

It proposes that authorization of the SCP by the CCA is implicit by sending the CCA to the SCP, i.e., by presenting the CCA_NFc received by the NF Service Consumer, the SCP shows it is authorized to act on behalf of the Consumer and to request access tokens on behalf of it. However, authorization is not explicitly stated in the CCA. No e2e authorization information is provided from the NFc to NRF or NFp. Hence an entity that is not authorized by the NF Service Consumer but somehow has obtained a valid CCA signed by the consumer could use it to request access tokens on behalf of the consumer. Thus, in this case the NRF or the NFp can provide the service response to an unauthorized consumer.

## 6.4	Solution #4: Service request authenticity verification in indirect communication

### 6.4.1	Introduction

This solution addresses the KI#5.

### 6.4.2	Solution details

This solution allows the NF Service Producer to verify that a service request of the NF Service Consumer received via SCP has not been modified.

In case of CCA is used for authentication, the service request received by NRF or NF Service Producer can be verified as the one to be originally sent by the NF Service Consumer. This would guarantee that in indirect communication no intermediary can modify the service request unrecognized.

NOTE: To avoid backward compatibility issues and allow to distinguish between releases, a new API URI with the respective release version can be used.

For this, the CCA is enhanced with a new payload value for 'service request verification' and a protected header list.

- 	The 'service request verification' (SRV) includes the service request message (or a hash of it) as one of the payload values. If not the hash but the whole message or header is included, impact on the throughput is expected.

-	The protected header list (HL) includes custom headers that shall be integrity protected and thus not be modifiable undetected by SCP.

If present, the NF Service Producer or the NRF can verify whether these data included in the CCA are matching the service request as sent by the NF Service Consumer. I.e., the NF Service Producer verifies that the data included in the payload is matching the service request received together with the CCA. The receiver also verifies that the headers in the protected header list are not modified.

Since CCA is digitally signed by the NF Service Consumer, thus the recipient can verify that the service request received from SCP is the original one as provided by the NF Service Consumer. The additional SRV payload provides authenticity of the service request.

NOTE: This solution assumes that an SCP does not need to modify service request details for providing its service of delegated discovery and access token request to NRF or transferring a service request to the NF Service Producer. If there are headers that need to be modified by SCP/Proxy, then those headers cannot be considered as payload of SRV. The NF Service Consumer provides in this case a separate list of headers (HL) to explicitly state what is covered under SRV. The destination endpoint (NRF or NF) can take them in consideration while verifying the received data.

In detail:

- 	NF Service Consumer creates a service request and creates a keyed hash value about those parts of the service request, that are not to be modifiable by the SCP, and generates CCA including a 'service request verification' (SRV) payload with the keyed hash value. If necessary, a protected HL is included.

NOTE: A keyed hash value is not necessary, because the hash is signed digitally in the CCA.

- 	NRF, after verifying the authenticity of NF Service Consumer by checking the CCA, it checks SRV, i.e., it verifies the authenticity of the service request by creating a hash of the service request and comparing it with the received SRV value. It also verifies that the headers in the protected HL are not modified.

- 	NF Service Producer, after receiving an access token and CCA/SRV from the SCP, it verifies the NF Service Consumer by checking the CCA, it checks whether the NF instance id for which the access token was provided, matches the identity in CCA and it verifies the authenticity of the service request by creating a hash of the service request and comparing it with the received SRV value. It also verifies that the headers in the protected HL are not modified.

### 6.4.3	Evaluation

This solution provides an approach how an NF Service Producer can verify that a service request of the NF Service Consumer received via SCP has not been modified.

This solution extends Client credentials assertion to include new payload value for service request verification and a protected header list.

When the service request verification includes whole service request message, which may double the size of the message and may impact on system throughput.

When the service request verification includes hash value of service request message, additional information should be transmitted to the NF Service Producer to inform HTTP headers and order among HTTP headers which should be considered in calculation of hash value.

When SCP appends HTTP standard header(s) such as Via header and Authenticate header, in this solution, NF Service Producer cannot recognize those headers should not be considered in calculation of hash as those are added by SCP and NF Service Producer will fail to calculate correct hash value of HTTP message.

## 6.5	Solution #5: End-to-end integrity protection of HTTP body and method

### 6.5.1   	Introduction

This solution addresses the key issue #5 (End-to-end integrity protection of HTTP messages).

The core steps of this solution are:

- 	Use Client Credentials Assertions (CCAs) based authentication as specified in TS 33.501 [2] Clause 13.3.8 for NF-NRF or/and NF-NF communication.

- 	Enhance the Client Credentials Assertions (CCAs) to optionally include a hash of the HTTP body and HTTP method to protect the message itself.

- 	The receiving node (NRF or NF producer) computes the hash of the HTTP body and HTTP method and validates that it is identical to the hash received in the Client credentials assertions (CCAs).

Since the added hash is an optional field in the ClientCredentialsAssertion as specified in 3GPP TS 29.500 [5] Table 5.2.3.2.11-1, this solves the backwards compatibility with Rel-16 NF producers supporting only existing CCA. A Rel-16 NF Service Producer will verify the signature of the CCA correctly but ignore the optional field that it does not recognize. The behaviour is similar to Rel-15 NF Service Producers' behaviour for IEs in access tokens that were introduced in Rel-16. As specified in TS 29.510 [6], Table 6.3.5.2.4-1 "Definition of type AccessTokenClaims", if an NF Service Producer receives an IE in the access token that it does not understand, the NF Service Producer ignores the IE. Similar behavior can be specified for IEs in the CCA, see Table 6.5.2-1 below.

### 6.5.2 	Solution details

Figure 6.5.2-1   CCA based Authentication with HTTP hash enhancement

1.	NF Service Consumer sends a service request including a signed Client credentials assertion (CCA) token to authenticate against NF Service Producer or NRF as described in TS 33.501 [2] Clause 13.3.8. But for this solution it is also proposed to add an optional field in CCA to protect the part of the message itself. The added field is a hash of HTTP body and HTTP method.

2.	NF Service Producer or NRF validates the CCA as described in 3GPP 33.501, clause 13.3.8.3. But since one optional field is supposed to be added to the CCA, the receiving end point (NF Service Producer or NRF) also needs to compute the hash of the HTTP body and HTTP method and validates that it is identical to the hash received in the Client Credentials Assertion.

The updated definition of type ClientCredentialsAssertion in 3GPP 29.500 [5] is (additions in bold style):

Table 6.5.2-1: Updated CCA based on Table 5.2.3.2.11 -1: Definition of type ClientCredentialsAssertion

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sub | NfInstanceId | M | 1 | This IE shall contain the NF instance ID of the NF service consumer, corresponding to the standard "Subject" claim described in IETF RFC 7519 [41], clause 4.1.2. |
| iat | integer | M | 1 | This IE shall indicate the time at which the JWT was issued, corresponding to the standard "Issued At" claim described in IETF RFC 7519 [41], clause 4.1.6. This claim may be used to determine the age of the JWT. |
| exp | integer | M | 1 | This IE shall contain the expiration time after which the client credentials assertion is considered to be expired, corresponding to the standard "Expiration Time" claim described in IETF RFC 7519 [41], clause 4.1.4. |
| aud | array(NFType) | M | 1..N | This IE shall contain the NF type of the NF service producer and/or "NRF", for which the claim is applicable, corresponding to the standard "Audience" claim described in IETF RFC 7519 [41], clause 4.1.3. |
| hash | string | O | 0..1 | This IE contains a hash of the body of the HTTP message and HTTP method. If an NF service producer that receives this IE in the CCA included in the 3gpp-Sbi-Client-Credentials header does not understand this IE, it shall be ignored. |
| halg | string or integer | O | 0..1 | This IE contains the hash algorithm information that is used by NF service consumer to compute the hash of the HTTP message. If an NF service producer that receives this IE in the CCA included in the 3gpp-Sbi-Client-Credentials header does not understand this IE, it shall be ignored. |

The details of the hash are proposed to be specified as following:

Option 1: For computation of the hash of the HTTP body and HTTP method for inclusion into the Client credential assertion, the input S to the KDF specified in Annex B of 3GPP TS 33.220 [4] is computed as follows:

-	P0 = HTTP body;

-	L0 = length of the HTTP body;

-	P1 = HTTP method;

-	L1 = length of HTTP method.

The input key KEY is equal to null. Note that the FC value will be allocated in the normative phase.

Option 2: Alternatively to using the fixed KDF as hash function, the choice of hash function can also be done similar as in JWT or JWS. The hash algorithm is chosen by NF Service Consumer. The selection of hash algorithm needs to be aligned between HTTP message sender and HTTP message receiver, i.e., mandatory to support algorithms need to be specified in a 3GPP profile. This option provides more crypto agility and is better aligned with JWT and JWS. For ease of implementation in initial deployments, the 3GPP profile for the hash algorithm could mandate the usage of a specific hash function, e.g., SHA256. This is similar to the JOSE profile of PRINS as specified in TS 33.501 [2], clause 13.2.4.9, which specifies the usage of specific AEAD and signature algorithms, but still provides crypto agility if changes should be necessary in the future.

### 6.5.3	Evaluation

This solution provides an approach how an NF Service Producer can verify that a service request of the NF Service Consumer received via SCP has not been modified.

This solution extends Client credentials assertion to include hash value of HTTP body and HTTP methods.

This solution does not handle integrity protection of HTTP headers.

## 6.6	Solution #6: Verification of Service Response from a NF Service Producer at the expected NF Set

### 6.6.1	Introduction

This solution addresses key issue #1. It extends solution #1. In order to verify the message from NF Service Producer in indirect communication, it is proposed to append CCA of NFp. An NF Service Consumer may accept the certificate if it is verified well and NF Service Producer instances belongs to the expected NF Producer instance(s).

### 6.6.2 	Solution details

#### 6.6.2.1 	For indirect communication without delegated discovery procedure

Figure 6.6.2.1-1: With mutual authentication between NF and NRF at the transport layer

Discovery of the NF Service Producer:

0. When a NF Service Consumer discover a NF Service Producer for a service, NRF provides information of target NF set and candidate target NF instance IDs belonging to the target NF set.

The NF set information in the discovery response from NRF to NF consumer needs to be end-to-end integrity protected, by, e.g., TLS or solution to Key Issue #5, so that the SCP cannot modify the NF set information in the discovery response.

NF Service Consumer authorization:

1-2. After mutual authentication between NF Service Consumer and NRF at the transport layer, the NF Service Consumer and NRF perform the "Access token request before service access" procedure. If the NF Service Consumer has already discovered the NF Service Producer, it can also perform the "Access token request for a specific NF Service Producer/NF Service Producer instance" procedure.

Service Request:

4. Among the candidates NF instances list, the NF Service Consumer may select an NF instance for a Service Request. And the NF Service Consumer keep the list of candidate NF instances and NF set for verification of expected Service Response.

After acquiring an access token from the NRF, a NF Service Consumer may send a Service Request to the SCP.  The service request includes the access token and CCA of the NF Service Consumer.

The service request includes the 3gpp-Sbi-Routing-Binding header and/or 3gpp-Sbi-Discovery header in order to specify target NF Service Producer and/or target NF Set, so that the SCP is instructed to perform the NFp reselection within the scope of NF Set.

5. An SCP forward a Service Request to the NF Service Producer. If needed, the SCP may reselect another NF Service Producer belonging to the same NF Set.

Service Response:

6-7. After receiving a Service Request, the NF Service Producer may verify the Service Request and may respond with a Service Response with CCA of the NF Service Producer. CCA of NF Service Producer includes NF instance ID of NF Service Producer and NF instance ID of NF Service Consumer.

8-9. When receiving a Service Response, the NF Service Consumer may verify whether the NF instances ID of NF Service Producer which sends the Service Response is in the list of candidate NF instances for the Service Request.

#### 6.6.2.2 	For indirect communication with delegated discovery

Figure 6.6.2.2-1: for indirect communication with delegated discovery

1.	The NF Service Consumer sends a service request to the SCP. The service request shall include NF service discovery factors such as target NF type and the NF Service Consumer's CCA as defined in clause 13.3.8.

2.	The SCP may perform a NF discovery operation with the NRF using NF service discovery factors received in step 1.

3. 	(same with step 3 in subclause 13.4.1.3.2 TS33.501.) The SCP sends an access token request (Nnrf_AccessToken_Get Request) to the NRF. The access token request includes parameters as defined in clause 13.4.1.1. The access token request may include the NF Service Consumer's CCA if received in Step 1.

4.	(same with step 4 in subclause 13.4.1.3.2 TS33.501.) The NRF authenticates the NF Service Consumer using one of the methods described in clause 13.3.1.2. If NF Service Consumer authentication is successful and the NF Service Consumer is authorized based on the NRF policy, the NRF issues an access token as described in clause 13.4.1.1. The NRF uses the NF Service Consumer instance ID as the subject of the access token.

5.	(same with step 5 in subclause 13.4.1.3.2 TS33.501.) The NRF sends the access token to the SCP in an access token response (Nnrf_AccessToken_Get Response).

6.	(same with step 6 in subclause 13.4.1.3.2 TS33.501.) The SCP sends the service request to the NF Service Producer. The service request includes an access token (i.e., received in Step 1, received in Step 5, or previously cached), and may include the NF Service Consumer's CCA if received in Step 1.

7.	(same with step 7 in subclause 13.4.1.3.2 TS33.501.) The NF Service Producer authenticates the NF Service Consumer by one of the methods described in clause 13.3.2.2 and if successful, it validates the access token as described in clause 13.4.1.1.

8.	If the validation of the access token is successful, the NF Service Producer may respond with a Service Response with CCA of the NF Service Producer. CCA of NF Service Producer may include NF type and NF instance ID of NF Service Producer and NF instance ID of NF Service Consumer.

9.	The SCP forwards the service response to the NF Service Consumer. The SCP may include the access token in the service response to NF Service Consumer for possible re-use in subsequent service requests.

10.	When receiving a service response, the NF Service Consumer may verify whether the NF Service Producer belongs to the target NF type and authenticate NF Service Producer using CCA and X.509 certificate of the NF Service Producer, and may verify whether the CCA of the NF Service Consumer in the CCA of NF Service Producer is the same with the CCA of the NF Service Consumer sent in Step 1.

#### 6.6.2.3 	Client credentials assertion of NF Service Producer

CCAs shall be JSON Web Tokens as described in RFC 7519 [44] and are secured with digital signatures based on JSON Web Signature (JWS) as described in RFC 7515 [45].

The CCA of NF Service Producer may include:

-	the NF instance ID of the NF Service Producer;

-	the NF set information of the NF Service Producer;

-	the NF instance ID of the NF Service Consumer;

-	The NF type of the NF Service Producer;

-	The CCA of the NF Service Consumer received in the service request message;

-	A timestamp and an expiration time.

The NF Service Consumer shall digitally sign the generated CCA based on its private key as described in RFC 7515 [45]. The signed CCA shall include one of the following fields:

-	the X.509 URL (x5u) to refer to a resource for the X.509 public key certificate or certificate chain used for signing the client authentication assertion, or

-	the X.509 Certificate Chain (x5c) include the X.509 public key certificate or certificate chain used for signing the client authentication assertion.

### 6.6.3 	Evaluation

This solution provides an approach how an NF Service Consumer can authenticate NF Service Producer, from which NF Service Consumer receives a service response, as intended NF for Service Response in indirect communication without delegated discovery and with delegated discovery.

This solution introduces Client credentials assertion or a token alike also for the NF Service Producer. The assertion includes NFp Instance ID, NFc Instance ID, and a signature using the certificate of NFp.

In indirect communication without delegated discovery, by reusing existing HTTP custom headers, it can also cover the case when SCP reselect another NF as NF Service Producer.

This solution works with assumption that the discovery results from NRF to NF Service Consumer are protected to detect any harmful modification in the middle. And it also assumes that NRF will inform NF Service Consumer about which NF Service Producers are in the NF Set and that SCP only re-selects another NF Service Producer within the NF Set.

NFc can only trust NFp if there has been a direct possibility to verify NFp's authenticity. This is done by proposing an assertion token (such as CCA) to be also used by NFp.

In indirect communication with delegated discovery, this solution requires extension of CCA and/or X.509 Certificate of NF Service Producer to include NF type of NF Service Producer.

This solution is to address KI#1 which basically assumes that the SCP and NFp are compromised or at least the SCP is compromised. If that the threat this solution is trying to address, thus the proposed solution only prevents such attack in the case when NF and NRF are mutually authenticated using TLS over direct communication without SCP being present. This means this solution does not address KI#1 in the following cases:

- Delegated Discovery, Model D,

- Model C when the NF Service Consumer communicates with NRF over indirect communication via SCP.

## 6.7	Solution #7: Access token request for NF Set

### 6.7.1	Introduction

This solution addresses KI#6.

3GPP introduces the concepts of NF Set and NF Service Set which allows essentially for a group of interchangeable NF instances/NF Service instances of the same type, supporting the same services and the same Network Slice(s). Rel-16 also allows re-selection of a NF instance or a NF Service instance within the Set for subsequent transaction.

5G SBA architecture design allows for the concept of stateless NFs.

To support the NF Set concept, i.e., a group of interchangeable NF instances or NF Service instances providing the same service, the context needs to be accessible to all NFs of the set.

The solution addresses NF Sets or NF Service Sets on the consumer side. It assumes that each NF Service Consumer of a set is known at NRF also with its NF Set ID or the NF Service Set ID. Thus, verification of the correctness of a set id is done when authenticating the NF Service Consumer as OAuth 2.0 client when registering at NRF. Thus, if NRF is then issuing an access token with a distinct set id related to the NF Service Consumer's NF Set IF or NF Service Set ID, the NF Service Producer can trust the correctness, or do another verification, if the set id is also included in CCA or NF certificate.

A NF Service Producer can also indicate in its profile, if it is allowing the NRF to provide access tokens for NF Sets or NF Service Sets.

NOTE: Whether to have this feature allowed per operator policy configured at NRF or per NF Service Producer or NF Service Producer Set is a deployment decision.

The solutions objective is to avoid that a NF from a NF Set needs to request a new access token, when targeting a service of an existing resource requested before by another NF of the NF Set, it is proposed that any NF in a NF Set can request an access token for the NF Set. Thus, any NF Service Consumer targeting a service of an existing resource it can use the access token provided to a NF Set of NF Service Consumers. All NFs of a NF Set need to retrieve the current status from the same place to allow for the same access token to be used by different NFs of the set.

NOTE: For any NF to make use of this solution, that NF is required to register its profile with the NRF.

### 6.7.2	Solution details

The NF Service Consumer belonging to a NF Set, it includes its NF Set ID in the Access Token Request message to NRF and also in the CCA or the NF certificate.

When the Access Token Request is processed by the NRF and a NF Set ID is included, the NRF knows that the NF Service Consumer requests an access token to be useable by all NF Service Consumer instances within the NF Set. If NRF authorization of the NF Service Consumer is successful, i.e., the NF Service Producer has indicated that an access token for a NF Set or NF Service Set can be issued, and the NF Set ID in the CCA matches the NF Set ID in the access token or in the NF certificate, NRF includes as claim the NF Set ID of the expected NF Service Consumer instances to allow the access token generated for usage by all NF Service Consumers in the NF Set. NRF sends the access token back to the requester. The request needs to put the context information of such common token at a place from which all NFs of the NF Set can retrieve it.

Figure 6.7.2-1 – Access Token Request procedure (TS 33.501 Figure 13.4.1.1.1-1) enhanced with NF Set ID in the Access Token Request message

How NFs of a NF Set or a NF Service Set manage the distribution of an access token issued for set or service set and their availability to other NFs within the NF Set, is for implementation and out of scope.

When a service is requested, the requester (NF Service Consumer or SCP) includes the NF Set ID of the NF Service Consumer in the Service API Request, as well as in the CCA, if the CCA is sent, in addition to the access token obtained from the NRF. NF Set ID in CCA is only reliable if the NF Set ID is included in the certificate related to the private key that the NF Service Consumer used to sign the CCA.

The NF Service Producer checks whether the Consumer NF Set Id in the Service Request matches with the NF Set ID claim in the Access token. If CCA is sent, it also verifies, if the NF Set ID matches the NF Set ID in the CCA. If included in NF certificate, it can also match the NF Set ID with the NF Set ID in the NF certificate. If yes, it proceeds with serving the request, otherwise it rejects the request.

There is always one NF representing a set at a given time. Only one NF of the NF Set or NF Service Set shall be able to use the access token issued for a set at a given time. The management of which NF within the set is currently the leader and using the OAuth token is for implementation and out of scope.

### 6.7.3	Evaluation

The solution proposed allows the authorization server, i.e., NRF, to issue an access token that can be used by all members of an NF Set or NF Service Set. The concept of NF Set and NF Service Set has been introduced by 3GPP. This solution enables optimization that is sought from the mutual redundancy among the NF instances of the set. It would be less optimized if each instance needs to request its own token.

Using the same access token for a NF Service Consumers belonging to one NF Set is not explicitly described by RFC 6749. Other literature mentions group access tokens, but further investigation on the impact managing an access token used by NF Service Consumers of the same set is needed.

According to RFC 6749, each NF instances needs to register with the authorization server (NRF) as a separate OAuth2.0 client before the authorization server is able to issue such a token which can be used by all members of the NF Set.

Since CCA is used for Indirect communications when SCP in the path between the NF Service Consumer and the NF Service Producer, including NF Set ID and/or NF Service Set ID into the CCA only work for the case of indirect communication but not in the direct communication case.

Including NF Set ID in the NF certificate is not a flexible mechanism which requires an intervention in case of the NF instance is removed from a specific NF set ID and/or NF service set ID or added to another NF Set ID or NF Service Set ID. On the other hand, if any of these operations are done to any NF instance, the NF instance will update its profile with the NRF automatically and the update is almost dynamic for the rest of the processes.

This solution requires that in case of any change to the list of members of the NF Set, all existing access token with the impacted NF Set ID and/or NF Service Set ID should be destroyed and not used. A new access token is required.

Since NF Service Producers can indicate in their profile allowedxxx IEs, configuration effort is necessary so that they are consistent with the NF Sets.

This solution requires each NF Service Consumer to register its profile and maintains it always updated at all the time with the NRF.

From a security perspective, having an access token issued for a specific NF service consumer to be used to authorize a single NF service consumer instance at a set of NF producer is quite different than having a single access token issued for a group of NF service consumers to receive service at a specific NF service producer or a set of service producers.

In the first case, only one NF service producer receives the access token at a time to validate the NF service consumer instance whether it is allowed to receive the service or not. Each NF service producer will have the ability to validate whether the NF service consumer instance was authorized or not via the access token validation process. There is no security risk what so ever in this scenario as the access token is issued to authorized a single entity, the NF service consumer instance. It is important to note that the NF service producer is NOT required to maintain or save the access token.

However, in the case of a set of NF service consumers sharing the same access token, the access token may ends up being saved at multiple places rather than one which causes a security risk. In addition, the access token does not reflect the identity of the NF service consumer instance that is requesting access to the service resource at the NF service producer and this way the NF service producer is not able to validate whether the NF service consumer instance requesting the service is authorized to receive service or not.

In comparison, that is why no certificate is issued for multiple entities to be used by a group of entities, e.g., NF service consumers set, to be used to blindly authenticate a member of the group., for example.

## 6.8	Solution #8: Integrity protection of HTTP message in consideration of update by SCP

### 6.8.1	Introduction

This solution addresses key issue #5.

It is proposed to use enhance CCA to include hash value of HTTP headers and HTTP body.

In the enhanced CCA, hash value of HTTP headers and hash value of HTTP body are included, separately.

For calculation of hash value of HTTP headers, Via and Authorization headers are not included.

There should be a rule for ordering HTTP headers. HTTP standard headers come first and HTTP custom headers come after HTTP standard headers. Among HTTP custom headers from NF Service Consumer or NF Service Producer, 3GPP-Sbi-Client-Credentials header should be positioned as the last once. When an SCP add HTTP custom headers, those should come after 3GPP-Sbi-Client-Credentials header from NF Service Consumer of NF Service Producer.

When NF Service Consumer of NF Service Producer receive an HTTP message, they compute the hash value of HTTP headers and HTTP body and compare those with the values in CCA for checking of integrity protection.

Figure 6.8.1-1: HTTP message with hash value in CCA for end to end message protection

### 6.8.2	Solution Details

This solution enables the NF Service Producer and NF Service Consumer to verify a HTTP message received via SCP has not been modified.

NF Service Consumer calculate hash value of HTTP standard headers except Authenticate header and HTTP custom headers and hash value of HTTP body and include in the CCA.

NF Service Consumer append CCA as the last of HTTP custom headers.

For this, the CCA is enhanced with additional payload values.

- 	hash value of HTTP headers including HTTP standard headers and HTTP custom headers generated by originating Network Function.

-	hash value of HTTP body generated by originating Network Function.

After SCP receives HTTP message from NF Service Consumer, it may append Via header and Authenticate header (only for delegated discovery case) and some HTTP custom headers after HTTP standard headers and HTTP custom headers at the received HTTP message, respectively.

NF Service Producer verify hash value of HTTP standard headers and HTTP extended headers except Via header, Authorization header and HTTP custom headers appended by SCP and hash value of HTTP body by comparing the calculated value and the ones in CCA.

Similarly, NF Service Producer can calculate hash value of HTTP standard headers and HTTP custom headers and hash value of HTTP body and include in the CCA if CCA is used.

After SCP receives HTTP message from NF Service Producer, it may append Via header and some HTTP custom headers after HTTP standard headers and HTTP custom headers at the received HTTP message, respectively.

NF Service Consumer can verify hash value of HTTP standard headers and HTTP extended headers except Via header and HTTP custom headers appended by SCP and hash value of HTTP body by comparing the calculated value and the ones in CCA.

### 6.8.3 	Evaluation

This solution provides an approach how an NF Service Producer can verify that a service request of the NF Service Consumer received via SCP has not been modified.

This solution extends Client credentials assertion to include hash value of HTTP headers and HTTP body.

This solution propose a rule how to compose the HTTP headers and how to calculate hash value of a HTTP message and it allows NF Service Producer to calculate hash value of HTTP headers without any extra information.

This solution provides how to calculate hash value of HTTP headers and HTTP body even though SCP modify HTTP messages in NF Service Producer. And when SCP modifies illegally a HTTP message, NF Service Producer can detect it.

NOTE: Whether a token similarly to CCA is used for NF Service Producer authentication is per decision of key issue #1.

## 6.9	Solution #9: Authorization mechanism negotiation

### 6.9.1	Introduction

This solution addresses Key Issue #7 "Authorization mechanism negotiation". It is proposed to use the two NRFs for the authorization mechanism negotiation.

### 6.9.2	Solution details

Figure 6.9.2-1: Authorization mechanism negotiation

Pre-requisite:

-	the vPLMN and hPLMN support at least static authorization dependent on the roaming partner

1.	The NF Service Consumer sends a discovery request to the vNRF.

2.	The vNRF sends a discovery request to the hNRF. In addition to the discovery request from the NFc, the vNRF add the vPLMN authorization Capability into the discovery request. The vPLMN authorization Capability indicates the supported authorization mechanisms, i.e., static, OAuth, or Both.

3.	The hNRF selects the Final authorization mechanism supported by both the received vPLMN authorization Capability and the hPLMN authorization Capability. The hPLMN, i.e., all NF producers of the hPLMN that handle service requests from other PLMN, need to support at least static authorization. In this way, the hNRF can always select an authorization method that is supported by both the vPLMN and the hPLMN.

NOTE: The Final authorization mechanism selected by hNRF is depend on operator.

4-5. The hNRF sends the Final authorization mechanism to the NFc.

Then, if the Final authorization mechanism indicates static authorization, then the NFc could use the static authorization to access the NFp service. If the Final authorization mechanism indicates OAuth authorization, then the NFc could get the token from the NRF before consuming the service from the NFp.

### 6.9.3	Evaluation

This solution addresses the threats and requirements of Key issue #7: "Authorization mechanism negotiation".

The pre-requisite is that the hPLMN supports at least static authorization.

This solution addresses for the inter-PLMN scenario the use case that the vPLMN has only implemented static authorization. The NF Service Producer needs to support whichever authorization mechanism is determined by the hNRF, i.e., using different authorization mechanisms depending on the roaming partner.

In the solution proposed, the VPLMN supporting static authorization only could enforce the HPLMN to overcome the mandated support for OAuth. I.e. to demand static authorization by only providing this specific capability to the roaming partner.

## 6.10	Solution #10: NRF deployment clarifications

### 6.10.1	Introduction

This solution addresses key issue #8. It provides clarification of handling access token requests in different NRF deployment scenarios.

### 6.10.2	Solution details

There are different deployment options for NRFs, as described in TS23.501 (see clause 5.15.5).

The NF Service Consumer may have discovered a specific NRF in advance, e.g., a slice specific NRF, and can send its request directly to this specific NRF. In this case, if the specific NRF is not the NF Service Consumer's local NRF, the authorization server part of this NRF does not have a record of this NF Service Consumer's Oauth2.0 client registration, when information about the NF Service Consumer instance and its NF Type is made available in the OAuth 2.0 Authorization server, i.e., the slice specific NRF.

This registration process is subject to implementation procedures of the operator, with the following consideration on authentication procedure: OAuth 2.0 clients are capable to authenticate securely with the authorization server, i.e., client type as specified in RFC 6749 [35] is "confidential".

If the NF Service Consumer requests an NRF, where the NF Service Producer is not registered (see NRF deployment options), the requested NRF needs to redirect/forward the service request to that NRF.

In a local NRF deployment, the NF Service Producer only gets the certificate of its local NRF. Thus, the local NRF of the NF Service Producer would need to trust the forwarding NRF that has authenticated the NF Service Consumer before the local NRF be able to authorize the NF Service Consumer.

### 6.10.3	Evaluation

The solution proposes that information about the consumer instance and its type is made available in the slice specific NRF. This is technically possible, potentially quite complicated. It is applied to an optimization in TS 23.501on a slicing-related procedure. Whether the procedure with this addition is still an optimization, would need further evaluation.

This proposal requires pre-configuration of the slice-specific NRF for solving a very specific issue of an optimization in TS 23.501 on a slicing-related procedure.

## 6.11	Solution #11: Registered NF Profile changes for Inter-Slice Access

### 6.11.1	Introduction

This solution proposes to address following security requirement of KI #9:

- 	The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

This solution gives the option for NF Service Producers to indicate which consumer’s S-NSSAIs are allowed to access which producer’s S-NSSAIs, and by which operations in its profile.

### 6.11.1	Solution Details

Using individual or combination of IEs like allowedNfTypes and allowedNssais, NRF can already filter the requests from NF Service Consumers not allowed to access any services of an NF Service Producer.

To address the issue of static configurations at NRF for restricted Inter-slice access, it is proposed that:

-	When an NF Service Producer registers its profile in NRF, it includes a new information element, “allowedSnssaisPerSnssai”, which specifies, for each NF-Service Consumer’s S-NSSAI, the list of NF-Service Producer’s S-NSSAIs it is allowed to access.

-	When an NF Service Producer registers its profile in NRF, it includes a new information element, “allowedOperationsPerSnssai”, which specifies allowed operations on its resources, for NF Consumers belonging to the given S-NSSAI.

These attributes can then be used by NRF to determine the “producerSnssaiList” to be included in the AccessTokenClaims. Additionally, NRF can determine whether a given resource/operation-level scope can be granted to an NF Service Consumer that requested an Oauth2 access token with a specific scope, or limit the scope or authorization irrespective of the request.

Additionally, to allow NF Service Producers (themselves) validate the “Inter-Slice” access requests from the NF Service consumers, it is proposed to include “Requester-NSSAI” in the access-grant, indicating the NRF validated NF-Consumer’s S-NSSAIs.

The NF Service Producer can thus check the decision of the NRF to assign an access token, but is still dependent on the NRF validating the consumer’s S-NSSAI and including it in the access token.

### 6.11.3	Evaluation

This solution proposes to address the following security requirement of KI #9:

- 	The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

1. Finer granularity for NF Producer’s profile:

The solution gives the option for NF Service Producers to indicate which consumer’s S-NSSAIs are allowed to access which producer’s S-NSSAIs, and by which operations, in its profile.

2. NFp authorizing NF Consumer’s S-NSSAI:

In this solution, the NF Service Producer is still dependent on NRF to include the validated S-NSSAIs, how NRF validates the NF Service Consumers is not covered in this solution.

## 6.12	Solution #12: Authorization of notification endpoint in “Subscribe-Notify” scenarios

### 6.12.1	Introduction

This solution addresses key issue #3: Service access authorization in the “Subscribe-Notify” scenarios by including “notification URI” in the token request.

This solution proposes to include “notification URI” and CCA of the NF which owns the “notification URI” at the access token request. The NRF verifies whether the notification URI is authorized to receive the service/notification that is requested in the access token request.

Editor’s Note: It is ffs how the solution works in case of notification target reselection as described in clause 6.3.1.0 of TS 23.501.

### 6.12.2 	Solution details

During access token request process, NF Service Consumer include "notification URI" in Nnrf_Access Token_Get Request when operation semantics of the requested service type is "Subscribe/Notify". NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. In addition, when "notification URI"(s) are included in the access token request, NRF may verify whether the "notification URI"(s) locate its location(s) with the same address(es) which are indicated by its own FQDN or IP address as registered in NRF. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s).

When the NF Service Consumer is authorized, the NRF shall then generate an access token with appropriate claims which may include “notification URI” as requested by NF Service Consumer.

Figure 6.12.2.-1: Access token request for "Subscribe-Notify" NF Service illustration 1

A NF Service Consumer (NF_A) may request to subscribe to NF Service offered by a NF Service Producer(NF_B) on behalf of NF_C according to the Service Request from NF_C (e.g., Figure 4.15.3.2.2-1: Nudm_EventExposure_Subscribe, Unsubscribe and Notify operation in TS23.502).

The Figure 6.12.2-2 describe the solution to verify the service request from NF_A including Notification URI of NF_C.

Figure 6.12.2-2: Access token request for "Subscribe-Notify" NF Service illustration 2

Step 0.	The NF_C sends a Service Request to the NF_A for notification service on an event with an access token including notification URI and CCA of the NF_C.

Step 1.	The NF_A decides to subscribe a service of the NF_B associated to the Service Request received at step 0 and sends an Access Token Request to the NRF for a Service Request toward the NF_B including notification URI and CCA of the NF_C.

Step 2.	The NRF verifies whether the NF_A is authorized and whether the NF_C identified by the appended CCA is a valid Network Function and authorized to receive the requested NF Service from the NF_B. NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF_C of the CCA of the NF_C, or those in the NF profile of the NF_C.

Step 3.	Upon successful verification, the NRF publishes an access token for a Service Request of the NF_A toward the NF_B. The access token includes notification URI of the NF_C in the claims as requested in step 2.

Step 4.	The NF_A sends a Service Request for a notification service toward NF_B including the access token received at step 3 and CCA of the NF_A.

Step 5.	The NF_B verifies the access token and CCA of the NF_A. During the verification, the NF_B can check whether notification URI in the Service Request are included in the access token. If the verification is successful, the NF_B may respond the NF_A and provide the NF_C with the notification service as requested by the NF_A. In case of notification target reselection as described in clause 6.3.1.0 of TS 23.501, binding can be used to indicate suitable NFs other than NF_C (see 23.501, Table 6.3.1.0-1 on binding, selection and reselection).

### 6.12.3 	Evaluation

The solution extends the existing token-based authorization method by including the notification URI of the NF_C in the token claims.

The token is used to indicate towards NF_B (the producer and sender of the notification) that NF_C (the notification receiver) authorizes NF_A (the subscriber) to subscribe services on behalf of NF_C (the notification receiver). The token is also used to indicate towards NF_B (the producer and sender of the notification) that the NRF authorizes the notification URI contained in the token to receive notifications from NF_B.

The solution has impact on the access token generation by adding the notification URI of the NF_C into the access token, and token verification.

Editor’s Note: Further evaluation is FFS.

## 6.13	Solution #13: Authentication of NF Service Producer in Indirect Communication

### 6.13.1	Introduction

This solution addresses KI#1.

This solution proposes a simple authentication procedure which allows an NF Service Consumer or any other entity (e.g., a delegated SCP) to authenticate an NF Service Producer before sending an actual service request. This ensures that an actual service request with request parameters, some of which may be sensitive (e.g., SUPI), is only sent to an NF Service Producer that has already been authenticated.

This property is consistent with the behaviour of HTTPS/TLS in which a client certificate with client information is sent to the server only after the authentication of a server certificate, and an HTTPS request is sent to the server only after the authentication of the server in TLS.

This simple authentication procedure can be executed by an NF Service Consumer or by an SCP which has been delegated by an NF Service Consumer to perform service discovery and re-selection on its behalf.

### 6.13.2	Solution details

A Client (e.g., an NF Service Consumer) first performs the service discovery to obtain information about an NF Service Producer (e.g., NF Service Producer Instance ID). Before the Client sends a service request to the NF Service Producer, it performs the following procedure to obtain a Server Credentials Assertion (SCA) from the producer. Similar to CCA, which allows to verify the identity of a consumer, SCA allows to verify the identity of the producer.

This procedure can be performed by an NF Service Consumer to verify the identity of an NRF or an NF Service Producer. For example, it can be performed to verify the identity of an NRF before an NF Service Consumer sends an access token request to an NRF.

In delegated discovery, this procedure can be performed by an NF Service Consumer before sending an actual service request. This procedure will trigger the SCP to perform the first service discovery and eventually return the SCA to the NF Service Consumer. Alternatively, if a delegated SCP is trusted by an NF Service Consumer to verify the identity of an NF Service Producer, an SCP will perform this procedure on the behalf of NF Service Consumer to verify the identity of an NF Service Producer before it forwards a service request from an NF Service Consumer to the NF Service Producer.

Editor’s Note: It is for further study whether the threats and security requirements are addressed if the SCP performs the procedure on behalf of the NF Service Consumer.

When a re-selection of an NF Service Producer is performed, this procedure can be re-performed against the newly selected NF Service Producer.

Editor’s Note: It is for further study whether re-performing the procedure after reselection addresses the key issue threats and requirements.

Figure 6.Y.2-1 Authentication Procedure for NF Service Producer in Indirect Communication

1. A client (e.g., an NF Service Consumer or an SCP) sends an HTTP request to a server (e.g., an NRF or an NF serviced producer) to obtain its Server Credentials Assertion (SCA). This HTTP request can be a simple HTTP Get request to a well-known resource (e.g., /SCA) or a service request (e.g., NFp_SCA_Get_Request) without any requesting parameter. This ensures that no client information is exposed to the server in such request.

2.	The SCP forwards the request (NFp_SCA_Get_Request, or HTTP Get) from the Client to the Server.

3.  The Server (e.g. an NRF or an NF Service Producer) receives the request and generates Server Credential Assertion (SCA). The SCA has the same format as CCA and includes the identity of the server (e.g., NF Service Producer  instance ID) and associated proof, which allows the verification of the server identity.

4.	The Server sends the SCA in the response (e.g., NFp_SCA_Get_Response or HTTP response) back to the SCP.  The response may include an indicator to request for the CCA from the Client.

5.  The SCP forwards the response including the SCA back to the Client.

6.  The Client verifies the SCA cryptographically and then verifies the server identity (e.g., NF Service Producer  Instance ID) in the SCA against the corresponding identity of server where a service request is to be sent.

### 6.13.3	Evaluation

TBD

## 6.14	Solution #14: SCP trust domain or technical domain grouping

### 6.14.1	Introduction

This solution is addressing key issue #2. It provides an optimization for SCP domain operation in trusted environments.

### 6.14.2	Solution details

According to 23.502 [7], SCP domains can be registered at the NRF. A NF Service Producer can register within its profile its SCP domain.

A SCP domain is called trusted if it is a group or subset of available SCPs within one PLMN that includes several SCPs operated in the same trust environment according to a security policy. Such technical SCP trust domains can be defined in large PLMNs, for example, based on computer centre boundaries, operators of subnetworks, regions/provinces, etc.

SCP trust domains could also be formed equipment-specific or release-specific. Large operators could have involved different organizations to operate the complete network and hence, structure their regional availability by SCP domains, where additional security means between these organizations could be needed for achieving trust.

For a SCP trust domain, one or several SCP domains can be grouped, i.e., a SCP trust domain is a superset of a distinct set of SCP domains. A SCP trust domain could be isolated from other SCP domains. It could be registered or configured together with a set of policies at NRF.

If the SCP is part of several SCP domains, all SCP domains have to be part of the same SCP trust domain as illustrated in the figure below.

Figure 6.14-1: Illustration of SCP trust domain

SCPs can discover boundaries of trust or technical domains based on

- the SCP domain, from which a request is received from and is forwarded to, and

- the configured knowledge of the relationship of trust or technical domains and SCP domains.

Policies for a SCP trust domain could include information such as which domain can communicate with which domain  or whether NFs from one SCP trust domain can access NFs of another SCP trust domain.

The policies are related to restriction in the communication between trust domains that the SCP enforces, as outlined in the examples below.

Some examples of policies for a SCP trust domain are as follows:

- Only SCPs from a particular (usually the same) SCP trust domain would be allowed to discover the NF Service Producer.

- An access token could be provided for all NFs of the same SCP trust domain.

- A service is provided only for NFs of a distinct NF Type within a specific SCP trust domain.

- The SCP domain of NF Service Consumer and SCP are the same as of the NF Service Producer.

- Access to certain services is not permitted over trust domain boundaries.

If the SCP discovers that a request message traverses a boundary between trust/technical domain, it can perform also policing on traffic between SCP trust domains, for example:

- Checking target URIs

- Checking delegated discovery parameters

- Checking routing binding indications

- Checking whether a request contains a CCA

The following are examples of using SCP trust domains:

- The access from NF or SCP to the SCPs of a SCP trust domain can be limited to this domain only. Hence, all SCPs forming a trust domain could be allowed to communicate with NRF and with NF Service Producers on behalf of a NF Service Consumer from the same trust domain. By this, SCPs forwarding a request to another SCP, e.g., for load balancing, have a mean to check whether the forwarding SCP is a member of the same SCP trust domain.

- If a policy for SCP trust domain is registered and the NF profile includes SCP trust domain information in the NRF, the NRF can authorize or reject a discovery request from NF or SCP based on whether the expected NF Service Producer or another SCP instance is part of the same SCP trust domain as the NF Service Consumer.

- If a NF Service Consumer has included in its request a target SCP trust domain, the NRF can provide an access token that provides authorization for all NFs within this domain.

- If the NF Service Producer receives a service request, it could provide the service only, when contacted by an SCP of the trust domain indicated in the access token (e.g., AMF getting a token from NRF granting access to any SMF in the same SCP domain).

- If domain information of the NF Service Consumer is included in the token, it enables the NF Service Producer or SCP to verify whether the requesting NF Service Consumer is part of the same SCP trust domain.

SCPs of a trust domain connecting to another SCP domain outside the trust domain shall provide confidentiality, integrity and replay protection.

### 6.14.3	Evaluation

The solution addresses SCP domains in particular. Trust domains do not need to be necessarily identical to SCP domains. Restricting network topology so that requests between certain elements are not allowed could be generalized as provided by the potential security requirement in clause 5.2.3.

Further evaluation is needed for deciding on the usefulness of the concept in general, such as addressing the moving of UEs between regions with different trust requirements.

## 6.15		Solution #15: Authorization mechanism for the involved NFs in the delegated “Subscribe-Notify” scenario.

### 6.15.1 	Introduction

This solution addresses KI#3 on how to assure that the notification messages could be only forwarded to an authorized NF identified by its NF instance ID in the delegated "Subscribe-Notify" scenarios. This solution does not address authorization of the notification URI.

The solution is based on the authorization of NF Service Consumers for data access via DCCF specified in Annex X.2 of  TS 33.501 [2]. It proposes to include two instance IDs in the access token request, allowing the NRF to check whether one NF (e.g. NF_C) is authorized to subscription and whether another NF (e.g. NF_A) as the proxy is allowed to request the service from the identified NF Service Producer on behalf the NF_C. The NF Service Producer verifies the access token is valid according to these instance IDs. After successful verification, the NF Service Producer should provide the notification service.

Editor's Note: It is ffs how the solution works in case of notification target reselection as described in clause 6.3.1.0 of TS 23.501.

### 6.15.2 	Solution details

In the "Subscribe-Notify" NF service illustration 2 (delegated scenario) specified in TS 23.501 [3], clause 7.1.2, an NF Service Consumer (e.g. NF_C) may subscribe the service of an NF Service Producer (e.g. NF_B) on behalf of another NF Service Consumer (e.g. NF_A).

The Figure 6.15.2-1 describes the detailed solution for authorization mechanism for the involved NFs in the delegated "Subscribe-Notify" scenarios.

Figure 6.15.2-1: Authorization mechanism for the involved NFs in the delegated "Subscribe-Notify" scenario

- The NF_A decides to subscribe the service of NF Service Producer (e.g. NF_B) on behalf of NF_C and gets an access token from the NRF for a Service Request toward the NF_A. The NF_C initiates an NF service request to the NF_A which includes the notification URI, the access_token_NF_C and the CCA of NF_C to be used for subscription.

- The NF_A should verify if the access token and the CCA of the NF_C is valid and executes the service.

- The NF_A sends a Nnrf_AccessToken_Get request to NRF including the information to identify the target NF (NF Service Producer), the source NF (NF Service Consumer e.g. NF_C), the NF Instance ID of NF_A and the CCA_NF_C received at step 2.

- The NRF should check whether the NF_C are allowed to access the service provided by the identified NF Service Producer, and whether the NF_A as the proxy is allowed to request the service from the identified NF Service Producer on behalf the NF_C. The NRF authenticates NF_C based on the CCA of NF_C.

- If the authorization is successful, the NRF shall then generate an access token with the identity of the NF_A and the identity of the NF_C, NF type of the NF Service Producer (audience), subscribe service name(s), (scope).

The NRF sends access token to the NF_A in the Nnrf_AccessToken_Get response operation.

- The NF_A requests service to the NF Service Producer. The Service Request also consists of the CCA_NF_C, so that the NF Service Producer authenticates the NF _C.

- The NF Service Producer authenticates the NF_C and verifies the access token to ensure that the access token is valid. After authentication and authorization is successful, the NF Service Producer assures that the NF_A as the proxy is allowed to receive the response message on behalf the NF_C, and executes the subscribe service.

- The NF Service Producer should respond the NF_A and provide the NF_C with the notification service based on the Notification URI.

### 6.15.3 	Evaluation

TBD

## 6.16	Solution #16: Selective End of End Protection of HTTP Request and Response in Indirect Communication

### 6.16.1	Introduction

This solution addresses KI#5.

It allows an NF to verify the integrity of selected attributes in a service request or response consumer. It requires the use of CCA and Server Credential Assertion (SCA) so that a recipient NF has access to the public key certificate of a sending NF. The solution does not propose any change to CCA or SCA.

Instead of protecting the integrity of entire request or response, which has impact on the functioning of SCP based on the feedback from CT, this solution proposes to protect only selected fields in a request or response that are considered critical and should not be modified by any intermediate entities such as SCP. Note there is a trade-off. The more of the message is protected, the fewer attacks are possible, but there is also a higher risk that the SCP may need to modify integrity protected parts of the message.

Further, to avoid complexity of signalling which attributes are to be integrity protected inline request or response, this solution proposes to use static configuration to indicate which attributes require integrity protection.

### 6.16.2	Solution details

Each NF, either NF Service Consumer or NF Service Producer, is provided with a static integrity protection policy, describing which attributes or information elements within a service request or service response require integrity protection. For example, AUSF and UDM may be configured with the security policy requiring the integrity protection of authentication method, and authentication vector.

Editor's Note: how to handle the dynamic security policy cases by the NF Service Consumer and NF Service Producer is FFS.

Based on the security policy, an NF creates one (or more) JWT tokens for the information elements in a service request or service response that require integrity protection. The JWT token is digitally signed by the private key of the NF. The JWT token may be included in an information element in a service request or service response, or in an HTTP header.

The public key certificate of the sending NF that is required to verify a received JWT token is communicated to a receipt NF in CCA or SCA.

When a recipient NF receives a service request or service response, it checks its security policy to see if any information elements in the received service request or service response require integrity protection. If yes, it extracts and verifies the JWT token to ensure that protected information elements have not been tampered with.

Editor's Note: how to mitigate replay of a JWT token is FFS.

### 6.16.3	Evaluation

The solution extends the existing token-based authorization method by including the instance ID of the NF_A and the instance ID of the NF_C in the token claims.

The token is used to indicate towards NF_B (the producer and sender of the notification) that NF_C (the notification receiver) authorizes NF_A (the subscriber) to subscribe services on behalf of NF_C (the notification receiver). The token is also used to indicate towards NF_B (the producer and sender of the notification) that the NRF authorizes NF_C (the notification receiver) to receive notifications from NF_B.

The solution has impact on the access token generation by adding the instance ID of delegated NF_C into the access token, and token verification.

Editor’s Note: Further evaluation is FFS.

## 6.17	Solution #17: Authorization mechanism negotiation using existing methods

### 6.17.1	Introduction

This solution addresses Key Issue #7 "Authorization mechanism negotiation". It is proposed to use the two NRFs for the authorization mechanism negotiation.

Editor's Note: it is ffs how to solve the case that one operator uses token-based authorization and its roaming partner uses static authorization.

### 6.17.2	Solution details

The solution assumes that discovery is used by both PLMNs according to the standard. Thus, both PLMNs are able to understand the OAuth2Required indication as specified in 29.510 when NRFs communicate with each other, and when a NF service is discovered at NRF.

The key issue use case assumption is that one PLMN uses only static authorization. Based on TS 33.501 Rel-15 (clause 13.4.1.0): "The authorization framework uses the OAuth 2.0 framework as specified in RFC 6749 [43]. […] The authorization framework described in clause 13.4.1 is mandatory to support for NRF and NF." and clause 13.4.0, static authorization can be used. "It can be used when token-based authorization is not used."

This solution follows the standard, i.e., that at least the support for OAuth2.0 is provided, thus, even if not used, NRF and NFs of a PLMN using only static authorization need to at least understand the attributes (IEs) provided during discovery.

This solution suggests that from the oAuth2Required indication, the vNRF can imply, whether OAuth2.0 or static authorization is to be used within one PLMN. This covers the use case, where within one PLMN maybe not yet all NFs use OAuth2.0.

For inter-PLMN stage 3 (TS 29.510 Table 6.1.6.2.3-1) "oauth2Required" can be used to handle the authorization method setting by the hNRF. Another type for NFService, the "perPlmnOauth2ReqList", is also specified and includes the Oauth2-based authorization requirement supported by the NF Service Instance per PLMN of the NF Service Consumer.

| oauth2Required | boolean | O | 0..1 | It indicates whether the NF Service Instance requires Oauth2-based authorization. Absence of this IE means that the NF Service Producer has not provided any indication about its usage of Oauth2 for authorization. |
| --- | --- | --- | --- | --- |
| perPlmnOauth2ReqList | PlmnOauth2 | O | 0..1 | When present, this IE shall include the Oauth2-based authorization requirement supported by the NF Service Instance per PLMN of the NF Service Consumer. This IE may be included when the Oauth2.0 authorization requirement supported by the NF Service Instance for different PLMN is different. When the requester PLMN Id is available in perPlmnOauth2ReqList IE, this IE shall override the oauth2Required IE. If the requester PLMN ID is not present in perPlmnOauth2ReqList IE, then the value of oauth2Required IE shall be applicable if available. |

Table 6.17.2-1: The IEs oauth2Required and perPlmnOauth2ReqList, part of TS 29.510 Table 6.1.6.2.3-1

In addition, stage 3 has specified for inter-PLMN usage the type PlmnOauth2 (see clause 6.1.6.2.102 3GPP TS 29.510 [6]. If the optional attribute "oaut2NotRequiredPlmnIdList" is used, the solution suggests that in this case pre-configured information by HPLMN (how to apply static authorization with a specific roaming partner) can be used.

Stage-3 details could be improved by stating explicitly that oauth2NotRequired refers to static authorization.

| oauth2RequiredPlmnIdList | array(PlmnId) | O | 1..N | It shall indicate the consumer PLMN ID list for which NF Service Instance requires Oauth2-based authorization. (See NOTE 1) |
| --- | --- | --- | --- | --- |
| oauth2NotRequiredPlmnIdList | array(PlmnId) | O | 1..N | It shall indicate the consumer PLMN ID list for which NF Service Instance does not require Oauth2-based authorization. (See NOTE 1) |
| NOTE 1: The same PLMN Id shall not be present in both oauth2RequiredPlmnIdList and oauth2NotRequiredPlmnIdList. |  |  |  |  |

Table 6.17.2-2: The IEs oauth2RequiredPLMNIdList and oauth2NotRequiredPLMNIdList, part of TS 29.510 Table 6.1.6.2.3

Further, the IE oauth2Required used in the bootstrapping information is used to indicate whether NRF requires OAuth2 based authorization for accessing its services:

| oauth2Required | map(boolean) | O | 1..N | When present, this IE shall indicate whether the NRF requires Oauth2-based authorization for accessing its services. The key of the map shall be the name of an NRF service, e.g. "nnrf-nfm" or "nnrf-disc". The value of each entry of the map shall be encoded as follows: - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that the NRF has not provided any indication about its usage of Oauth2 for authorization. |
| --- | --- | --- | --- | --- |

Table X: oauth2Required for service access from NRF, part of 29.510, see Table 6.4.6.2.2-1: Definition of type BootstrappingInfo

If set to false, OAuth2 based authorization is not required. This solution proposes to clarify stage 3 that this mean, the only other available authorization mechanism “static authorization” is the default in this case.

GSMA has provided the following recommendations given in NG.113 [8], clause 7.6.3.4:

"It is recommended that both VPMN and HPMN use either static authorization or
authorization using OAuth2 access token.
Note: Authorization is not possible in case the HPMN only uses authorization
using OAuth2 access token and the VPMN only uses static authorization.

If using authorization using OAuth2 access token it is recommended that both VPMN and HPMN support oauth2Required IE as specified in 3GPP Release 16 TS 29.510 [16].

If the HPMN wants to use authorization using Oauth2 only for some VPMNs then HPMN must support perPlmnOauth2ReqList IE as specified in 3GPP Release 17 TS 29.510 [16]."

How to configure the NF profile in case the operator wants to change to another authorization method is out of scope of this solution. By OAM this could be automated for each NF affected.

If the operator of one PLMN does not want to follow the specification and only supports static authorization, and the other operator requires the usage of OAuth, then in a consequence, those operators cannot have a roaming agreement. The service request has to be rejected if one PLMN does not accept a request without an OAuth.

### 6.17.3	Evaluation

Several means to allow a PLMN's hNRF to provide to the requesting vNRF information on the authorization method used exist in the current specification TS 29.510 [6]. If in the array(PlmanId) of hPLMN NRF a roaming partner is on the "oauth2NotRequiredPlmnIdList", static authorization can be used with this roaming partner. When operators follow the recommendations given in NG.113 [8], the key issue seems to be covered sufficiently by the existing methods.

Editor's Note: Further evaluation is FFS.

## 6.18	Solution #18: Avoiding slice isolation violation

### 6.18.1	Introduction

This solution is addressing KI#9.

### 6.18.2 	Solution details

To avoid network slice isolation violation as described in detail by KI#9, clarification of the specification clause 13.4.1.1.2 is required to mitigate potential slice related attacks, as described in S3-213209 (reply-LS to GSMA on 5G CN slicing attack).

The solution proposes

-  in step 1a of clause 13.4.1.1.2 of TS 33.501 [2], NRF to verify that the S-NSSAI of the NF Service Consumer in the access token request is consistent. Specification text is proposed as follows:

"The NRF may additionally verify the S-NSSAIs of the NF Service Consumer." The NRF checks whether the NF Service Consumer is authorized to access the requested service(s). "For example, the NRF may verify that the NF Service Consumer can serve a slice which is included in the allowed slices for the NF Service Producer."

- in step 2 of clause 13.4.1.1.2 of TS 33.501 [2], NF Service Producer checking the audience claim to verify that the producer NSSAIs in the access token (received in the service access request) contains the allowed NSSAI of a UE if the request is for UE related information. Specification text is proposed as follows:

It checks that the audience claim in the access token matches its own identity or the type of NF Service Producer. If a list of NSSAIs or list of NSI IDs is present, the NF Service Producer shall check that it serves the corresponding slice(s). "If applicable (e.g., when the request is for information related to a specific UE), the NF Service Producer may check that the NF Service Consumer is allowed to access (as indicated by the NF Service Producer’s NSSAIs in the access token presented by the NF Service Consumer) at least one of the slice(s) that the UE is currently registered to, e.g., by verifying that the UE’s allowed NSSAI(s) intersect with the NF Service Producer's NSSAIs in the access token. As the NSSAI supplied by NF Service Consumer is forged, NF Service Producer (e.g. AMF) should obtain the NSSAI of the UE from the UDM for verification."

### 6.18.3	Evaluation

## The text proposed provides input to normative work on this topic. It avoids slice isolation violation.

## 6.19	Solution #19: Hosted SEPP requirements

### 6.19.1	Introduction

This solution addresses key issue #12. It provides input for text that needs to be adapted for clarification of a deployment option for SEPP of a PLMN, if hosted by an entity external to the PLMN.

### 6.19.2	Solution details

This solution is addressing the Hosted SEPP as described in KI#12.

If the Hosted SEPP provider is contracted by several PLMNs, it hosts several distinct Hosted SEPP instances for the different PLMNs.

A PLMN can have a Local SEPP (managed by the PLMN) and a Hosted SEPP (instance), both handling different sets of roaming relations.

It is proposed to add the following definitions to 33.501:

“Hosted SEPP: A SEPP that is hosted and operated by a provider outside the PLMN, for example an IPX provider, in which case the trust environment of a distinct PLMN extends to the Hosted SEPP instance representing this PLMN. A Hosted SEPP provider can operate Hosted SEPPs for multiple PLMNs.

Local SEPP: A SEPP of a PLMN that is operated by an MNO and taking care of a distinct set of roaming relations different from the set of roaming relations taken care by a potential Hosted SEPP.”

It is further proposed to have an addition to the general requirements in 33.501 clause 5.9.3 (Requirements for e2e core network interconnection security) as follows:

“From the roaming partner’s point of view, a Hosted SEPP shall behave in the same way as the SEPP.

All SBI messages transmitted between the PLMN and the Hosted SEPP deployment shall be confidentiality, integrity, and replay protected.

A Hosted SEPP provider shall isolate the processing for each PLMN from the processing for other PLMNs.

A Hosted SEPP provider shall use a different TLS certificate per PLMN, as the MNC and MCC of that PLMN are encoded in the certificate (as specified in TS 23.003, Table 6.1.3c.3-1). This certificate shall be issued to the Hosted SEPP provider under its own unique name.

A Hosted SEPP shall only be able to obtain a certificate with an MCC/MNC combination if the affected MNO has previously authorized this.”

Finally, it is proposed to have an additional NOTE referring to Hosted SEPP concept in 33.501 clause 13.1.2 (Protection between SEPPs) as follows:

“If a Hosted SEPP is deployed as the edge of one PLMN and in the same security zone, the same protection requirements apply to this Hosted SEPP as they apply to a SEPP in general. Furthermore, the Hosted-SEPP-specific protection requirements in clause 5.9.3 apply.”

### 6.19.3	Evaluation

Whether to use a Hosted SEPP is a business decision. Nevertheless, it is important to provide a minimal set of requirements covering this deployment option.

Guidelines for naming are provided for Hosted SEPP by GSMA already and need to be checked for a potential certificate update in 3GPP TS 33.310.

Editor’s Note: Further evaluation is FFS.

## 6.20		Solution #20: PRINS for Roaming Hubs

### 6.20.1		Introduction

This solution addresses key issue #10.

There may be one or two roaming hubs involved in the communication between two PMNs. The roaming hubs require the ability to allow roaming relations between these PMNs via these roaming hubs. Furthermore, roaming hubs with financial liability require the ability to shape roaming traffic between the PMNs in order to control how much roaming traffic they will have to pay for.

PRINS can be used to achieve this, allowing and disallowing establishment of N32c through standard HTTP proxy functionality. PRINS can also be used to dynamically introduce a traffic shaper under control of the RH into the data plane.

### 6.20.2		Solution details

Figure 6.20.2-1: Establishment of N32c via roaming hubs

0:	The cSEPP is configured to make use of RH1

1:	After receiving an initial request, the cSEPP may set up a TLS connection with RH1's proxy. The cSEPP then sends a CONNECT command to RH1's proxy indicating the destination to be pSEPP.

Editor's Note: whether additional methods to determine pSEPP name other than its well known FQDN are required is FFS

2:	RH1's proxy shall verify that cSEPP is allowed to set up a roaming relation with pSEPP. If not, the flow continues with the error message of step 6. Otherwise, it continues with step 3.

3:	If pSEPP requires another roaming hub to be reached, RH1's proxy sets up a TLS connection with RH2's proxy. R1's proxy then sends a CONNECT command to RH2's proxy indicating the destination to be pSEPP. RH2's proxy shall verify that RH1 is allowed to set up a roaming relation with pSEPP. If not, the flow continues with the error message of step 5. Otherwise, it continues with step 4.

4:	If RH2 is present, RH2's proxy, otherwise RH1's proxy shall set up a TCP connection to pSEPP. This connection may be protected by TLS.

5:	If successful, if RH2 is present, RH2 shall return success with code 200.

6:	If successful, RH1 shall return success with code 200.

If successful, pSEPP and cSEPP shall establish a TLS connection via the proxy/proxies for N32c and establish an N32f context, selecting PRINS as the protection mechanism.

Figure 6.20.2-2: N32f via roaming hubs

1:	cSEPP may either intiate a new TLS connection to RH1's proxy, or reuse the TLS connection from setting up N32c. In any case, communication between cSEPP and RH1's proxy shall be TLS protected.

2: 	After receiving a request from an NF, the cSEPP shall encapsulate the request as defined in TS29.573 [9] subclause 5.3.2.3 and send the reformatted message to RH1's proxy. The URL to send the request to shall be located on RH1's proxy, thus RH1's proxy is able to read the request.

3:	If another RH is present, RH1's proxy shall forward the message to RH2's proxy, including a via header and an asserted identity header asserting the identity of the cSEPPs PMN. If RH2 is not present, then this request shall be sent directly to pSEPP.

4:	If another RH is present, RH2's proxy shall forward the request to pSEPP, including a via header giving both RH proxies. An asserted identity header is not necessary as pSEPP can identify the source PMN from the N32f context.

5:	pSEPP shall decapsulate the request, forward it to the pNF and receive the response.

6:	pSEPP shall reformat the response and forward to RH2 (if present) or to RH1 (if RH1 is not present).

7:	If RH2 is present, then RH2's proxy shall forward  the response to RH1's proxy.

8: 	RH1's proxy shall forward the response to cSEPP.

9:	cSEPP shall decapsulate the response, forward it to the pNF

All communication shall be TLS protected on the hops between cSEPP and RH1's proxy, RH1's proxy and RH2's proxy, RH2's proxy and pSEPP.

RH1's proxy and RH2's proxy both may include modifications, or return errors in case of roaming policy violations.

The following two messages sequence charts give examples:

Figure 6.20.2-3: Roaming hub inserting a traffic shaper

To insert a traffic shaper into the roaming user data plane, the RH may include a modifications block that rewrites the N9 endpoints IP address and TEID for the visited networks UPF in message 3, and another modifications block that rewrites the N9 endpoints IP address and TEID for the visited networks UPF in message 7. As these modifications blocks are signed, the modifications are attributable.

Figure 6.20.2-4: Roaming rejecting a session creation request

A roaming hub may also reject a PDU session setup request by returning an error in message 3. This error is forwarded to the vSMF in message 4. This may be triggered by some information in the request visible to the RH. If different SUPIs require different handling, then of course, SUPI shall not be encrypted in the request.

The examples can be extended to include two roaming hubs and potentially even two traffic shapers.

### 6.20.3		Evaluation

The solution allows a RH to allow or deny roaming traffic between operators based on the contractual relations with the RH. In addition, it allows traffic shaping of the user plane roaming traffic between two PMNs making use of roaming hubs by dynamically introducing a traffic shaper.

The solution exposes user plane traffic to roaming hub's traffic shapers (when present). However, at both PMNs, the presence of the traffic shapers is visible in the modifications blocks. Furthermore, modifications performed by the roaming hubs are attributable.

Editor's Note: further evaluation is FFS.

## 6.21	Solution #21: Certificate solution for NRF validation of NFc for access token requests

### 6.21.1	Introduction

This potential solution addresses the KI #11, NRF validation of NFc for access token requests. It describes what entities that need to be validated, what needs to be in place from a certificate perspective, and finally some additional details how the verification is performed.

The basic idea of this solution is that the NRF uses the information in the NF Consumer's certificate to base its authorization decision on.

### 6.21.2	Solution details

#### 6.21.2.1	NF consumer information to validate at Service Request Authorization

The authorization of a NF consumer needs to be performed by validating verified information about the NF consumer stored in NRF. This solution proposes that the NRF validates NF Type, NF Instance ID, PLMN-ID, and FQDN.

#### 6.21.2.2	Certificates

This solution proposes that the NRF uses the NF consumer information in the NF consumer's certificate to authorize the NF consumer. The public key certificate can be the TLS certificate of the NF Consumer, if the NF Consumer itself requests the access token, or the certificate used to sign the CCA of the NF Consumer, if the SCP requests the access token on behalf of the NF Consumer.

If the TLS certificate of the NF Consumer is used, this solution requires that the NF consumer information described in clause 6.21.2.1 is available in the TLS certificate.

Today the following entities, relevant for authorization purposes, are mandatory in the TLS certificate: PLMN-ID and FQDN.

The following are currently only optional, but need to be present in the certificate, otherwise this solution will not work:

-	NF instance ID

-	NF type

Having this information present in the certificate can be reached in different ways. It could either be mandated in the specification or required to be present in deployments in order for this procedure to work. If it is not mandated that the information is present in the certificates, this may lead to interoperability problems between different vendor implementations.

#### 6.21.2.3	NRF validation solution

The NRF checks whether the NF Service Consumer is authorized to access the requested service(s) by performing the following validation:

- The NRF retrieves the available NF consumer information in the public key certificate of the NF Service Consumer to decide whether the NF consumer is authorized to invoke the NF Producer's service.

In deployment scenarios with only one NRF, the NRF does not use the information about the NF Consumer in the access token request to base its authorization decision on, since this information is provided by the NF Consumer itself and therefore not reliable.

In hierarchical NRF deployments, additional measures are necessary. The NRF that receives the access token request needs to verify the information about the NF Consumer in the access token request with the certificate. The NRF that issues the token can then base its authorization decision on the information about the NF Consumer in the access token request, since the NRF that received the access token request has verified the information.

### 6.21.3	Evaluation

TBD

Editor's Note: This solution assumes that it is not clearly specified for all releases including Rel-17 which mechanism the NRF uses to validate whether the NF service consumer is authorized to receive the requested service. It is ffs whether the mechanism in this solution is needed, or whether the NRF can use the NF Service Consumer profile to validate the NF Service Consumer in the process of authorization of requested services including access token request.

## 6.22	Solution #22: Combined certificate and profile solution for NRF validation of NFc for access token requests

### 6.22.1	Introduction

This potential solution addresses the KI #11, NRF validation of NFc for access token requests. It describes what entities that need to be validated, what needs to be in place from a provisioning perspective, and finally some additional details how the validation is performed.

The basic idea of the solution is that the NRF uses the information in the NF Consumer's certificate and additional information in the NF profile as basis for its authorization decision.

### 6.22.2	Solution details

#### 6.22.2.1	NF consumer information to validate at Service Request Authorization

The authorization of a NF consumer needs to be performed by validating verified information about the NF consumer stored in NRF. This solution proposes that the NRF validates NF Type, NF Instance ID, PLMN-ID, and FQDN.

#### 6.22.2.2	O&M Provisioning solution

Any Network Function is allowed to use the NF management API to register its profile at the NRF. However, it can happen that NF consumers do not use the NF management API to register their profile at the NRF. In this case, this solution proposes that the NRF is provisioned with the NF profiles for these NF consumers, containing the NF consumer information described in clause 6.22.2.1, by an O&M provisioning operation.

The O&M provisioning operation need not to be standardized

#### 6.22.2.3	Certificates

In this solution, the public key certificate can be the TLS certificate of the NF Consumer, if the NF Consumer itself requests the access token, or the certificate used to sign the CCA of the NF Consumer, if the SCP requests the access token on behalf of the NF Consumer.

If the TLS certificate of the NF Consumer is used, this solution requires that the NF instance ID is available in the TLS certificate, so that the certificate can be linked to the NF profile.

Having the NF instance ID present in the certificate can be reached in different ways. It could either be mandated in the specification or required to be present in deployments in order for this procedure to work. If it is not mandated that the information is present in the certificates, this may lead to interoperability problems between different vendor implementations.

#### 6.22.2.4	NRF validation solution

The NRF checks whether the NF Service Consumer is authorized to access the requested service(s) by performing the following validation(s):

1.	The NRF retrieves the available NF consumer information in the public key certificate of the NF Service Consumer.

2.	The NRF uses the NF instance ID of the NF Consumer that was obtained during authentication of the NF Consumer to retrieve the NF profile.

3.	The NRF uses the information about the NF Consumer obtained in steps 1 and 2 to decide whether the NF Consumer is authorized to invoke the NF Producer's service.

In deployment scenarios with only one NRF, the NRF does not use the information about the NF Consumer in the access token request to base its authorization decision on, since this information is provided by the NF Consumer itself and therefore not reliable.

In hierarchical NRF deployments, additional measures are necessary. The NRF that receives the access token request needs to verify the information about the NF Consumer in the access token request with the certificate and/or the NF profile. The NRF that issues the token can then base its authorization decision on the information about the NF Consumer in the access token request, since the NRF that received the access token request has validated the information.

### 6.22.3	Evaluation

TBD

Editor's Note: This solution assumes that it is not clearly specified for all releases including Rel-17 which mechanism the NRF uses to validate whether the NF service consumer is authorized to receive the requested service. It is ffs whether the mechanism in this solution is needed, or whether the NRF can use the NF Service Consumer profile to validate the NF Service Consumer in the process of authorization of requested services including access token request.

# 7	Conclusions

## 7.1	KI#1: Authentication of NRF and NF Service Producer in indirect communication

### 7.1.1	Analysis

The key issue addresses the scenario of an intermediary such as a standalone SCP to be compromised. In this case, the NF Service Consumer is not able to verify if the NRF response or the NF Service Producer response has been received by a legitimate entity. However, it also needs to be considered that reselection of the NF Producer by the SCP can be a desired feature.

3 solutions are presented to cover this key issue.

Solution #1 and extended solution #6 are based on using the concept of CCA for the NF Service Producer or the SCP, i.e., similar to the CCA used for the NF Service Consumer as specified in 3GPP TS 33.501 [2]. Such a token is introduced to allow a client to validate the sender of a response directly, even if an SCP is in between.

Solution #1 has a limited scope as provided in the respective evaluation part in clause 6.1.

Solution #6 superseeds solution #1, overcoming some of the limits of solution#1 in case of Model C, also addressing the scenario of reselection of the target NF.

The optional inclusion of such a token including a NF Set Id allows a NF Service Consumer to validate if the NF sending the response is the producer that NFc has selected by itself or if it is a producer of the same NF Set or NF Service Set as indicated in the OAuth token received from NRF. The token cannot be used, if SCP has applied reselection of NFp outside of an NF Set.

Solution #6 addresses Model C with direct TLS between NF consumer and NRF for discovery. It does not address Model D or Model C without direct TLS between NF consumer and NRF for discovery. It requires that the NF Service Consumer has knowledge about which NF Service Producers are in the NF Set of the producer.

Editor's Note: Solution #13 analysis is FFS.

Editor's Note: FFS how to address the following questions: what should the NFc do if the response comes from another entity than the intended producer.  Should the NFc assume that the SCP has reselected the producer and accept the response? Or should the NFc reject the response?

Editor's Note: It needs to be clarified what are the cases of compromised SCP and whether they are addressed by the proposed solutions.

### 7.1.2	Conclusion

TBD

## 7.2	KI#2: Need for additional security at operational level among SCP domains

### 7.2.1	Analysis

The key issue provides for operational aspects of SCP domains. Usually within an SCP domain, trust is assumed. If several SCP domains form a wider SCP trust domain, additional means for securing such trust domain against other SCP domains could be considered. However, further elaboration would be needed, whether such operational concept is useful.

### 7.2.2	Conclusion

The topic is not followed up normatively.

## 7.3	KI#3: Service access authorization in the "Subscribe-Notify" scenarios

### 7.3.1	Analysis

TBD

### 7.3.2	Conclusion

TBD

## 7.4	KI#4: Authorization of SCP to act on behalf of an NF or another SCP

### 7.4.1	Analysis

One of the main 5G security features is the usage of OAuth 2.0 to authorize service requests. In indirect communication scenarios, however, an SCP is not explicitly authorized, it could request a service or a token without the knowledge of the NF Service Consumer (NFc).

2 solutions (Sol#2 and Sol#3) have been proposed, mandating the usage of CCA, by which the NFc can at least "implicitly authorize" the SCP. This achieves a minimum level of trust, but cannot avoid the threat described in clause 5.4.2 within the validity time of the CCA, i.e., that "SCP can send a service request and receive a valid service response on behalf of NF Service Consumer", even though the NF Service Consumer has not authorized the SCP”. Thus, CCA validity time need to be chosen very carefully to minimize this possibility.

Solution #3 argues that existing mechanisms are sufficient: a NF sending a request (with its CCA) to SCP, implicitly authorises SCP against NRF to process this request. The limit of this solution is that the CCA can be stolen or misused and, without NFc having requested a service, NRF could provide an access token to SCP or NFp could provide a service to SCP.

Solution #2 proposes to include the SCP identifier (i.e., either SCP Instance and/or Domain info) in the CCA generated by the NF Service Consumer. Thus, only the SCP, which SCP Identifier is included in the CCA from NFc is able to get a response from NRF to its request. This ensures that the CCA cannot be misused by a MitM and thus achieves an additional level of trust, when the CCA is more than once useable. This however comes with a trade-off, since it impacts existing implementation (CCA generation by NFc and NRF handling of CCA). Further, if more than one SCP is in the path, with this solution SCP needs to create its own CCA including the SCP identifier from the requesting SCP in order to keep the chain of trust.

A conclusion to this key issue therefore needs to weight the level of security and trust achieved against the impact of suggested enhancements to the current release.

Another aspect, which requires further analysis is whether explicit authorization of the SCP would be needed. NRF can only authorize NFs. NRF needs to trust that any request forwarded by SCP was initiated by a NFc. Thus, in general only implicit authorization of SCP by the NF Service Consumer is possible with the current solutions proposals based on CCA.

In summary, further study is needed. Whether to go for Sol#2 or Sol#3 is a trade of between security enhancement versus complexity added.

### 7.4.2	Conclusion

TBD

## 7.5	KI #5: End-to-end integrity protection of HTTP messages

### 7.5.1	Analysis

The KI #5 is about end-to-end protection of HTTP message for indirect communication. The key issue seek for solutions on how an NF Service Producer can verify that a service request of the NF Service Consumer received via SCP has not been modified tampered with.

Parameters in the request messages may be modified by the intermediate node according to the reply LS S3-212418 from CT WG4, and end-to-end protection of HTTP headers and bodies is discouraged, i.e., CT4 discourages end-to-end protection of HTTP headers and bodies and segregating the parameters based on SCP (or SEPP) modification criteria. There can be multiple SCPs and/or SEPPs involved in the service request/response path where SCP can modify the HTTP headers, and SEPP (or IPX) can modify the HTTP headers and/or HTTP body attributes. In addition, CT4 also raises potential compatibility issues with end-to-end protection of HTTP headers and bodies.

### 7.5.2	Conclusion

Since 3GPP CT4 WG indicated that one of the fundamental functionalities of SCP is message mediation which may require SCP to modify certain headers and/or content of the HTTP message, typical end-to-end message integrity protection between the far end NFs seems not feasible.

For this reason, 3GPP SA3 so far relies on hop-by-hop security.

Editor's note: Normative work is FFS.

## 7.6	KI#6: Access token usage by all NFs of an NF set

### 7.6.1	Analysis

Only solution #7 is provided for this key issue. When implementing NF Sets a common context storage needs to be organised, from which all NFs of the set can retrieve the same status. There is no difference in this handling for NF Service Producer and NF Service Consumer.

The main hurdle is that a NF Service Consumer is not necessarily a NF Service Producer, in which case no NF profile registration at NRF is needed. However, NF Service Consumers need to register with the OAuth 2.0 server and as described in the solution, the NF Set ID or NF Service Set ID is needed to be known at the NRF for the solution to work.

The NF Set management is out of scope but not different from using services of different NF Service Producers within the same set by the same token.

Editor's Note: Further analysis is FFS.

### 7.6.2	Conclusion

TBD

## 7.7	KI#7: Authorization mechanism determination

### 7.7.1	Analysis

The key issue is for studying that 5GS should provide mechanisms to handle the case that one operator uses token-based authorization and its roaming partner uses static authorization. Solutions (Solution #9, and #17) were proposed in this regard.

TS 33.501 is mandating the support of OAuth2.0 since Rel-15 and GSMA recommmends (NG.113[8], clause 7.6.3.4) that roaming partners support the same authorization method. However, if one operator uses token-based authorization and its roaming partner uses static authorization, current specification text 29.510 is not clear, because only the OAuth2required use case is described.

Solution #9 provides a negotiation method for the case that one operator uses token-based authorization and its roaming partner uses static authorization. The usage of static authorization only by VPLMN seems to involve additional management effort on the HPLMN hNRF side for defining authorization policies per roaming partner. It further involves the risk that a vNRF can dictate the hNRF its own conditions on which authorization method to use.

Solution #17 is using existing stage 3 methods, which allow hNRF to configure per PLMN whether OAuth2.0 method is required. However, also for this approach some management effort is needed. Further, the existing stage 3 methods emphazise on OAuth2.0 only. The solution proposes to provide an explicit statement on static authorization if OAuth2.0 is not required. Solution #17 requires that one network knows the capability of the other network.

### 7.7.2	Conclusion

TBD

## 7.8	KI#8: Service access authorization requirements in intra-PLMN scenarios for PLMN deploying multiple NRFs (in OAuth2.0 AS role)

### 7.8.1	Analysis

As described in clause 6.2.6.1 of TS 23.501 [1], an operator network can deploy multiple NRFs, for example due to network slicing or network segmentation.

A clause on handling access token requests in deployments with several NRFs is missing in 3GPP TS 33.501 [2].

Solution #10 addresses one part, i.e. making NF Service Consumer instance and type available to a slice specific NRF. As provided by evaluation, the solution is technical possible but seems to have high impact in realization.

### For normative work, rather a generic description is desired how to handle multiple NRF scenarios.

### 7.8.2	Conclusion

A new clause to TS 33.501 along the lines of the following text will be added to clarify NRF deployment scenarios.

If an NRF receives an access token request for an NF Service Producer that is not registered at this NRF, the NRF has to determine the target NRF where the NF Service Producer is registered. Only afterwards it can forward the access token request to the target NRF.

There can also be several hops of NRFs between the NRF that receives the access token request from the NF Service Consumer and the target NRF where the NF Service Producer is registered.

One option of hierarchical NRF deployment is the local NRF deployment, where the local NRF checks if the NF Service Consumer is authorized to receive the requested service and, if yes, issues and signs the access token. In the case when the access token request from the NF Service Consumer was forwarded by another NRF, the local NRF of the NF Service Producer needs to trust the NRF which forwarded the access token request.

Editor’s Note: Further conclusion regarding solution #10 is FFS.

## 7.9	KI #9: Authorization for Inter-Slice Access

### 7.9.1	Analysis

Solution #11 and solution #18 address this key issue.

Solution #11 proposes to include "allowedSnssaisPerSnssai" or "allowedOperationsPerSnssai" in the NF/NF-Service profile registered by the NF Service Producers in NRF. The parameter "allowedOperationsPerSnssai" allows finer control of scopes allowed to a consumer from a different slice.

Additionally, use of parameter "Requester-NSSAI" is an enhancement to existing Access-Token Grant, and allows NF-producers to authorize the NF-Consumer's S-NSSAI if the NRF has validated the S-NSSAIs.

Solution #18 provides a minimal update as normative text proposal by demanding the NRF to check on the NF Service Consumer being authorized to access a slice of the NF Service Producer.

### 7.9.2	Conclusion

It is proposed to add the following requirements in normative specification:

The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

The 5GS should provide a mechanism that allows NRF not to provide access tokens to NF consumers which are not authorized to access a slice.

Further, normative text in clause 13.4.1.1.2. of TS 33.501 will be added to update in line with solution #18.

## 7.10	KI #10: N32 security in mediated roaming scenarios

### 7.10.1	Analysis

TBD

### 7.10.2	Conclusion

TBD

## 7.11	KI #11: NRF validation of NFc for access token

### 7.11.1	Analysis

TBD

### 7.11.2	Conclusion

TBD

## 7.12	KI #12: Security in Hosted SEPP scenarios

### 7.12.1	Analysis

TBD

### 7.12.2	Conclusion

TBD

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-01 | SA3#102-e | S3-210420 |  |  |  | Skeleton of TR eSBA SEC | 0.0.0 |
| 2021-01 | SA3#102-e | S3-210679 |  |  |  | S3-210562; Introduction. S3-210422; Scope. S3-210564; Authentication of NRF and NFp in indirect communication. S3-210565; SCP deployment models. S3-210653; KI on Verification of UE in subscription and notification in the delegated “Subscribe-Notify” scenarios. S3-210566; KI on Dynamic authorization between SCPs or NF and SCP. S3-210567; End-to-End Critical HTTP headers and body parts integrity protection. | 0.1.0 |
| 2021-03 | SA3#102bis-e | S3-211344 |  |  |  | S3-211224; Rapporteurs update to 33.875. S3-211217; Service response verification in indirect communication. S3-211218; More details on SCP deployment models. S3-211046; New Solution to KI#4: Using existing procedures for authorization of SCP to act on behalf of an NF Consumer. S3-211220; NF-SCP authorization. S3-211221; KI details added to End-to-end integrity protection of HTTP messages. S3-211205; New Solution to KI#5: End-to-end integrity protection of HTTP body and method. S3-211223; Service request authenticity verification in indirect communication. S3-211225; Mapping of solutions to key issues. Rapporteur additional work done; Updating references, heading numbers and mapping tables in line with TR implementation, updating adding missing ed notes in TBD/empty clauses. | 0.2.0 |
| 2021-05 | SA3#103-e | S3-212297 |  |  |  | S3-212292; New solution for key issue#1: Authentication of NF service producer in indirect communication. S3-212298; Sol1 deployment scenarios. S3-212299; ENs on Sol1 Service response verification in model C. S3-212300; EN resolution on sol 2 - NFc authorizing SCP to act on its behalf. S3-212301; Evaluation on sol 2 - NFc authorizing SCP to act on its behalf. S3-211973; KI on Access token usage by all NFs of an NF Set. S3-212394; Solution on Access token request for NF Set. S3-212303; Trust model. S3-212372; Evaluation of Solution #3 "Using existing procedures for authorization of SCP to act on behalf of an NF Consumer". | 0.3.0 |
| 2021-08 | SA3-104-e | S3-213167 |  |  |  | S3-213053; Sol 1 NFp verification – EN resolutions and evaluation. S3-213141 S3-213142; Update on Solution 6 Evaluation for solution 1. S3-213166; Requirement of subscribe-notification key issue. S3-213054 S3-212888 S3-212763 S3-213043; SCP authorization SCP authorization solution evaluation Correction of implementation of S3-211046 Update to Solution #3 "Using existing procedures for authorization of SCP to act on behalf of an NF Consumer". S3-212764; Update Solution #5: End-to-end integrity protection of HTTP body and method. S3-212930; Evaluation for solution 4. S3-213143; Evaluation for solution 5. S3-212928; New solution on key issue #5. S3-213055; Access token request for NF Set – EN resolution. S3-213056; Access token request for NF Set – RFC clarification. S3-213120; New Key issue on authorization mechanism negotiation. S3-213121; New solution for the authorization mechanism negotiation. S3-213057; KI and solution to NRF deployments. S3-213139; KI on Authorization for Inter-Slice Access. S3-212883; Editorial update on trust clause. S3-213058; EN resolution on trust model for SCP. rapporteur; Mapping table update in clause 6.0. | 0.4.0 |
| 2021-11 | SA3#105-e | S3-214369 |  |  |  | S3-214183; eSBA Editorial updates to 33875-040. S3-214387; Adding the evaluation for solution #9. S3-214388; Adding the conclusion for key issue #5. S3-214365; eSBA KI#5 EN resolution. S3-214366; eSBA KI#4 conclusion. S3-214397; [eSBA] KI on inter slice access. S3-214398; Solution to KI#9: Authorization for Inter-Slice Access. S3-214399; [eSBA] New Solution for Key Issue #3: Authorization of notification endpoint. rapporteur; Mapping table update in clause 6.0, corrections to formatting and typos, adding of subheadlines in conclusion clause. | 0.5.0 |
| 2021-12 | SA#94e | SP-211401 |  |  |  | Presented for information | 1.0.0 |
| 2022-02 | SA3#106e | S3-210512 |  |  |  | S3-220498; Evaluation and Conclusion for Key Issue#9. S3-220511; New KI on N32 security in Roaming Hub scenarios. S3-220409; Resolution EN on NF Set per KI6-Sol7. | 1.1.0 |
| 2022-05 | SA3#107e | S3-221280 |  |  |  | S3-220910; Rapporteur update to TR 33.875. S3-220932; Update to KI on roaming hub. S3-221287; Requirement to KI on roaming hub. S3-220955; New KI, NRF validation of NFc for access token requests. S3-221232; Update of Solution #12. | 1.2.0 |
| 2022-08 | SA3#108e | S3-222295 |  |  |  | S3-222278; Editorial updates to 33875-120 by rapporteur. S3-222279; Scope update. S3-221845; Clause 3 clean up of subsections. S3-222280; Scope general remark. S3-222281; Trust clause editorial update. S3-222282; KI1 EN resolution in KI1 - threat clarification. S3-222327; Resolving EN in solution #6. S3-222326; Authentication of NF Producer in Indirect Communication. S3-222283; KI2 update with solution and conclusion on trust domain. S3-222328; New solution for the authorization of the delegated subscribe. S3-222284; KI4 EN resolution of solution 3. S3-222285; KI4 conclusion on authorization of SCP to act on behalf of another SCP. S3-222286; KI5 Sol4 ENs resolution. S3-221858; KI5 Sol5 ENs resolution. S3-221859; KI5 Sol8 ENs resolution. S3-222339; Selective End of End Protection of HTTP Request and Response in Indirect. S3-222287; KI5 conclusion on e2e integrity prot of HTTP msg. S3-222288; KI6 EN resolution in KI threat. S3-222289; KI6 related Sol7 update and conclusion on access token usage by NFs of an NF Set. S3-222290; KI7 evaluation of solution 9 on authorization method negotiation. S3-222291; KI7 Adding a solution on how to handle static auth in roaming with existing methods. S3-222292; KI8 update of solution 10 and evaluation. S3-222293; KI8 conclusion on deploying multiple NRFs. S3-222311; Resolve EN for solution#11. S3-222294; Conclusion on KI9 authorization for inter-slicing access. S3-222314; Conclusion for KI#9. S3-222436; Update to and split of KI#10 to clarify the scenarios. S3-222437; Solution for KI#10 (hosted SEPP). S3-222438; pCR to TR33.875 new solution on PRINS for roaming hubs. S3-221872; KI10 conclusion on N32 roaming. S3-222302; Certificate solution for NRF validation of NFc for access token requests. S3-222303; Combined certificate and profile solution for NRF validation of NFc for access token requests. S3-221873; KI11 conclusion on NFc registration at NRF. | 1.3.0 |
| 2022-10 | SA3#108-Ad Hoc-e | S3-222981 |  |  |  | S3-222802; Editorial updates to 33.875-1.3.0. S3-222803; Abbreviations. S3-222962; Extend trust in inter-PLM. S3-222967; KI1 analysis on NFp authentication in indirect communication. S3-222954; Resolving ENs in Sol#12. S3-222953; Resolving ENs in Sol#15. S3-222975; KI6 EN resolution Sol7. S3-222976; KI7 Sol17 EN resolution. S3-222977; KI7 conclusion on authorization mechanism determination in inter-PLMN. S3-222813; Solution 18 update. S3-222814; KI9 Sol11 EN resolution. S3-222817; Editorial update to Sol20 RHUB PRINS. | 1.4.0 |