3GPP TSG-SA3 Meeting #108e 	S3-221907

# e-meeting, 22 - 26 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1442 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to multiple registrations in different PLMNs\access types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Scenario: According to clause 6.3.2.1 in TS 33.501: “The UE shall independently maintain and use two different 5G security contexts, one per PLMN's serving network. Each security context shall be established separately via a successful primary authentication procedure with the Home PLMN. The ME shall store the two different 5G security contexts on the USIM if the USIM supports the 5G parameters storage. If the USIM does not support the 5G parameters storage, then the ME shall store the two different 5G security contexts in the ME non-volatile memory. Both of the two different 5G security contexts are current 5G security context.” When the the UE is performing a registration request towards the target AMF on a different access type with a new PLMN compared to the access type and PLMN at the source AMF, then in clause 5.2.2.2 in TS 29.518, its rather clear already how the SM context shall be handled by the old AMF in this scenario and it implicitly says MM context shall not be transferred in this case i.e. the MM/Security context of another PLMN/access shall not be transferred to a different PLMN/access. This SA3 CR can be interpreted as an alignment with stage 3. It is therefore proposed in this CR to TS 33.501 to clarify stage 2 in TS 33.501 in order to align with CT4 specification TS 29.518 as follows: During the Registration procedure in CM-IDLE mode, if the source AMF receives and detects that the UE is performing a registration request towards the target AMF on a different access type with a new PLMN compared to the access type and PLMN at the source AMF and the new PLMN is not considered as equivalent PLMN, then the source AMF shall indicate no UE context exist to the target AMF if there is no UE context related to the access type or PLMN indicated by the target AMF. The target AMF shall then retrieve the SUCI from the UE and after receiving the Identity Response, the target AMF shall initiate a new primary authentication with the UE in order to establish a new security context with the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that: As described above in reason for change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 is not aligned with stage 3 in TS 29.518. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGE ****

#### 6.3.2.1	Multiple registrations in different PLMNs

The UE shall independently maintain and use two different 5G security contexts, one per PLMN's serving network. Each security context shall be established separately via a successful primary authentication procedure with the Home PLMN.

The ME shall store the two different 5G security contexts on the USIM if the USIM supports the 5G parameters storage. If the USIM does not support the 5G parameters storage, then the ME shall store the two different 5G security contexts in the ME non-volatile memory. Both of the two different 5G security contexts are current 5G security context.

The latest KAUSF result of the successful completion of the latest primary authentication shall be used by the UE and the HN regardless over which access network type (3GPP or non-3GPP) it was generated.

The HN shall keep the latest KAUSF generated during successful authentication over a given access even if the UE is deregistered from that access, but the UE is registered via another access.

During the Registration procedure in CM-IDLE mode, if the source AMF receives and detects that the UE is performing a registration request towards the target AMF on a different access type with a new PLMN compared to the access type and PLMN at the source AMF and the new PLMN is not considered as equivalent PLMN, then the source AMF shall indicate no UE context exist to the target AMF if there is no UE context related to the access type or PLMN indicated by the target AMF. The target AMF shall then retrieve the SUCI from the UE and after receiving the Identity Response, the target AMF shall initiate a new primary authentication with the UE in order to establish a new security context with the UE.

**** END OF CHANGE ****