3GPP TSG-SA3 Meeting #110Ad-Hoc-e	S3-231809

# Electronic meeting, Online, 17 - 21 April 2023

# Source:	Qualcomm Incorporated

# Title:	Update on solution #9

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

This contribution proposes to update the solution #9.

# 2	References

[1]	TS 23.586

[2]	TS 23.287

[3]	TR 33.893

# 3	Rationale

This contribution proposes to update the solution #9. Particularly, it is proposed to update an evaluation.

The clause 5.2.1 of TS 23.586 [1] specifies that the V2X capable UEs support the integrated discovery as defined in TS 23.287 [2]. This means the V2X capable UEs do not support the restricted 5G ProSe Direct Discovery.

Based on the above, it is proposed to update the evaluation that this solution is not aligned with SA2.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [3].

***** START OF CHANGES *****

### 6.9.3	Evaluation

The solution fulfills the security requirements of Key issue #1: Privacy protection for Ranging/SL Positioning services, Key Issue #3: Protection of discovery procedure and Key Issue #4: Protection of unicast direct communication.

The solution addresses how the V2X-capable Ranging UEs discover and establish the PC5 link. To secure the link establishment, the security materials (i.e. the long term credentials) are used. By reusing the direct security establishment procedure specified in TS 33.536 [5], the peer Ranging UEs can authenticate each other and protect the Ranging/SL Positioning information.

In this solution, the V2X-capable Ranging UEs use the discovery security materials associated with Ranging Service Code to protect the privacy information in DCR messages. The provisioning of discovery security materials reuses the discovery request procedure defined in clause 6.1.3.2 of TS 33.503 [6].

This solution requires 5G network entities (i.e., 5G DDNMF, PCF) to provide discovery security materials to V2X-capable UEs, which needs to be coordinated with SA2.

Editor’s Note:	Whether the Ranging Service Code is used in the Ranging/SL Positioning needs to be coordinated with SA2.

Editor’s Note:	Further evaluation is ffs.

***** END OF CHANGES *****