3GPP TSG-SA3 Meeting #110	S3-231170

# Athens Greece, 20 – 24 February 2023

# Source:	Apple

# Title:	PINE authentication

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Proposed to add a new solution in PIN security TR 33.882

# 2	References

Null

# 3	Rationale

This pCR proposes to add a new solution for key issue#1.

# 4	Detailed proposal

****START OF CHANGES ***

### 6.X	Solution #X: PINE authentication and authorization

### 6.X.1	Introduction

This solution provides an authentication method using primary authentication procedure. It is assumed in this solution that

- PEGC has connected with PINEs with some wireless technologies (e.g. WiFi, Bluetooth, etc),

- PINEs are provisioned with all the credentials needed for primary authentication.

- PEGC has been provisioned all the IDs of PINEs.

### 6.X.2	Solution details

Figure 6.x.2-1 Call flow of authentication for PINE

As show in the Figure 6.X.2-1, the details of authentication for PINE is as following, it was assumed that PEGC has acknowledged the PINE identities.

1. During the registration phase, in addition to its own SUCI/5G-GUTI, the PEGC will also include SUCIs/5G-GUTIs of other PINEs, that are in the same PIN and have connected to this PEGC, in the Registration Request Message. These UE-IDs are sent in a sequential order, i.e. SUCI1, SUCI2, etc.

2. When the AMF sends the request to the UDM/AUSF of the HPLMN, it will inform the HPLMN that all these UE-IDs (i.e.SUPI/SUCI here) belong to the same PIN. This can be done by either defining a new message, in which all SUPIs will be sent, or sending several requests and creating a linkage between them by means of (e.g.) using a “Link-ID” or “Operation-ID”, etc.

3. When this operation is used, the HPLMN UDM/AUSF will use the exact same RAND for all these PINE devices belonging to the same PIN and create all the Authentication Vectors for all of them

4.  UDM/AUSF then send only RAND and AUTN for PEGC to AMF.

5. The AMF then will send only ONE “RAND” in the Authentication Request message to the PEGC and also only one AUTN, corresponding to the PEGC’s RAND. In addition, the NW will also inform the PEGC that this particular Authentication Request also applies to one or more PINEs with the corresponding UE-IDs that were provided in the Registration Request Message.

6. The PEGC will then locally verify the NW according to the AUTN parameter check. Once the NW has been verified by the PEGC, it will send the RAND to all other PINE devices. PEGC will also calculate the PEGC RES.

7. PEGC sends RAND to all PINEs. All PINEs (behind the PEGC) have been configured to rely on the PEGC for the NW verification.

NOTE: As an alternative, the PEGC informs the PINE(s) that it has already verified the NW

8. Each PINE will calculate its own “RES” and send it to the PEGC.

9. The PEGC will then respond to the NW by Authentication Response message. In this message, it will include its own RES (PEGC RES) as well as RES(s) from other PINEs. The RES(s) from the other PINEs are sent in the same sequential order and based on the same bitmap that the NW sent to the PEGC in the Authentication Request Message.

10. AMF send Nausf_UEAuthentication_Authenticate Request including all the RES(s) to AUSF.

11. AUSF verifies all the RES(s).

12. AUSF respond to AMF on the authentication result.

13. AMF responds to PEGC on the authentication result.

### 6.X.3	Evaluation

This solution addresses the requirement in KI#1 on authentication for PINE.

This solution has impact on the current primary authentication.

****END OF CHANGES ***