3GPP TSG-SA3 Meeting #112 	S3-234066

Goteborg, Sweden, 14 - 18 August 2023

Title:	[DRAFT] Reply LS on Handling of SOR counter and the UE parameter update counter if stored in NVM

Response to:	LS S3-233506 / C1-232696 on Handling of SOR counter and the UE parameter update counter if stored in NVM from CT1

Release:	Rel-18

Work Item:	5GProtoc18

Source:	Thales (to be SA3)

To:	CT1

Cc:

Contact person:	Mireille Pauliac

mireille.pauliac@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none.

# 1	Overall description

SA3 thanks CT1 for their LS on Handling of SOR counter and the UE parameter update counter if stored in NVM.

SA3 considers that the problem described in this LS does not require to define additional mechanism for the following reasons:

The handling in the USIM of the key KAUSF, the key KSEAF, the SoR counter, and UE parameter update counter is specified in TS 31.102 [2]. Those keys and counters are stored in the file named EF5GAUTHKEYS.

Service n°123 for the storage of the key KAUSF was specified in Rel-15, while the Service n°133 to store SoR counter and UE parameter update counter was introduced in Rel-16. Consequently, from Rel-16 and beyond, the clause 4.4.11.6 in TS 31.102 related to the file EF5GAUTHKEYS also covers the scenario where the Service n°133 may not be “available” in EFUST with the sole objective to ensure backward compatibility with Rel-15. In terms of services, the Service n°133 goes hand in hand with Service n°123 in USIM from Rel-16 and beyond to avoid synchronization issue, e.g. as described in CT1 LS S3-233506/C1-232696.

When configuring Rel-16 and beyond USIM, if operator requests to have Service n°123 “available” in the EFUST, then the recommendation is to also have Service n°133 “available” in the EFUST. So, the problem described in LS from CT1 should not happen on the field for USIM from Rel-16 and beyond.

For Rel-15 USIM on the field with Service n°123 “available”, it is recommended to update by OTA (Over- The-Air) the USIM in order to make “available” the Service n°133 with associated counters in the file EF5GAUTHKEYS, under conditions that the USIM has been issued with the required OTA capabilities for such an update. This update does not impact the ME.

# 2	Actions

To CT1

ACTION: 	SA3 kindly asks CT1 to take into consideration information provided above.

# 3	Dates of next TSG SA WG 3 meetings

SA3#113	6 -10 November 2023	Chicago, US

SA3#114	22 -26 January 2024	UE (TBD)