3GPP TSG-SA3 Meeting #112 	S3-234259

# Gothenburg, Sweden, 14 - 18 August 2023                     merger of 233674 and 233877

# Source:	China Telecom, Huawei, HiSilicon, Interdigital, Philips, Ericsson, Qualcomm

# Title:	Selection methods between mechanisms with or without network assistance

# Document for:	Approval

Agenda Item:	4.25

# 1	Decision/action requested

It is requested to approve this proposal in the 5G_ProSe_Ph2 living doc.

# 2	References

[1]   S3‑233374  "Living document for 5G_ProSe_Ph2"

# 3	Rationale

This contribution proposes to add the selection methods between security mechanisms with or without network assistance.

As concluded in TR 33.740, it is need to specify a selection method between security mechanisms with or without network assistance for 5G ProSe UE-to-UE Relay Communication.

# 4	Detailed proposal

**** START OF CHANGE ****

#### 6.6.3.3	Selection between mechanisms with or without network assistance

A Network Assistance Security Indicator per RSC is provisioned in the 5G ProSe End UEs and 5G ProSe UE-to-UE Relay to indicate which mechanism is to be used between the security procedures with the network assistance and the security procedures without network assistance. The 5G ProSe End UEs shall select the mechanism between security procedures with network assistance and security procedures without network assistance based on the Network Assistance Security Indicator, while the 5G ProSe UE-to-UE Relay shall select the mechanism between security procedures with network assistance and security procedures without network assistance based on the Network Assistance Security Indicator and its 3GPP coverage status.

For 5G ProSe UE-to-UE Relay Communication with model A discovery, the 5G ProSe UE-to-UE Relay may select both RSCs associated with the security procedures with network assistance and the security procedures without network assistance when the 5G ProSe UE-to-UE Relay is in 3GPP coverage. The 5G ProSe UE-to-UE Relay shall only select the RSC associated with the security procedures without network assistance when the 5G ProSe UE-to-UE Relay is out of 3GPP coverage. Then, the 5G ProSe UE-to-UE Relay broadcasts a Discovery Announcement message including the selected RSC. The End UE shall use the security procedures with network assistance if the Network Assistance Security Indicator associated with an RSC indicates the security procedures with network assistance (as described in clause 6.6.3.1). Otherwise, if the Network Assistance Security Indicator associated with an RSC indicates the security procedures without network assistance, the End UE shall use the security procedures without network assistance (as described in clause 6.6.3.2).

For 5G ProSe UE-to-UE Relay Communication with model B discovery, the source End UE may select both RSCs associated with the security procedures with network assistance and the security procedures without network assistance, based on the desired mechanism. Then, the source End UE broadcasts a Discovery Solicitation message including the selected RSC. The UE-to-UE Relay shall use the security procedures with network assistance if the Network Assistance Security Indicator associated with the RSC indicates the security procedures with network assistance and it is inside 3GPP coverage. Otherwise, if the Network Assistance Security Indicator associated with the RSC indicates the security procedures without network assistance, the UE-to-UE Relay shall use the security procedures without network assistance.

**** END OF CHANGE ****