# 3GPP TSG-SA3 Meeting #112 	S3-233723

# Goteborg, Sweden, 14 - 18 August 2023	(revision of xx-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Authorization of AF or 5GC NF for UEs belonging to several PLMNs

# Document for:	Approval

Agenda Item:	4.26

1	Decision/action requested

Approve the changes in TS 33.533.

2	References

[1]	TS 23.273

3	Rationale

Considering the UEs involved in the ranging service may belong to different PLMNs, the GMLC may interacts with the UDM directly for UEs within the same PLMN or retrieve the UE privacy profile from UE’s home UDM. To achieve this, NL3 (between two GMLCs) and NL6 (between GMLC and UDM) interfaces are used.

In addition, the editorial change with normative language is provided.

4	Detailed proposal

************ START OF CHANGES ************

6.3.3	Procedure for authorization of AF/5GC NF for Ranging/SL positioning service exposure

For the authorization of the AF or 5GC NF for Ranging/SL Positioning service exposure, the MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards both target UE and reference UE. If none of the UEs grants or one of the UEs does not grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF/5GC NF.

Editor’s Note: the MT-LR procedure and related reference need to be updated based on the progress of TS 23.586[2].

When receiving the Ranging/SL Positioning service request from the AF/5GC NF, the GMLC shall interact with the UDM directly to check the UE privacy profile for the UEs belonging to the same PLMN. In addition, for the UEs belonging to different PLMNs, NL3 and NL6 interfaces are used for GMLC to retrieve the UE privacy profile from UDM in UE’s home PLMN. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

Editor’s Note: the details of the content of UE privacy profile for Ranging/SL positioning services is FFS.

The GMLC shall interact with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message shall be sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification is required, the AMF shall provide failure response to the GMLC.

************ END OF CHANGES************