3GPP TSG-SA3 Meeting #112	S3-233877

# Gothenburg, Sweden, 14 - 18 August 2023

# Source:	Qualcomm Incorporated

# Title:	Updating selection mechanisms in ProSe draft CR

# Document for:	Approval

Agenda Item:	4.25

# 1	Decision/action requested

This pCR proposes to update the selection mechanism.

# 2	References

[1]	TR 33.740 v0.7.0

[2]	S3-232007 ProSe Phase 2 draft CR

# 3	Rationale

This pCR proposes to add a description regarding the selection between mechanisms with or without network assistance based on the conclusion [1].

# 4	Detailed proposal

SA3 is kindly requested to approve the below pCR for inclusion in the draft CR in [2].

************ START OF 1st CHANGE*******

#### 6.6.3.3	Selection between mechanisms with or without network assistance

A Network Assistance Security Indicator is provisioned in the End UEs and 5G ProSe UE-to-UE Relay along with the RSC to indicate which mechanism is to be used between the security procedures with the network assistance and the security procedures without network assistance.

NOTE 1: When both RSC with network assistance and RSC without network assistance are provisioned at the UE for the same relay service, which RSC to use is up to UE implementation.

For 5G ProSe UE-to-UE Relay Communication with model A discovery, the 5G ProSe UE-to-UE Relay may select both RSCs, each associated with its respective mechanisms when the 5G ProSe UE-to-UE Relay is in 3GPP coverage. The 5G ProSe UE-to-UE Relay shall only select the RSC associated with the security procedures without network assistance when the 5G ProSe UE-to-UE Relay is out of 3GPP coverage.

For 5G ProSe UE-to-UE Relay Communication with model B discovery, the source End UE shall select the RSC based on the desired mechanism. Then, the source End UE broadcasts a discovery message including the selected RSC to discover a target End UE via a 5G ProSe UE-to-UE Relay that supports the indicated mechanism. The 5G ProSe UE-to-UE Relay may support both RSCs, each associated with its respective mechanisms when the 5G ProSe UE-to-UE Relay is in 3GPP coverage. The 5G ProSe UE-to-UE Relay shall only support the RSC associated with the security procedures without network assistance when the 5G ProSe UE-to-UE Relay is out of 3GPP coverage.

************ END OF 1st CHANGE*******