3GPP TSG-SA3 Meeting #112 	S3-234195

# Goteborg, Sweden, 14 - 18 August 2023										(revision of S3-234003)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | draftCR | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on ECS and EES authentication method indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Lenovo, InterDigital, Intel, Thales, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This contribution proposes the draftCR of EDGE_Ph2, according to the conclusion of key issue#2.2 inTR 33.739 on authentication method selection |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2 and 6.3 are updated as follows: Clause 6.2: Authentication method negotiation mechanism shall re-use existing TLS v1.3 as specified in TS 33.222 [12]. UE may receive supported authentication method of ECS as part of ECS configuration information. Details of ECS configuration information are specified in TS 23.558 [5]. If the UE has the information about the authentication method supported by the ECS, then the EEC/UE may use this information for the authentication method negotiation. Clause 6.3: Authentication method negotiation mechanism shall re-use existing TLS v1.3 as specified in TS 33.222 [12]. UE may receive supported authentication method of EES as part of ECS configuration information. Details of ECS configuration information are specified in TS 23.558 [5]. If the UE has the information about the authentication method supported by the EES, then the EEC/UE may use this information for the authentication method negotiation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete work for the phase 2 edge features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This draftCR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

## 6.2	Authentication and authorization between EEC and ECS

The ECS shall be configured with the information of authorization methods (token-based authorization or local authorization) used by EESes.

Authentication between EEC and ECS shall be done during the execution of the TLS handshake protocol. Server side certificate-based TLS authentication shall be supported. Details of the authentication method (e.g., TLS certificates, usage of AKMA [11] or GBA [12] as methods to arrange the PSK for TLS) are out of scope of the present document.

NOTE:	If only Server side certificate-based TLS authentication is performed, it is left to implementation on which information within a service procedure and services will be provided by the ECS.

The authentication method negotiation mechanism shall re-use the existing TLS v1.3 negotiation. UE may receive the supported authentication method of the ECS optionally as part of the ECS configuration information. Details of the ECS configuration information are specified in TS 23.558 [5]. If the UE has the information about the authentication method supported by the ECS, then the EEC/UE may use this information for the authentication method negotiation.

NOTE:	Further optimization regarding having prior knowledge about the capability, such as UE storing the selected algorithm from the past negotiation results, is left to EEC/UE implementation.

If the GPSI is required, the ECS shall retrieve the GPSI from the core network no matter whether the EEC sends the GPSI to the ECS.

NOTE: If the ECS identifies a mismatch between the GPSI received from the EEC and the GPSI received from the network, the decision and action to be taken by the ECS for such mismatch cases are left to implementation.

After successful authentication, the ECS shall authorize the EEC by its local authorization policy.

After successful authentication and authorization, the ECS decides whether OAuth 2.0 [15] access tokens are required for the candidate EESes using the configuration information and issues separate EES access tokens to be used for each candidate EESes that use token-based authorization. The ECS, EEC and EES respectively assume the role of authorization server, client and resource server roles defined in [15]. "Client Credentials" grant type and bearer tokens [16] shall be used. JSON Web Token (JWT) as specified in IETF RFC 7519 [17] for encoding and the JSON signature profile as specified in IETF RFC 7515 [18] for protection of tokens shall be followed. This token profile also applies for clause 6.3 of the present document. The claims of the EES service tokens in the form of JWT [17] shall include the ECS FQDN (issuer), EEC ID (client_id), GPSI (subject), expected EES service name(s) (scope), EES FQDN (audience), expiration time (expiration). The ECS shall send the service response back to the EEC, which may include EES access token(s).

**** Next Change****

## 6.3	Authentication and authorization between EEC and EES

Authentication between EEC and EES shall be done during the execution of the TLS handshake protocol. Server side certificate-based TLS authentication shall be supported. Details of the authentication method (e.g., TLS certificates, usage of AKMA [11] or GBA [12] as methods to arrange the PSK for TLS) are out of scope of the present document.

NOTE:	If only Server side certificate-based TLS authentication is performed, it is left to implementation on which information within a service procedure and services will be provided by the EES.

The authentication method negotiation mechanism shall re-use the existing TLS v1.3 negotiation. UE may receive the supported authentication method of the EES optionally as part of the EES configuration information. Details of the EES configuration information are specified in TS 23.558 [5]. If the UE has the information about the authentication method supported by the EES, then the EEC/UE may use this information for the authentication method negotiation.

NOTE:	Further optimization regarding having prior knowledge about the capability, such as UE storing the selected algorithm from the past negotiation results, is left to EEC/UE implementation.

If the GPSI is required, the EES shall retrieve the GPSI from the core network no matter whether the EEC sends the GPSI to the ECS.

NOTE: If the EES identifies a mismatch between the GPSI received from the EEC and the GPSI received from the network, the decision and action to be taken by the EES for such mismatch cases are left to implementation.

For authorization of EEC by the EES, the EEC shall send the OAuth 2.0 [15] access token, if received from the ECS, to the EES. The token profile is specified in clause 6.2 of the present document. If the EES requires access token for authorization, then the EES shall authorize the EEC by using the token. Otherwise, the EES shall authorize the EEC by its local authorization policy.

After successful authentication and authorization, the EES shall process the request and sends the service response back to the EEC.

**** End of Changes****