3GPP TSG-SA3 Meeting #114e ad-hoc	S3-240036-r2

Electronic meeting, online, 22 - 26 January 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0151 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to the clause 4.2.2.2.2 - Protection at the transport layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-12-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to NESAS, the test case applies to SBI only, but is not explicitly stated in clause 4.2.2.2.2 of TS 33.117 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to update clause 4.2.2.2.2 to explicitly state that the test case on “Protection at the transport layer “ applies only to SBI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear test cases in TS 33.117. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of Change *************

##### 4.2.2.2.2	Protection at the transport layer

Requirement Name: Protection at the transport layer

Requirement Reference: TS 33.501 [10], clause 5.9.2.1, clause 13.1, clause 13.3.2

Requirement Description:

NF Service Request and Response procedure supports mutual authentication between NF consumer and NF producer as specified in TS 33.501 [10], clause 5.9.2.1 .

All network functions supports TLS. Network functions support both server-side and client-side certificates. The TLS profile follows the profile given in Annex E of TS 33.310 [9] with the restriction to be compliant with the profile given by HTTP/2 as defined in RFC 7540 [11] as specified in TS 33.501 [10], clause 13.1.

Authentication between network functions within one PLMN uses one of the following methods:

-	If the PLMN uses protection at the transport layer as described in clause 13.1, authentication provided by the transport layer protection solution is used for authentication between NFs as specified in TS 33.501 [10], clause 13.3.2.

This test case only applies to service-based interfaces that is used between NF consumer and NF producer defined in TS 33.501[10].

Threat References: TR 33.926 [4], clause 5.3.6.3, Weak cryptographic algorithms

Test case:

Test Name: TC_PROTECT_TRANSPORT_LAYER

Purpose:

Verify that TLS protocol for NF mutual authentication and NF transport layer protection is implemented in the network products based on the profile required.

Procedure and execution steps:

Pre-Conditions:

Network product documentation containing information about supported TLS protocol and certificates is provided by the vendor.

A peer implementing the TLS protocol configured by the vendor shall be available.

The tester shall base the tests on the profile defined by 3GPP in Annex E of TS 33.310 [9] with the restriction that it shall be compliant with the profile given by HTTP/2 as defined in RFC 7540 [11].

Execution Steps

1.	The tester shall check that compliance with the TLS profile can be inferred from detailed provisions in the network product documentation.

2.	The tester shall establish a secure connection between the network product under test and the peer and verify that all TLS protocol versions and combinations of cryptographic algorithms that are mandated by the TLS profile are supported by the network product under test.

3.	The tester shall try to establish a secure connection between the network product under test and the peer and verify that this is not possible when the peer only offers a feature, including protocol version and combination of cryptographic algorithms, that is forbidden by the TLS profile.

Expected can Results:

-	The network product under test and the peer establish TLS if the TLS profiles used by the peer are compliant with the profile requirements in TS 33.310 [9] Annex E and RFC 7540 [11].

- 	The network product under test and the peer fail to establish TLS if the TLS profiles used by the peer are forbidden in TS 33.310 [9] Annex E or RFC 7540 [11].

Expected format of evidence:

Provide evidence of the check of the product documentation in plain text. Save the logs and the communication flow in a .pcap file.

************* End of Change *************