3GPP TSG-SA3 Meeting #114-Ad Hoc-e	draft_S3-240063-r1

# Online, , 22nd Jan 2024 - 26th Jan 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0168 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for 4.3.4.2 - 33.117 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarifications in the test case preconditions, results and format of evidences |  |  |  |  |  |  |  |  |
| Summary of change: |  | Same as above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test case specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* FIRST CHANGE *******

#### 4.3.4.2	No system privileges for web server

Requirement Name: No system privileges for web server.

Requirement Reference: In accordance with industry best practice

Requirement Description:

No web server processes shall run with system privileges. This is best achieved if the web server runs under an account that has minimum privileges. If a process is started by a user with system privileges, execution shall be transferred to a different user without system privileges after the start.

Threat References: TR 33.926 [4]

Test Case:

Test Name: TC_NO_SYSTEM_PRIVILEGES_WEB_SERVER

Purpose:

Verify that the Web server is not run under system privileges.

Procedure and execution steps:

Pre-Conditions:

-	The tester has needed administrative privileges.

-	A tester machine is available.

-	Recommended: an automatic assessment tool has been configured /script adapted in line with the Requirement Description.

Execution Steps

1.	Check that no web server processes run with system privileges. Check that this is the case even for processes that may have been started by a user with system privileges.

a.	Start the web server process as web server user and check process privileges.

b.	If possible, start the web server process as with system privileges and check if process privileges get dropped.

2.	Check in relevant system settings and web server configurations that a web server user is configured with minimal privileges needed to run the web server and the web server is executable by that user.

Expected Results:

-	There are no findings of web server processes that run with system privileges.

-	System settings are set to ensure that no processes will run with system privileges.

Expected format of evidence:

A testing report providing the following information:

-	Log files / command line output and screen shots of test executions

-	Part of web server and/or system configuration (plain text or screenshot) showing the configured user for the web server process

-	Test result (Passed or not)

******* END FIRST CHANGE *******