# 3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241113-r3

# Electronic meeting, online, 15 - 19 April 2024	revision of S3-24yyyy

Source:	vivo, Interdigital, ZTE, Ericsson, OPPO, CATT, Qualcomm, Xiaomi, Lenovo and Apple

Title:	Key issue on privacy for AIoT services

Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve the pCR on new key issue on privacy for AIoT services.

2	References

3		Rationale

This contribution proposes a new key issue on the privacy for AIoT services.

4	Detailed proposals

*** BEGIN CHANGES ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-49: "Study on Enhancement of support for Edge Computing in 5G Core network - Phase 3".

[xx]	3GPP TR 23.700-13: “Study on Architecture support of Ambient power-enabled Internet of Things”.

*** NEXT CHANGES ***

5.X	Key issue #X: Privacy for AIoT services

5.X.1	Key issue details

5G Ambient IoT service is a type of cellular IoT communication system where Ambient IoT devices utilize harvested energy to generate RF signals for bi-directional information transmission.

TS 22.369 [x] clause 5.2.6 defines the following privacy-related requirements:

“The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE.”

In AIoT services, device ID is a unique identifier associate to a device, and is exclusively used to identify the  device and related information for the associated service. If the privacy of device ID is not ensured, an attacker can identify and track an AIoT device based on the device ID associated to the AIoT device.

Thus, this key issue is to investigate potential mechanisms to ensure privacy of AIoT device identifiers considering the specific use cases and limited device capability that are differentiated from the exiting IoT technologies such as eMTC, NB-IoT and RedCap.

5.X.2	Security Threats

If the privacy of device ID is not ensured, an attacker can identify and track an AIoT device based on the device ID associated to the AIoT device.

5.X.3	Potential security requirements

The 3GPP System shall support a mechanism for mitigating trackability and linkability attacks on AIoT Devices.

Long-term identifiers of AIoT devices shall be confidentiality protected over the air.

NOTE: 	Not all AIOT services/device types are expected to require support of features to solve the above requirement and hence support of such features are not mandated.

*** END OF CHANGES ***