3GPP TSG-SA3 Meeting #115AdHoc-e	S3-241305-r1

Electronic meeting, online, 15 - 19 April 2024

# Source:	Huawei, HiSilicon, Ericsson

# Title:	Update to the profiling for TLS 1.3

# Document for:	Approval

Agenda Item:	4.8

# 1	Decision/action requested

Approve the pCR to the draft CR for TS 33.210

# 2	References

[1]	https://www.bsi.bund.de/SharedDocs/Downloads/EN/BSI/Publications/TechGuidelines/TG02102/BSI-TR-02102-2.pdf

# 3	Rationale

According to the guidelines in [1] by BSI, ffdhe2048 can only be used up to 2022 and psk_ke only up to 2026. Some TLS 1.3 libraries do not support ffdhe2048 and psk_ke due to the inadequate security. Thus, ffdhe2048 shall not be supported.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.2.2	Profiling for TLS 1.3

TLS 1.3 shall support the following restrictions and extensions:

TLS cipher suites and Diffie-Hellman groups

-	The requirements given in section 9.1 of TLS 1.3 RFC 8446 [66] shall be followed. In addition:

-	Key exchange with secp384r1 should be supported.

-	Ffdhe2048 shall not be supported.

TLS signature schemes

-	ecdsa_secp384r1_sha384 should be supported.

TLS extensions

-	The requirements given in section 9.2 of TLS 1.3 RFC 8446 [66] shall be followed. In addition:

-	The OCSP Status extension (a.k.a. certificate status request), as defined in RFC 6066 [57] and RFC 8466 [66] should be supported.

*** End of 1st Change ***