3GPP TSG-SA3 Meeting #116	S3-241957

Jeju, South Korea, 20th - 24th May 2024           								         (revision of S3-24xxxx)

# Source:	Huawei, HiSilicon

# Title:	Revocation supporting multiple USS

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR to TR 33.759

# 2	References

None

# 3	Rationale

The current revocation procedures works only for single USS. This contribution proposes amendments to the procedure in order to support multiple USS. As a side note, the re-authentication procedure in 5.2.1.4 in TS 33.256 is not affected as long as UUAA is updated to support mulitple USS.

# 4	Detailed proposal

pCR

*** 1st CHANGE ***

## 6.Y	Solution # Y: revocation supporting multiple USS

### 6.Y.1	Introduction

This solution addresses the key issue #1. The solution adapts the revocation procedure in TS 33.256 [4] to support multiple USS.

### 6.Y.2	Solution details

With reference to the clause 5.2.1.5 in TS 33.256 [4], the steps are amended as follows to support multiple USS:

1-2. no change to the corresponding steps in the clauses 5.2.1.4 in TS 33.256 [4].

3a or 3b. The UUAA revocation message that the UAS NF sent to the target AMF or SMF also includes the USS identifier.

3c-4. no change.

4a or 4b. The UUAA revocation indication message that the target AMF or the SMF sent to the UE contains also the USS identifier.

5. The UE on receiving the revocation indication only delete authorization data corresponding to the USS identifier.

### 6.Y.3	Evaluation

This solution addresses the key issue #1. It amends the revocation procedures in TS 33.256 [4] in order to support multiple USS. It has the following impacts:

- UAS NF: sends USS identifier to the UE which deletes authorization data specific to the USS, not all.

*** END OF 1st CHANGE***