3GPP TSG-SA3 Meeting #119AdHoc-e	draft_S3-250058-r10

# Online, Electronic meeting, 13 -16 January 2025

Source:	OPPO, Huawei, HiSilicon, Apple,…

Title:	Pseudo-CR on Conclusion on KI#5 AIoT Authentication

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.5.0

Work Item:	FS_Ambient_IoT_Sec

Comments

It is proposed to agree the conclusion on KI#5 of AIoT authentication.

There are 16 solutions for KI#5 AIoT authentication in 33713 v 050. In order to make progress on conclusion for KI#5, observations and proposals for authentication solutions are listed as following

- Observation 1: Direction of authentication: most of the authentication solutions proposed mutual authentication instead of one-way authentication. Therefore, it is proposed to agree mutual authentication as baseline for normative work.

- The number of mutual authentication solutions is TWELVE: #6, #7, #8, #10, #11, #12, #13, #32, #35, #36, #37, #39.

- The number of network authenticating device only solutions is FOUR: #4, #9, #38, #42.

- Observation 2: Network element performing authentication: All of SIXTEEN solutions proposed to use network element to perform authentication. Therefore, it is proposed to agree network layer authentication as baseline for normative work.

Which exact network element will store and process subscription data of AIoT device is pending on SA2 agreement.

  - AIoT Authentication Function/ Authentication Server: #4, #7, #32, #42

  - AIoT Controller: #6

  - AIoT UDM/AUSF: #8, #11, #12, #37, #39

  - AIoT F/NF: #9, #13, #35, #36, #38

  - AAA-S: #10

- Observation 3: Authentication credential: Twelve out of SIXTEEN proposed to use pre-provisioned shared key between AIoT device and network for authentication. Therefore, it is proposed to agree to use pre-provisioned shared key between AIoT device and network for authentication.

  - Root Key/K/ pre-provisioned key: #4, #7, #8, #9, #11, #12, #13, #32, #35, #37, #38, #39, #42

  - Derived Kaiot: #6

  - AIoT credentials: #10

  - Not clear: #36

- Observation 4: All these solutions propose Challenge-Response based procedure to perform authentication.

- Observation 5:  fresh parameter, ELEVEN out of SIXTEEN proposed to use random number from network side for authentication. Therefore, it is proposed to agree to use random number from network side for authentication.

  - RAND/NONCE from network side: #4, #7, #8, #9, #11, #12, #13, #35, #37, #38, #39

  - RAND/NONCE from device side in addition of a): #4, #9, #35, #39

  - Counter: #6,

  - freshness parameter: #42

  - L1 parameter: #32,

  - Not clear: #10, #36

Proposed Changes

* * * First Change * * * *

# 7	Conclusions

## 7.x 	Conclusion on KI#5

The credential (including device ID and authentication credential) for AIoT device authentication can be owned by an operator or by a third party. The principles for device ID management are concluded in 8.2.1 in TR 23.700-13 [4].

The AIoT system shall support a means to provide one-way authentication of AIoT device in the Inventory-only procedure.

NOTE X: Details of the one-way authentication procedure (e.g., parameter(s) used, whether one-way authentication can be implicit or not, etc.)  are to be resolved during normative phase.

The AIoT system shall support a means to provide mutual authentication between AIoT device and the network in the Inventory and Command procedure.

NOTE Y: Details of the mutual authentication procedure (e.g., parameter(s) used for the network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

Editor’s Note: Where to store the AIoT device subscription will be based on SA2 and SA3 coordination.

* * * End of Changes * * * *