3GPP TSG-SA3 Meeting #119AdHoc-e	draft_S3-250087-r11

# Online, Electronic meeting, 13 -16 January 2025

Source:	Samsung, ZTE, China Telecom, Ericsson, Xiaomi, Nokia

Title:	Updates to conclusion for key issue#2

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.700-22

Version:	0.3.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This pCR proposes to update the conclusion for key issue#2 on CAPIF interconnection security.

Proposed Changes

* * * First Change * * * *

## 7.2	Conclusion for KI #2: CAPIF interconnection security

### 7.2.0 	General

It is assumed that the API invoker onboards to CCF-A, which is referred as onboarded CCF.

It is assumed that the API invoker is onboarded to CCF-A and the target AEF is registered to a different CCF-B.

### 7.2.1 	Conclusion for CAPIF 6/6e security

It is concluded that for CAPIF-6 and CAPIF-6e reference points, same security mechanisms specified in clauses 6.6 and 6.10 of TS 33.122 [4] for CAPIF-3/4/5 and CAPIF-3e/4e/5e reference points will be used, respectively.

### 7.2.2 	Conclusion for security method negotiation

For security method negotiation procedure (as per requirement 2), clause 6.3.1.2 in TS 33.122 [4] will be used as baseline with the necessary enhancements (if any).

The details of how security method selection is done for the CAPIF 2/2e reference point based on the capabilities of the API Invoker and the AEF capabilities (that belongs to CCF-B) are up to normative work.

### 7.2.3	Conclusion for API invoker authentication and authorization mechanism

For mutual authentication and authorization between API invoker (onboarded to CCF-A) and the AEF (registered to CCF-B), the procedures as defined in clause 6.5.2 of TS 33.122 [4] can be re-used with the following enhancement:

- When using TLS-PSK or PKI:

  - On receiving the request from the AEF, CCF-B requests the security information (AEFPSK/root CA) from CCF-A (over CAPIF-6/6e reference point).

  - The AEF learns the access control policy from CCF-B.  If the authorization access policy at CCF-B is not enough, then CCF-B learns access control policy from CCF-A and provides it to the AEF.

- When using TLS with OAuth token:

  - On receiving the request from the AEF, CCF-B requests the security information (e.g., root CA) from CCF-A (over CAPIF-6/6e reference point).

  - CCF-A sends the access token request to CCF-B.

  - CCF-B provides an access token to the API invoker via CCF-A as specified in clause 6.5.2.3 in TS 33.122 [4].

  - The AEF verifies the access token as described in 6.5.2.3 in TS 33.122 [4].

  - Whether CCF-A can issue an access token is to be determined during normative work.

- For the case of TLS-PSK or PKI, and TLS with OAuth token methods listed above, the specific details of how the CCF-B requests the security information of the API Invoker from the right CCF-A, i.e. where API invoker is onboarded, are up to normative work.

- For the case of TLS with OAuth token methods, how CCF-A determines to request token from CCF-B is up to the normative work.

- How CCF-B verifies that CCF-A is authorized for the service is up to normative work.

- Further details of the procedure are to be determined during normative work.

* * * * End of Changes * * * *