3GPP TSG-SA3 Meeting #120	S3-250398

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on Technical Content

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	v0.0.0

Work Item:	FS_CryptInv

Comments

This TR has been new created and first and initial content should be provided.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	TS 33.501, 5G System Security Architecture

[x2]	SECG SEC 1, http://www.secg.org/sec1-v2.pdf

[x3]	SECG SEC 2, http://www.secg.org/sec2-v2.pdf

[x4]	…

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES			Advanced Encryption Standard

AKA			Authentication and Key Agreement

DSA			Digital Signature Algorithm

ECC			Elliptic Curve Cryptography

ECIES			Elliptic Curve Integrated Encryption Scheme

ECDH			Elliptic Curve Diffie-Helman

ECDHE			Elliptic Curve Diffie-Helman Ephemeral

ECDSA			Elliptic Curve	Digital Signature Algorithm

EdDSA			Edwards-Curve Digital Signature Algorithm

FIPS			Federal Information Processing Standards

GSM			Global System for Mobile Communications

GSMA			GSM Association

HMAC			Hash-based Message Authentication Code

JSON			JavaScript Object Notation

JWE			JSON Web Encryption

JWS			JSON Web Signature

LS			Liaison Statement

MAC			Message Authentication Code

RSA			Rivest-Shamir-Adleman

SECG			Standards for Efficient Cryptography Group

SEPP			Security Edge Protection Proxy

SHA			Secure Hash-Algorithm

TLS			Transport Layer Security

UE			User Equipment

* * * Next Change * * * *

# 4	Inventory of security protocols

## 4.y	3GPP Cryptographic Inventory – 5G System

### 4.y.1	General

The cryptographic inventory is divided into three main parts, i.e., one for the symmetric, another for the asymmetric, and one for the mixed algorithms. The algorithms and corresponding details will be presented in tabular format with additional text description.

* * * Next Change * * * *

### 4.y.2	Symmetric Cryptographic Algorithms

The security for the NAS messages between UE and AMF and the RRC signalling as well as the UP data between UE and gNB is based on symmetric algorithms.

The two primary authentication methods, i.e., the 5G AKA and the EAP-AKA, are challenge-and-response protocols which are based on a shared secrete key (K). The shared secrete is classified as being a symmetrical key. The 5G-AKA is computing hash values (see Annex A.5 of TS 33.501). The EAP-AKA’ is based on EAP-TLS procedures (see Annex B of TS 33.501, [x1]).

For the NPN the two authentication methods (e.g., 5G AKA and EAP-AKA’ might apply).

Table 4.y.2-1: 3GPP specific symmetric algorithms (5G System)

| Protocol | Traditional Algorithm | Algorithm Development Organisation | Algorithm Specification | 3GPP Cryptographic Profile | Usage Type |
| --- | --- | --- | --- | --- | --- |
| PDCP (TS 38.323) | 128-NxA1 | ETSI SAGE | TS35.215, TS35.216, TS35.217, TS35.218 | TS 33.501 | Confidentiality and Integrity Protection for the Air Interface (Uu-Interface). |
|  | 128-NxA2 |  | FIPS PUB 197 |  |  |
|  | 128-NxA3 |  | TS35.221, TS35.222. TS35.223 |  |  |
| EAP | MILENAGE | ETSI SAGE | TS35.205, TS35.206, TS35.207, TS35.208 | TS 24.501 | Authentication and Key Agreement (EAP-AKA’, 5G AKA) |
|  | MILENAGE-256 |  | TS35.234, TS35.235, TS35.236, TS35.237 |  |  |
|  | TUAK-128/256 |  | TS35.231, TS35.232, TS35.233 |  |  |

NOTE x1: NxA1 is Snow 5G based encryption/integrity protection algorithm, NxA2 is AES based encryption/integrity protection algorithm, NxA3 is ZUC based encryption/integrity protection algorithm.

NOTE x2: MILENAGE-128 is AES based algorithm for authentication and key generation, while the MILENAGE-256 employs Rijndael-256-256.

NOTE x3: TUAK is Keccak based algorithm for authentication and key generation.

* * * Next Change * * * *

### 4.y.3	Mixed Cryptographic Algorithms

The Elliptic Curve Integrated Encryption Scheme (ECIES) specified in SECG is used to encrypt a SUPI. The ECIES specification combines the ECC-based asymmetric cryptography with symmetric ciphers to provide data encryption.

The below Table 4.y.3-1 shows the cryptographic algorithms which are directly used by 3GPP functions. The term ‘directly used’ refers to the fact that these algorithms are used without any modification/adaptation.

Table 4.y.3-1: 3GPP Directly Used Algorithms (5G System)

| Protocol | Traditional Algorithm | Algorithm Development Organisation | Algorithm Specification | 3GPP Cryptographic Profile | Usage Type |
| --- | --- | --- | --- | --- | --- |
| ECIES | ECC | SECG | SECG SEC 1, SEC 2 (NOTE n1) | TS 33.501 | Elliptic Curve Cryptography, SUCI |
| KDF | HMAC-SHA-256 | IETF | FIPS 180-4, RFC 2104, RFC 6151 | TS 33.501, TS 33.220 | Key Derivation Function (KDF) |

NOTE n1: For the specifications SECG SEC1 please refer to [x2] and for SECG SEC2 please refer to [x3]. The documents specify public-key cryptographic schemes based on elliptic curve cryptography (ECC). In particular, it specifies signature schemes, encryption and key transport schemes and key agreement schemes.

Editor’s Note: Provide definition for the term ‘mixed cryptographic algorithms’, specifically what is the meaning of the term ‘mixed’.

* * * Next Change * * * *

### 4.y.4	Asymmetric Cryptographic Algorithms

Table 4.y.4-1: 3GPP indirectly used algorithms (5G System)

| Protocol | Traditional Algorithm | Algorithm Development Organisation | Algorithm Specification | 3GPP Cryptographic Profile | Usage Type |
| --- | --- | --- | --- | --- | --- |
| TLS (RFC8446) | (EC)DHE | IETF | RFC8996 | TS 33.210 | Key Agreement |
|  | RSA |  | RFC8017 |  |  |
|  | (EC)DHE |  | RFC8996 |  | Digital signature and authentication |
|  | RSA |  | RFC8017 |  |  |
| OAuth 2.0 | Tbd |  |  |  |  |
| DTLS | Tbd |  |  |  |  |
| IPSec (RFC6071) | Diffie-Hellman | IETF | RFC2631 | TS 33.210 | Key Agreement |
|  | (EC)DHE |  | RFC8996 |  | Digital signature and authentication |
|  | RSA |  | RFC8017 |  |  |
| JWS (RFC7515) | ECDSA | IETF | RFC6605 | TS 33.210 | Integrity Protection |
| JWE (RFC 7516) (NOTE p1) | (EC)DHE |  | RFC8996 |  | Key Exchange |
|  | RSA |  | RFC8017 |  |  |

Editor’s Note: The ‘Tbd’ is to be replaced by valid content in next versions of this TR.

NOTE p1: JWE is used for the message encryption between the SEPPs and it uses ECDHE or RSA (e.g., TLS based key exchanged via separate N32c) for key exchange.

* * * End of Changes * * * *