3GPP TSG-SA3 Meeting #120	S3-250434

Athens, Greece, 17 - 21 February 2025

Source:	Qualcomm Incorporated

Title:	Updates on KI#1 conclusions

Document for:	Approval

Agenda item:	5.12

Spec:	3GPP TR 33.743

Version:	1.0.0

Work Item:	5G_ProSe_Ph3_SEC

Comments

This contribution proposes to address the Editor’s Notes in the KI#1 conclusion.

First, according to S3-2501295 in SA2, it is updated that the PC5 link establishment with its parent Intermediate UE-to-Network Relay or 5G ProSe UE-to-Network Relay before sending the Announcement message became mandatory. Thus, it is proposed to remove the following Editor’s Note.

Editor’s Note:  How the PC5 link establishment is performed based on the optional procedure (step 6 in clause 6.3.2.5.2 of TS 23.304 [4]) is FFS.

Second, authentication and authorization between the 5G ProSe Remote UE and potential 5G ProSe UE-to-Network Relay is done in a hop-by-hop manner during PC5 link security establishment for each hop according to the existing conclusion of 1) and 2). Thus, it is proposed to remove the following Editor’s Note.

Editor’s Note: Further work on authentication and authorization between the 5G ProSe Remote UE and potential 5G ProSe UE-to-Network Relay via the network is FFS.

Third, it is already captured that the existing Remote UE report procedure is reused as a baseline for normative work. Thus, it is proposed to remove the following Editor’s Note and specify the detailed procedure during normative work if needed.

Editor’s Note: Further work on Remote UE report procedure is FFS.

Proposed Changes

* * * First Change * * * *

## 7.2	Key issue #1: Security for multi-hop UE-to-Network Relay

The following statements are agreed as a basis for normative work:

For 5G ProSe Multi-hop UE-to-Network Relay discovery:

- The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay’s HPLMN respectively, based on the procedure specified in clause 6.1.3.2 of TS 33.503 [5].

- The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

- The Intermediate UE-to-Network Relay processes the received discovery message using the provisioned discovery security materials associated with the RSC as specified in clause 6.1.3.2 of TS 33.503[5]. If the processing is successful, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count, Relay Info.) and protects the updated message using the same security materials associated with the RSC as specified in clause 6.1.3.2 of TS 33.503 [5].

NOTE 1: Since the Intermediate UE-to-Network Relay and 5G ProSe UE-to-Network Relay are provisioned with a single set of discovery security materials from the 5G PKMF/5G DDNMF in their own HPLMN, it is assumed Intermediate UE-to-Network Relays and 5G ProSe UE-to-Network Relay belong to the same HPLMN during 5G ProSe multi-hop UE-to-Network Relay discovery.

- Both discovery with Model A and discovery with Model B are supported.

- For discovery with Model A, the Intermediate UE-to-Network Relay may (as specified in step 6 in clause 6.3.2.5.2 of TS 23.304 [4]) protect and send the Announcement message after a secure PC5 link establishment with the upstream Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay.

- For discovery with Model B, the 5G ProSe Remote UE protects the discovery message (i.e., Relay Discovery Solicitation) reusing the procedures as specified in clause 6.1.3.2.3 of TS 33.503[5], i.e., using the discovery security materials associated with the RSC and HPLMN ID of the potential 5G ProSe UE-to-Network Relay.

For Key issue #1 on the support Security for multi-hop UE-to-Network Relay communication, the following principles apply:

1)	For UE-to-Network Relay communication security following Model A discovery:

- The security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay is based on CP-based/UP-based procedures as specified in clause 6.3.3 of TS 33.503 [5].

2)	For UE-to-Network Relay communication security following Model B discovery, the existing CP-based/UP-based procedures as specified in the in clauses 6.3.3 of TS 33.503 [5] are used as baseline for the normative work with the following principles:

- To trigger the multi-hop U2NW Relay link security establishment, the Remote UE sends a DCR message as specified in the in clauses 6.3.3 of TS 33.503 [5] to the Intermeditate UE-to-Network Relay.

- The Intermediate UE-to-Network Relay plays the role of the Remote UE to establish secured link with the UE-to-Network Relay or with the next Intermediate UE-to-Network Relay, reusing the procedure as specified in the in clauses 6.3.3 of TS 33.503 [5], if the Intermediate UE-to-Network Relay does not have an existing PC5 link with the UE-to-Network Relay or the next Intermediate UE-to-Network Relay, and the next Intermediate UE-to-Network Relay is in-coverage.

NOTE 2: The next Intermediate UE-to-Network Relay are considered in-coverage if they have connection to the network.

NOTE 3: 	Whether a separate RSC for multi-hop or the same RSC as for single hop is provisioned is to be determined during normative phase in coordination with SA2.

3)	For authentication and authorization, the existing CP-based/UP-based procedures as specified in clause 6.3.3 of TS 33.503 [5] can be used as baseline for each hop during the normative work.

4)	For Remote UE report procedure, the existing procedures for Remote UE Report as specified in clause 6.3.3 of TS 33.503 [5] can be used as baseline for the normative work.

NOTE 4: How the Remote User ID of the Remote UE is obtained by the 5G ProSe UE-to-Network Relay for the Remote UE Report procedure is to be determined during normative phase in coordination with SA2.

NOTE 5: For the security procedure over User Plane, it is assumed that an Intermediate UE-to-Network Relay is able to access to the 5G PKMF in its HPLMN (i.e., PC8 establishment via Intermediate UE-to-Network Relay(s) or UE-to-Network Relay).

* * * End of Changes * * * *