3GPP TSG-SA3 Meeting #120	S3-250435

Athens, Greece, 17 - 21 February 2025

# Source:	Qualcomm Incorporated

# Title:	U2N relay discovery security

# Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

This contribution propose text for U2N relay discovery security.

# 2	References

[1]	TR 33.743

[2]	Skeleton for the draft CR

# 3	Rationale

This contribution proposes text for U2N relay discovery security based on the conclusions in the TR 33.743 [1].

# 4	Detailed proposal

It is proposed that the below changes to the draft CR [2] are approved.

# * * * * First change * * * *

#### 6.1.3.X	5G ProSe Multi-hop UE-to-Network Relay Discovery

##### 6.1.3.X.1	General

This clause describes the security requirements and the procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery defined in TS 23.304 [2].

##### 6.1.3.X.2	Security requirements for 5G ProSe Multi-hop UE-to-Network Relay Discovery

The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of discovery and communication messages in multi-hop UE-to-Network Relay discovery.

The 5G System shall provide a means for mitigating trackability and linkability attacks on UEs in multi-hop UE-to-Network Relay discovery.

The 5G system shall provide a means to securely provision the security materials for multi-hop UE-to-Network Relay discovery.

##### 6.1.3.X.3	Security flows for 5G ProSe Multi-hop UE-to-Network Relay Discovery

###### 6.1.3.X.3.1	Discovery with Model A

The security procedure for multi-hop UE-to-Network Relay discovery with Model A is shown in Figure 6. 1.3.X.3.1-1.

Figure 6.1.3.X.3. 1-1: Model A Discovery operation supporting multi-hop UE-to-Network Relay

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay is provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay’s HPLMN based on the procedure specified in clause 6.3. The discovery security materials shall contain a Discovery User Integrity Key (DUIK) for the integrity protection of Relay Discovery Announcement.

Editor’s Note: Discovery security materials provisioning for inter-PLMN scenario is FFS.

1.	The 5G ProSe UE-to-Network Relay shall protect a Relay Discovery Announcement using the discovery security materials associated with the RSC as specified in clause 6.3. Then, the 5G ProSe UE-to-Network Relay broadcasts the Relay Discovery Announcement.

2a.	The Intermediate UE-to-Network Relay shall process the received Relay Discovery Announcement message using the discovery security materials associated with the RSC as specified in clause 6.3. If the processing is successful and Intermediate UE-to-Network Relay does not have a PC5 link with the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall establish a PC5 link with the 5G ProSe UE-to-Network Relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication specified in clause 6.3.3.

2b.	Once the PC5 link is established between the Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall update the path information (e.g., hop count, Relay Info.) and protect the updated message using the discovery security materials associated with the RSC as specified in clause 6.3. The Intermediate UE-to-Network Relay broadcasts the updated message.

3.	Upon receiving the Relay Discovery Announcement message from the Intermediate UE-to-Network Relay, the 5G ProSe Remote UE shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.3.

###### 6.1.3.X.3.2	Discovery with Model B

The security procedure for multi-hop UE-to-Network Relay discovery with Model B is shown in Figure 6.1.3.X.3. 2-1.

Figure 6.1.3.X.3. 1-1: Model B Discovery operation supporting multi-hop UE-to-Network Relay

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay is provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay’s HPLMN based on the procedure specified in clause 6.3. The discovery security materials shall contain a Discovery User Integrity Key (DUIK) for the integrity protection of Relay Discovery Solicitation and Relay Discovery Response.

Editor’s Note: Discovery security materials provisioning for inter-PLMN scenario is FFS.

1.	The 5G ProSe Remote UE shall protect a Relay Discovery Solicitation using the discovery security materials associated with the RSC as specified in clause 6.3. Then, the 5G ProSe Remote UE broadcasts the Relay Discovery Solicitation.

2.	The Intermediate UE-to-Network Relay shall process the received Relay Discovery Solicitation using the discovery security materials associated with the RSC as specified in clause 6.3. If the processing is successful, the Intermediate UE-to-Network Relay shall update the path information (e.g., hop count) and protect the updated message using the discovery security materials associated with the RSC as specified in clause 6.3. Then, the Intermediate UE-to-Network Relay broadcasts the message.

3.	Upon receiving the Relay Discovery Solicitation from the Intermediate UE-to-Network Relay, the 5G ProSe UE-to-Network Relay shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.3. If the processing is successful, the 5G ProSe UE-to-Network Relay shall construct a Relay Discovery Response and protect it using the discovery security materials associated with the RSC as specified in clause 6.3.

The 5G ProSe UE-to-Network Relay replies to the Intermediate UE-to-Network Relay with the Relay Discovery Response.

4.	Upon receiving the Relay Discovery Response from the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.3. If the processing is successful, the Intermediate UE-to-Network Relay shall update the path information (e.g., hop count) and protect the updated message using the discovery security materials associated with the RSC as specified in clause 6.3. Then, the Intermediate UE-to-Network Relay replies to the 5G ProSe Remote UE with the message.

5.	Upon receiving the Relay Discovery Response from the Intermediate UE-to-Network Relay, the 5G ProSe Remote UE shall process the received message using the discovery security materials associated with the RSC as specified in clause 6.3.

# * * * * End of change * * * *