3GPP TSG-SA3 Meeting #120	S3-250667

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	NEF security requirements

Document for:	Approval

Agenda item:	4.24

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	AIML_CN_SEC

Comments

The proposal is to add clause X.12.3 to address privacy across VFL members during sample alignment for scenario when NWDAF act as a server.

* * * First Change * * * *

## X.12.3	NEF security requirements

### X.12.3.a VFL privacy during sample alignment (NWDAF as a Server)

Figure X.12.3.a-1 provide the means to enable privacy protection during sample alignment in case NWDAF act as VFL Server.

Figure X.12.3.a-1: Privacy of VFL participants during sample alignment (NWDAF Acts as VFL Server)

1.  VFL members perform profile registration with supporting VFL attributes as specified in clause 6.2H.2.1 of TS 23.288[105].

2. VFL Server perform discovery with VFL attributes as specified in 6.2H.2.1 of TS 23.288[105].

NOTE: Handling of temporary NWDAF instance ID (s) is left to NEF implementation.

3. The NWDAF as VFL Server send Nnef_VFLTraining_Request to NEF and the token includes the Analytics ID, VFL interoperability indicator per analytics ID and AF ID as described in 6.2H.2.2.1 of TS 23.288[105].

4. The NEF VFL Naf_VFLTraining request to VFL client(s) with AF ID, VFL Interoperability indicator and analytics ID.

5. The AF sends VFL Naf_VFLTraining response with (success or failure) via the NEF to the VFL server with the list of sample IDs.

6. The NEF forwards the Nnef_VFLTraining response with the sample ID(s) to the NWDAF acting as the VFL server.

7. The VFL server determines the final list of samples considering the samples that all selected VFL client’s support.

8. VFL members determine the overlapped samples ID(s).

Editor’s Note:	This clause provides description of NEF security requirements, and the related conclusion is made in clause 7.2 and clause 7.3 in TR 33.784.

* * * End of Changes * * * *