3GPP TSG-SA3 Meeting #120	S3-250766

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	Solution for inter-CU LTM and conclusion

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	NR_Mob_Ph4_Sec

Comments

Based on the RAN2 feedback (R2-2411136), this contribution proposes a new solution and an additional conclusion for key issue#1 for the case when CU is acting as MN and DC is not configured.

Based on the LS response received from RAN2 (R2-2411136), RAN2 prefers avoiding RRC configuration between cell switches for subsequent inter-CU LTM. Therefore, it is required for SA3 to revisit all the solution alternatives in the draftCR and decide on the procedures that aligns with RAN2 feedback.

### 2.1.2	Security threats

An attacker may manipulate the timing advance information, and cause the desynchronization between the UE and the BS. The Target Configuration ID may be tampered with, leading to connection failure between UE and the target cell. Furthermore, if the NCC, keySetChangeIndicator or the chosen algorithms of the target gNB are carried by the LTM Cell Switch Command MAC CE, they could be tampered, if the UE has accepted the altered NCC value, this may result in out-of-sync, key mismatch or security negotiation failure between the UE and target gNB.

In SA3#119 meeting following security threat was captured with regards to IEs sent in unprotected MAC CE (in Cell Switch Command message).

Observation 1: As the unprotected LTM command MAC CE already includes IEs such as Target Configuration ID and timing information, including NCC (three bits) in the unprotected MAC CE message does not introduce any new threats. Manipulating any parameter/value results in HO failure and UE can recover from the HO failure (for example, transiting to Idle and then to connected state).

Proposal 1: Based on the above observation and honouring the RAN requirement on avoiding RRC configuration between cell switches for subsequent inter-CU LTM, it is proposed to include NCC in the Cell Switch Command MAC CE (as in solution#3 and #4).

Observation 2:	 Most of the solutions (5 solutions) propose key distribution during the execution phase. Only 1 solution proposes key distribution during preparation phase and 2 solutions propose during both preparation phase and execution phase.

Observation 3: Further among the key distribution during the execution phase, 4 solutions perform key update procedure to all candidate CUs, whereas 3 solutions provide the key only to the selected target. Updating the key for every LTM to all candidate CUs is overhead and unnecessarily increases the Xn signalling. Further it might not be a preferred approach to perform new signalling, between the source and the candidate cells, between cell switches for subsequent inter-CU LTM. Further it introduces additional delay to update all the candidates before initiating an LTM procedure. Therefore, providing the key only to the target CU during execution phase is more efficient and effective.

Observation 4: Baseline procedure to TS 38.300 for support of inter-gNB LTM is specified in R3-247031. As specified in R3-247031, CELL SWITCH NOTIFICATION which is sent by the source gNB to the target gNB in execution phase is a mandatory message to indicate the initiation of Cell Switch command to the UE.

Proposal 2: Key distribution between source and target gNBs (CUs) is performed during LTM execution phase and key is provided only to the target CU using the Cell switch notification message.

The corresponding pCR introduces normative text for security procedure for inter-gNB LTM as specified in S3-250767.

* * * First Change * * * *

Annex α (Informative): 
Security mechanisms for Inter-CU LTM

Editor’s Notes: This clause contains the key issues and security solutions considered for analyzing the security mechanism and procedure for inter-CU LTM based on work progress in RAN WGs.

# 1	References

[1]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP LS S3‑241773: "LS on security handling for inter-CU LTM in non-DC cases	(R2-2404037)".

[4]	3GPP TS 38.423:	"Xn Application Protocol".

[xx]	R3-247932: "(BL CR to 38.300) Support for Inter-CU LTM".

* * * Next Changes * * *

## 3.Y	Solution #Y: Security procedure for inter-gNB LTM (MAC CE based)

### 3.Y.1	Introduction

This solution proposes the key handling mechanism for inter-CU LTM case when the CU is acting as MN and DC is not configured. As per the LS response in R2-2411136, the following principles are followed combining existing solutions in the draftCR:

- 	Key distribution between source and target gNBs (CUs) is performed during LTM execution phase.

-	Key is provided only to the target CU during execution phase using the Cell switch notification message.

-	MAC CE in Cell Switch Command to be used to provide the NCC value to the UE between subsequent LTM mobility procedures for security key handling.

### 3.Y.2	Solution details

Figure 3.Y.2-1:  Inter-CU LTM procedure when CU is acting as MN

The procedure for inter-gNB LTM is as follows (c.f., R3-247932 (BL CR to 38.300) Support for Inter-CU LTM [xx]):

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3-11.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate. Step 3-11 is followed as specified in clause 9.2.3.5.X in TS 38.300 [1].

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s)

13.	The source gNB decides to execute cell switch to a target. The source gNB generates the KNG-RAN* from target PCI, its frequency ARFCN-DL/EARFCN-DL, and either from currently active KgNB or from the NH as described in Annex A.11 or A.12 in TS 33.501 [2].

14.	The source gNB sends the Cell Switch command to the UE via MAC CE. The MAC CE includes the NCC used for the derivation of KNG-RAN*.

Upon receiving the cell switch command, if the NCC value associated with the current KgNB is different from the received NCC value, then the UE synchronize the locally kept NH parameter by computing the function defined in Annex A.10 iteratively as specified in TS 33.501 [2] and computes the KNG-RAN* from the synchronized NH parameter. If the NCC is equal to the NCC value associated with the current active KgNB, then the UE uses the active KgNB to derive the KNG-RAN* using the function defined in Annex A.11 of TS 33.501 [2].

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE. In this notification message the source gNB includes the generated {KNG-RAN*, NCC} pair to the target gNB.

NOTE :	Steps 14 and 15 are performed in parallel, so that the target gNB gets the {KNG-RAN*, NCC} before step 19 in figure 3.Y.2-1.

16. Further, the target gNB detects the UE access and follows the rest of the procedure as specified in TS 38.300 [1].

### 3.Y.3	Evaluation

This solution addresses the security requirement of key issue#1. Solution supports both vertical and horizontal key derivation for LTM cell switch procedure, based on the NCC value received in the MAC CE. Further this solution supports subsequent LTM, as the source provides relevant security parameters. The solution aligns with inter-gNB LTM specified in TS 38.300 [1].

* * * Next Changes * * * *

# 5	Conclusions

## 5.1	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	Solution #Y is considered as the baseline for the normative work.

- Key distribution between source and target gNBs (CUs) is performed during LTM execution phase and Key is provided only to the target CU during execution phase using the Cell switch notification message.

- MAC CE in Cell Switch Command to be used to provide the updated NCC to the UE by the source gNB between subsequent LTM mobility procedures for security key handling.

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

- If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

- It initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

- It updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

* * * End of Changes * * * *