3GPP TSG-SA3 Meeting #120	S3-250788

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	pCR on Security for S&F operation on split MME architecture

Document for:	Approval

Agenda item:	4.19

Spec:	3GPP TS 33.401

Version:	18.2.0

Work Item:	5GSAT_Ph3_SEC

Comments

This pCR proposes to security procedure for S&F operation on split MME architecture.

* * * First Change * * * *

Annex X (normative):
Security for Store and Forward Satellite operation

# X.1	General

Editor’s Note: This clause provides general description for Annex X.

# X.2	Security aspects on Split MME architecture

## X.2.1	General

This clause describes the security aspects on Split MME architecture in S&F operation. MME is split into two functions, i.e., MME-onboard and MME-ground. MME-onboard is in charge of handling NAS protocol signalling from/to UEs via the onboard eNB, so the NAS security is terminated on the MME-onboard. The MME-onboard obtains the EPS authentication vectors when the feeder link is available, and stored the authentication vectors when the service link is unavailable. The security procedures specified in clause 6 are used for AKA, NAS security mode command etc.

Enhancements to provide protection against DoS attacks before a security context is established are described in clause X.2.2. For the enhancements, the network provisions UEs and satellites with a set of credentials for Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI) [xx]. The credentials include public validation token (PVT), secret signing key (SSK) associated with UE ID or satellite ID, and KMS public authentication key (KPAK). UE ID is the UE's identity associated with ECCSI algorithm, and satellite ID is the satellite's identity associated with ECCSI algorithm.

## X.2.2	Procedure

Figure X.2.2-1: Protected attach procedure with split MME onboard the satellite in S&F operation

PHASE 1. (Service link is available, Feeder link is unavailable)

1. The satellite provides a random number generated by the satellite (RANDSAT) and S&F indicator indicating that the satellite is operating in S&F mode.

2. The UE initiates the attach procedure by transmitting signed Attach Request message. The Attach Request message includes UE ID, RANDUE, RANDSAT, S&F indicator, and digital signature. The digital signature is generated by the UE using the Attach Request message and RANDSAT through ECCSI. S&F indicator indicates that the UE supports S&F operation. The satellite checks the validity of the UE by verifying the signature.

3. If the verification is successful, the satellite stores the received attach request message and transmits signed Attach Reject message. The Attach Reject message includes satellite ID, S&F wait timer, S&F monitoring list, and a digital signature. The digital signature is generated by the satellite using the Attach Reject message and RANDUE through ECCSI algorithm. The UE checks the validity of the satellite by verifying the signature. If verified, UE waits for step 7 based on S&F wait timer or S&F monitoring list.

PHASE 2. (Service link is unavailable, Feeder link is available)

4-6. Procedures as specified in clause 6.1.2 are applied.

PHASE 3. (Service link is available, Feeder link is unavailable)

7. The UE retries the network connection by transmitting the Attach Request. This message can be protected using the similar method to step 2.

8-11. Procedures as specified in clause 6.1.1 are applied. As a result of the authentication and key agreement, an intermediate key KASME is shared between the UE and the satellite.

12. NAS SMC procedure is performed as specified in clause 7.2.4.4.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 33.102: "3G security; Security architecture".

[5]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[9]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[10] – [11]	Void.

[12]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification"

[13]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[14]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197) "

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18] – [20]	Void.

[21]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[23]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Service principles".

[24]	3GPP TS 25.331: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Resource Control (RRC); Protocol Specification ".

[25]	3GPP TS 44.060: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol.

[26]	3GPP TS 23.122: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	3GPP TS 33.320: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[28]	(void)

[29]	ETSI TS  102 484 V10.0.0: "Smart Cards; Secure channel between a UICC and an end-point terminal".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 31.116 "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications".

[32]	ETSI TS 102 221 V9.2.0: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[33]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[34]	RFC 4301: "Security Architecture for the Internet Protocol".

[35]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[36]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[37]	3GPP TS.33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[38]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[39]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[40]	3GPP TS 36.463: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[41]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[42]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[43]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[44]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[45]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[xx]	IETF RFC 4303: " Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CHO	Conditional Handover

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECCSI	Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption

ECM	EPS Connection Management

EDT	Early Data Transmission

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

gNB	Next Generation Node-B

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IAB	Integrated Access and Backhaul

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KMS	Key Management Service

KPAK	KMS Public Authentication Key

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

PVT	Public Validation Token

RAND	RANDom number

RAU	Routing Area Update

RLOS	Restricted Local Operator Services

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SgNB	Secondary gNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

SSK	Secret Signing Key

S-TMSI	S-Temporary Mobile Subscriber Identity

S&F	Store and Forward

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

* * * End of Changes * * * *