3GPP TSG-SA3 Meeting #120	S3-250833

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi Technology Spain S.L

Title:	Security aspects on Split MME architecture

Document for:	Approval

Agenda item:	4.19

Spec:	3GPP TS 33.401

Version:	18.2.0

Work Item:	5GSAT_Ph3_SEC

Comments

Based on the conclusion in TR 33.700-29, it is proposed to define the security aspects on Split MME architecture in TS 33.401.

* * * First Change * * * *

X.2	Security aspects on Split MME architecture

The security aspects on Split MME architecture have below principles:

- The NAS security is terminated on the MME-onboard. The latest NAS security context of the UE is available at the MME-onboard which processes the NAS security.

- The security procedure for Store and Forward Satellite operation is as follows:

Step A:	When the service link is available, the UE sends the Attach Request, including IMSI.

Step B:	The MME-onboard temporarily stores the Attach request and may return the Attach Reject to the UE. The Attach Reject may include the S&F Wait Timer and the S&F Monitoring List.

Step C:	When the feeder link is available, the MME-onboard forwards the stored Attach Request to the MME-ground. After checking the UE S&F authorization, the same or another MME-onboard obtains the EPS authentication vectors, and stores the EPS authentication vectors when the service link is unavailable.

Step D:	When the service link is available, the UE re-sends the Attach Request.

Step E:	The legacy security procedures (e.g. AKA, NAS security mode command, etc.) specified in the present document are reused to establish the security between UE and MME-onboard.

NOTE 1:	The exchanged NAS messages before the NAS security context is established are unprotected, i.e. Attach Request and Attach Reject are unprotected.

NOTE 2:	The distribution and synchronization of NAS security context between the MME-ground and MME-onboard is out of the scope of 3GPP.

- The security of communications between the proxies on satellite and ground is out of 3GPP scope.

* * * End of Changes * * * *