3GPP TSG-SA3 Meeting #120	S3-250842

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi Technology Spain S.L

Title:	Security procedure for 5G ProSe Multi-hop UE-to-Network Relay communication

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Ph3_Sec

Comments

Based on the conclusion in TR 33.743, it is proposed to define the security procedure for multi-hop UE-to-Network Relay communication in TS 33.503.

* * * First Change * * * *

6.3.Z.2	Security procedure for 5G ProSe Multi-hop UE-to-Network Relay communication

6.3.Z.2.1	5G ProSe Multi-hop UE-to-Network Relay communication following Model A discovery

Based on RSC configuration, the Intermediate UE-to-Network Relay determines whether it should establish a secure PC5 link with its parent relay (incl. parent Intermediate UE-to-Network Relay or 5G ProSe UE-to-Network Relay) during the Multi-hop UE-to-Network Model A discovery.

If the secure PC5 link(s) between the Intermediate UE-to-Network Relay and its parent relay is established during the Multi-hop UE-to-Network Model A discovery, the 5G ProSe Remote UE initiates the User Plane (UP) based procedure as specified in clause 6.3.3.2 or the Control Plane (CP) based procedure as specified in clause 6.3.3.3 with the Intermediate UE-to-Network Relay.

Otherwise, the User Plane (UP) based procedures as specified in clause 6.3.3.2 and the Control Plane (CP) based procedures as specified in clause 6.3.3.3 are used to provide authentication, authorisation and security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay.

* * * End of Changes * * * *