3GPP TSG-SA3 Meeting #120	S3-250849

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi

Title:	Pseudo-CR on 33.721: CAPIF-based Solution for Key Issue #4

Document for:	Approval

Agenda item:	5.18

Spec:	3GPP TR 33.721

Version:	0.6.0

Work Item:	FS_Metaverse_Sec

Comments

This pCR proposes a new solution based on CAPIF framework to address the requirements in key issue #4 in TR 33.721.

* * * First Change * * * *

6.Y	Solution #Y: Avatar authentication and user authorization supported by CAPIF

6.Y.1	Introduction

This solution addresses KI #4 on authentication and authorization of digital representation. It is based on solution #7 while extending it by using CAPIF framework.

In DA service architecture defined in TS 23.438 [8] clause 6.2, the DA server is a type of SEAL server. VAL server can be a metaverse application server. When CAPIF is used to support metaverse enablement services, it serves as the framework for north bound API of the DA service provided by the DA server. In this case, the DA server can be mapped to the AEF and the API invoker in VAL server be mapped to the API invoker in CAPIF functional model.

The service request message sent from the VAL client in the UE to the VAL server (i.e. metaverse application server) is assumed to include the avatar used in the application and the information of user represented by the avatar. For authenticating the avatar and authorizing the user, the API invoker in the metaverse application server invokes the DA service API for retrieving DA profile from the AEF (i.e. DA server). Based on validation of avatar authenticity and the retrieved DA profile containing the association between avatar and user, the metaverse application server is able to determine whether the avatar can represent the user in the application and whether the user is allowed to use the avatar in the application.

6.Y.2	Solution details

1.	The VAL client in the UE sends a metaverse service request to the metaverse application server. The request message contains at least the client ID, avatar ID and corresponding avatar (media), user ID, app ID.

As the avatar is created and signed by the DA server, the metaverse application server first verifies the authenticity of the avatar by verifying the signature of the avatar using the certificate of the DA server if locally available. Only when the avatar is verified as authentic, the metaverse application server will then proceed with the following steps. Otherwise, the metaverse application server returns a failure response to the UE.

Figure 6.Y.2-1: Avatar authentication and user authorization by metaverse application server

2.	The API invoker in the metaverse application server sends a DA service API invocation request to the DA server, including at least invoker ID, identifier of DA service API to be invoked (i.e. SS_DAProfileManagement_Retrieve in TS 23.438 [8]), avatar ID and optionally app ID. If the metaverse application server is not able to verify the authenticity of the avatar (e.g. no DA server certificate), it also sends the received avatar to the DA server for verification.

3.	If avatar is included in the request, the DA server first verifies the authenticity of the avatar, and then checks the app ID if received. Only when the avatar is verified as authentic and allowed to be used in the application, the DA server will then accept the DA service API invocation request. Otherwise, the DA server returns a failure response to the metaverse application server. If needed, the DA server may obtain more authorization information from the CCF before responding.

4.	The DA server sends the DA service API invocation response to the metaverse application server, which contains either the requested DA profile or a failure cause.

5.	If the requested DA profile is received, it implies that the authenticity of the avatar is successfully verified, which means that avatar is implicitly authenticated. The metaverse application server then determines whether the authentic avatar can be used to represent the user or whether the user is allowed to use the avatar, by checking the association between avatar and user in the DA profile.

6.	The VAL client in the UE receives the metaverse service response, which contains either the indication of success/failure or the requested service result.

6.Y.3	Evaluation

This solution addresses KI #4 on authentication and authorization of digital representation by applying the CAPIF framework.

From the perspective of the avatar, the combination of successful verification of avatar authenticity and identified association between the avatar and user can ensure that the avatar is authenticated to represent the user in a mobile metaverse application. From the perspective of the user, this combination also ensures that the user is authorized to use the avatar in a mobile metaverse application. Hence, both requirements in key issue #4 are fulfilled.

This solution reuses the procedures in CAPIF without impact on CAPIF or DA service architecture. The DA server or the metaverse application server is required to verify the authenticity of the avatar.

* * * End of Change * * * *