3GPP TSG-SA3 Meeting #120	S3-250868

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi

Title:	Pseudo-CR on Authorization of candidate VFL participants for vertical federated learning

Document for:	Approval

Agenda item:	4.24

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	AIML_CN_SEC

Comments

This contribution proposes to add the authorization procedure of candidate VFL participants for vertical federated learning.

.

Start of Change

X.12.2.1	Authorization of candidate VFL participants for vertical federated learning when NWDAF is acting as the VFL server

Figure X.12.2.1-1:Authorization of candidate VFL participants for vertical federated learning when NWDAF is acting as the VFL server

Steps 1 to 6 are identical to step 1 to step 6 defined in clause 6.2H.2.1.1 of TS 23.288 [105].

The VFL authorization procedure is given as follows.

7.The NWDAF sends the token request to the NRF. The token request includes the NEF identifier , the AF identifier(s), the expected VFL service, the Analytics ID, and the NWDAF identifier.

8.If the discovered VFL client is the external AF, the NRF authorizes the request based on the NEF's NF profile and local policy.  The NF profile of NEF includes the AF identifier and analytic ID supported by the AF.

If the discovered VFL client is the NWDAF/trusted AF, the NRF authorizes the request based on the NWDAF’s/AF's NF profile and local policy.

9. If the NWDAF is authorized to request the service, the NRF sends the token to the NWDAF.

If the discovered VFL client is the external AF, the token includes the allowed service, the NEF identifier, the Analytics ID supported by the AF, the AF identifier, and NWDAF identifier. The audience claim of the token is NEF identifier.

If the discovered VFL client is the NWDAF/trusted AF, the audience claim of the token is NWDAF identifier/AF identifier.

VFL service request/response procedure for the VFL client external AF (option 1)

10. NWDAF sends the VFL Service Request to the NEF. The service request includes the Analytics ID, the AF identifier, the NWDAF identifier, and the token.

11.The NEF verifies the token and check the VFL service request against the verified token.

If allowed service, the Analytics ID supported by the AF, and AF identifier are present in the token, the NEF checks if these information matches the VFL Service Request message. If all the checks are successful, the token is validated. If any of the checks fails, the authorization procedure should be terminated.

12.If the information in the request is allowed by the token, the NEF forwards the request to the AF. Otherwise, the NEF rejects the request.

13-14. The AF sends the VFL service response to the NWDAF via the NEF.

VFL service request/response for the VFL client NWDAF (option 2)

10-11. The VFL NWDAF server sends the VFL Service Request to the VFL client NWDAF, the VFL client NWDAF authorizes the VFL service request as defined in clause 13.3

VFL service request/response for the VFL client trusted AF (option 3)

10-11. The VFL NWDAF server sends the VFL Service Request to the trusted AF, the trusted AF authorizes the VFL Service Request as defined in clause 13.3

X.12.2.2	Authorization of candidate VFL participants for vertical federated learning when external AF is acting as the VFL server

FigureX.12.2.2-1	Authorization of candidate VFL participants for vertical federated learning when external AF is acting as the VFL server

Steps 1 to 10 are identical to step 1 to step 10 defined in clause 6.2H.2.1.2 of TS 23.288 [105].

11.The AF sends the VFL Service Request to the NEF. The request includes the Analytics ID supported by the NWDAF, AF identifier, external NWDAF identifier.

12.The NEF sends the token request to the NRF. The token request includes the expected VFL service, the AF identifier and the NWDAF identifier.

13.The NRF authorizes the request based on the NWDAF's NF profile and local policy.

14.If the AF is authorized to request the service, the NRF sends the token to the NEF. The token includes the allowed service, the AF identifier, NEF identifier, the Analytics ID supported by the NWDAF, and NWDAF identifier.

15.The NEF sends the VFL service request to the target NWDAF. The request includes the Analytics ID, the AF identifier and the token.

16.The NWDAF verifies the token as described in clause 13.4.1.1.

If the allowed service, the Analytics ID supported by the NWDAF, and AF identifier are present in the token, the NEF checks if these information matches the VFL service request message. If all the checks are successful, the token is validated. If any of the checks fails, the authorization procedure should be terminated.

17-18. If the information in the request is allowed by successful validation of the token, the NWDAF sends VFL service response to the AF via the NEF. Otherwise, the NWDAF rejects the request.

End of Change