3GPP TSG-SA3 Meeting #120	S3-250950

# Athens, Greece, 16-21 February 2025

Source:	Nokia

Title:	NRF cert retrieval operation

Document for:	Approval

Agenda item:	4.11

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	TEI19, SBA

Comments

pCR against DRAFT CR (was S3-245334) for resolution of

Editor’s Note: It is FFS, if AccessToken service is the correct service for adding a management operation.

It is proposed to use the Nnrf_AccessToken service for the new operation.

Editor’s Note: It is FFS, which fields the OAuth 2.0 token should include, e.g. URIs or key ids, to allow for identification or location of the public key/cert of the access token issuer. To be considered, certificates may be managed by a different entity than the NRF.

It is proposed to add related details.

Proposed Changes

*** BEGIN OF CHANGES 4 ***

### 14.3.1	General

The following table illustrates the security related services for OAuth 2.0 that NRF provides. OAuth 2.0 based authorization is described in clause 13.4.1.

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnrf_AccessToken | Get | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF |
|  | RetrieveCertificate | Request/Response | AMF, SMF, PCF, NEF, NSSF, SMSF, AUSF, NRF |

The complete list of NRF services is defined in TS 23.501 [2], clause 7.2.6, and further refined in TS 23.502 [8], clause 5.2.7.

The service operation shall allow for fetching more than one certificate.

Certificates can be managed by a different entity than the NRF. The OAuth 2.0 token may include URIs or key ids, to allow for identification or location of the public key/cert of the access token issuer.

Editor’s Note: it is FFS, whether a NF Service Producer has only one certificate/public key available, which can result in a failure and delay for fetching a new cert/pub key or if NF Service Producers should have at least two certificates of potential access token issuers. Also to support different NF services, the NRF may have several certificates (eg slice specific).

Editor’s Note: It is FFS if a NF Service Producer should be able to subscribe for updates of NRF cert/pub keys, request status of a stored cert/pub key.

*** END OF CHANGES 4 ***

*** BEGIN OF CHANGES 5 ***

### 14.3.X	Nnrf_AccessToken_RetrieveCertificate Service Operation

Service Operation name: Nnrf_AccessToken_RetrieveCertificate.

Description: NF Service Consumer requests NRF to provide public key to validate the signature of the Access Token.

Inputs, Required: the NF Instance Id of the token issuer NRF, the key identifier (Key ID from the Header Parameter of the JOSE header of the access token), reason for request, certificate identifier.

Inputs, Optional: UUID of NF Service Producer issuing the request (since certain certificates may be applicable only for specific NF Service Producers).

Outputs, Required: X.509 public key certificate(s) or certificate chain(s) used for signing the access token.

Outputs, Optional: None.

Editor’s Note: An attacker may pretend to be an NRF and has issued an access token signed with its private key, not known to NF Service Producer, i.e. not previously configured at NFp and not created from the root cert. How to handle such situation at NF Service Provider is FFS.

*** END OF CHANGES 5 ***