3GPP TSG-SA3 Meeting #120	S3-251060

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon, Nokia

Title:	Authorization for selection of VFL participants in VFL Group (NWDAF act as VFL Server)

Document for:	Approval

Agenda item:	4.24

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	AIML_CN_SEC

Comments

The proposal aims to add authorization handling for VFL members while participating in the same VFL process in clause X.12. The authorization for selecting VFL members i.e NWDAF/AF in the same VFL process utilises NRF token-based authorization as specified in clause 13.4.1 for the participating in VFL procedure.

This change is covered in X.12.2.1 when NWDAF act as VFL server. Hence it is proposed to add the authorization aspect for VFL members in this change.

* * * First Change * * * *

#### X.12.2.x Authorization of candidate VFL participants for VFL when NWDAF is acting as the VFL server

Figure X.12.2.x -1 depicts the authorization mechanism for NWDAF as VFL Server. The authorization is based upon the VFL capability information per supported Analytics ID which includes VFL capability type (VFL server and/or VFL client), Analytics ID and VFL Interoperability Indicator per Analytics ID provided by VFL members during registration.

Figure X.12.2.x-1: Authorization of VFL participants (NWDAF Acts as VFL Server)

1. In case of AF as VFL Client, the NEF update its NF profile at the NRF with AF’s VFL capability information per supported analytics ID which includes VFL Capability type i.e. (VFL Client) and VFL interoperability indicator(s) per analytics ID and associated AF ID. NWDAF as VFL Client and NWDAF as VFL Server register its profile as described in clause 5.2 or 6.2H.2.1.1 of TS 23.288[105].

2. The NWDAF as VFL Server discovers VFL Client(s) from NRF by invoking the NFDiscovery_Request service operation with VFL capability information per supported Analytics ID that includes VFL capability type (i.e. VFL client), VFL interoperability indicator per analytics ID, Analytics ID(s) as specified in clause 6.2H.2.1.1 of TS 23.288[105].

3. The NWDAF as VFL Server sends token request(s) to NRF as specified in clause 13.4.1. The token request(s) includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID.

4. The NRF checks whether the VFL server NWDAF is authorized to access the NEF and NWDAF (VFL Client) for VFL service(s) by optionally verifying the VFL interoperability indicator in the token request is in the VFL interoperability indicator(s) as received from VFL server and VFL client in Step 1. If the server NWDAF is authorized NRF will issue access token(s) as specified in clause 13.4.1. The token claim may include VFL interoperability indicator for that particular analytics ID, Analytics ID and AF ID.

5. The NRF sends the access token(s) to the NWDAF acting as VFL Server, or rejects the request in case of failed authorization, as described in clause 13.4.1.

6. The NWDAF as VFL Server may send VFLService_Request to NEF and the token received in Step 5. The service request includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID .

7. The NEF verify the access token received from the NWDAF. The NEF extract the Analytics ID from the access token and check whether it matches the Analytics ID received in step 6. If the AF ID is included in the token, the NEF also verify that the AF ID matches the AF ID requested in step 6.

8. The NEF sends the VFLService_Request to the AF (VFL as client) that includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID. The AF performs authorization of VFL service request based on its local policy and implementation. Protection of NEF-AF interface is as specified in clause 12 of TS 33.501.

9a. The AF sends VFL VFLService response (success or failure) to the NEF.

9b. The NEF forwards the VFLService response to the NWDAF acting as the VFL server.

Editor’s Note: The addition of Internal AF as a server to be covered in the present clause is FFS.

* * * End of Changes * * * *