3GPP TSG-SA3 Meeting #120	S3-251085

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon, Ericsson

Title:	Adding token based mechanism to draftCR of MEC ph3

Document for:	Approval

Agenda item:	4.17

Spec:	3GPP TS 33.558

Version:	18.2.0

Work Item:	EDGE_Ph3

Comments

This contribution proposes to add token based mechanism on the ID verification concluded in the clause 7.2 of TR 33.749.

This document is the merger of S3-250882 and S3-250591.

* * * First Change * * * *

## X.2	UE ID Token based mechanism

### X.2.1	Overview

According to the mechanism, a new entity called UE ID Server located inside the operator domain provides a separate UE ID Token to the Client running on the UE for each request coming from the Client. The Client interacts with the AF by using the UE ID Token and then the AF interacts with network by using the received UE ID Token. The network resolves the UE ID Token by interacting with the UE ID Server to identify the UE. The UE ID Server and the interactions with the UE ID Server can be realized by implementation specific means.

### X.2.2	Details

Generic representation of the mechanism is presented in Figure X.2.2-1.

Figure X.2.2-1: Generic view of UE ID token based mechanism

Step 1: The client obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. The UE ID Server can fetch the IP address from the IP packets received from the Client. The UE ID Server as a trusted AF can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE. For the UE ID Server service consumption by the client, the UE where the client is running on needs to execute the primary authentication successfully and have a PDU session. The client can authenticate the UE ID Server by using server certificate. The Client needs to be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used. The client does not need to authenticate with the server since the server issues the UE ID token, which does not include cleartext sensitive information, to the client running on the UE who has already executed the primary authentication.

NOTE 1:	Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

Step 2: The client forwards the token to the AF.

Step 3: The AF invokes a NEF service and use the UE ID token as UE identifier.

NOTE 2:	The UE ID token can be provided as input in any NEF Service that takes GPSI as input (as NAI/External Id format).

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Note that if the AF is a trusted AF, it can directly interact with the UE ID Server to identify the UE and the obtain required information such as IP address, and then invoke the NF/NEF directly with the required inputs.

Application of the mechanism to the case of NEF API invocation by the EAS/EES is presented in Figure 6.9.2-2.

Figure X.2.2-2: The EAS/EES invokes NEF API

Step 1: The AC/EEC obtains a UE ID token from the UE ID server.

Step 2: The AC or the EEC respectively forward the token to the EAS or the EES.

Step 3: The EAS/EES invokes a NEF service and use the UE ID token as UE identifier.

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Application of the mechanism to the case of EES API invocation by the EAS is presented in Figure 6.9.2-3.

NOTE 3:	The invocation to existing APIs in step 3 and 4 by this mechanism in the following figure is left to implementation.

Figure X.2.2-3: The EAS invokes the EES API

Step 1: The AC obtains a UE ID token from the UE ID server.

Step 2: The AC forwards the token to the EAS.

Step 3: The EAS invokes a EES service and use the UE ID token as UE identifier.

Step 4: The EES invokes the NEF API by sending the UE ID token.

Step 5: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 6: The UE ID server returns the requested information.

Step 7: The NEF sends the UE ID to the EES.

Step 8: The EES returns the result of the EES service execution.