3GPP TSG-SA3 Meeting #120	S3-251086

# Athens, Greece, 17 - 21 February 2025															was_S3-250883

Source:	Huawei, HiSilicon

Title:	Adding hash based mechanism draftCR of MEC ph3

Document for:	Approval

Agenda item:	4.17

Spec:	3GPP TS 33.558

Version:	18.2.0

Work Item:	EDGE_Ph3

Comments

This contribution proposes to add the hash based mechanism on the ID verification concluded in the clause 7.2 of TR 33.749.

*** Start of The First Change ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	Void

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications."

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

[8]	Void

[9]	Void

[10]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[11]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[12]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[13]	Void

[14]	Void

[15]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[16]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[17]	IETF RFC 7519: "JSON Web Token (JWT)".

[18]	IETF RFC 7515: "JSON Web Signature (JWS)".

[19]	IETF RFC 9113: "HTTP/2".

[20]	IETF RFC 9110: "HTTP Semantics".

[xx]	3GPP TS 23.502: "Procedures for the 5G System"

*** End of The First Change ***

*** Start of The Second Change ***

Z.x	Hash based mechanism

Z.x.1	Introduction

This mechanism proposes to use fresh hash value with count, which is a fresh value shared between the UE and the 5GC, to authorize the AF to retrieve the UE ID.

Z.x.2	Details

This mechanism supports the 5GC to verify whether the AF is authorized by the UE to retrieve the UE ID based on the IP address, and verify whether the parameters used for verification is replayed by the adversary. Specifically, when the UE receives the UE ID request, it generates a hash value based on the count, salt, SUPI and IP address, and then sends the generated hash value and count to the AF, which uses the IP address to request for the UE ID from the NEF. The NEF then requests BSF to provide information of the UE (i.e., SUPI). Before providing the SUPI of UE, the BSF shall also generate a hash' based on the shared count, salt, SUPI and IP address, and then compare the hash' value with the received hash value. If the hash value is equal to the hash' value, the BSF can provide the SUPI to the NEF. Otherwise, the BSF will provide a failure indication to the NEF.

Figure Z.2.2-1: Authorization of AF and verification of AF provided IP address

0.	During the PDU session establishment procedure, SMF generates the salt, which is a random value, and then sends the salt to both UE and BSF. SMF can send the salt to the BSF along with SM policy Association Establishment or SM policy Association Modification message in clauses 4.16.4 or 4.16.5 of TS 23.502 [xx]. If the SM policy Association Establishment and SM policy Association Modification procedures are not performed, SMF will not send the salt to the UE.

1.	UE receives the UE ID request.

2.	The UE generates a hash value based on the SUPI, private IP address, salt, and count, and other values that is only known to both the EEC and BSF (e.g., DNN of the PDU Session, and S-NSSAI of the PDU Session). After that, the UE increases the count by 1, which is used for the next hash value generation procedure. Note that the count can be configured to start from 0 in each PDU session. In order to mitigate the rainbow table attack, the following mechanisms should be adopted:

(1) using strong hash algorithms, e.g., SHA-256.

(2) increasing the length of the hash algorithm's input.

The length of the salt can be set as 128bits to make the rainbow table attack infeasible [15].

The input space of the SUPI is large enough to mitigate the rainbow table attack, considering that the length of IMSI is 15-16 digits, and NAI is 1-759 bits, as specified in TS 23.003 [16].

Other inputs, including DNN of the PDU Session, and S-NSSAI of the PDU Session, can also help defend against the rainbow table attack.

(3) key stretching. The inputs can be run through the hash function multiple times to increase the time to build a precomputed rainbow table.

The UE sends the hash value and the count together to avoid the out-of-synchronization between the UE and BSF.

3.	The UE sends the generated hash value and count to the AF.

4.	The AF invokes the Nnef_UEId_Get request with the parameter of hash value, count, and public IP address to get the UE ID.

5-8.	This mechanism reuses Steps 3-6 in Clause 4.15.10, TS 23.502 [8].

9.	When the NEF invokes the Nbsf_Management_Discovery service operation, the NEF provides the hash value and private IP address to the BSF.

10.	The BSF determines the SUPI, salt and count based on the private IP address. The BSF then generates the hash' value, and then compares the hash' value with the received hash value.

11.	If the hash' value is equal to the received hash value, the BSF will respond to the NEF with a SUPI. Otherwise, the BSF will respond to the NEF with an indication that request is denied with the reason that the private IP address is not associated with the UE.

12-14.	This mechanism reuse Steps 9-11 in Clause 4.15.10, TS 23.502 [xx].

*** End of The Second Change ***