3GPP TSG-SA3 Meeting #120	S3-251091

Athens, GR, 17 -21 February 2025

Source:	Google, AT&T, TELUS

Title:	pCR to ACME-SBA living document on: certificate enrollment and renewal in 5G SBA using ACME

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.0

Work Item:	ACME_SBA

Comments

This document proposes to add the procedures for certificate enrolment and renewal in 5GC SBA using ACME.

Proposed Changes

* * * First Change * * * *

## YY.3	Certificate enrolment and renewal

### YY.3.1	Introduction

This clause describes the ACME certificate enrolment and renewal procedures for the 5GC SBA. The procedures are based on the ACME protocols specified in RFC 8555 [XX]. The steps for certificate enrolment and renewal procedures have been grouped as follows: Certificate issuance (YY.3.2) for order submission, and Challenge validation (YY.3.3) for proving control of identifiers in the certificate, CSR, certificate download.

### YY.3.2	Certificate issuance

Certificate issuance as defined in RFC 8555 [XX] is used for certificate enrolment and renewal.

Pre-authorisation is not used in ACME for 5G SBA, therefore:

- The newAuthz resource is not supported.

- The newOrder resource is used for all enrolment and renewal.

Clause 6.1.3c defines the certificate profiles for 5GC SBA. X.509 version 3 certificates are used for all entities in 5GC SBA. ACME supports X.509 version 3 certificates and the necessary extensions.

The policy-based certificate renewal can be used as described in the Annex I.2 of the present document.

Editor’s note: Additional recommendations to support ACME certificate enrolment and renewal are FFS.

* * * Second Change * * * *

2	References

[XX]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

* * * End of Changes * * * *