3GPP TSG-SA3 Meeting #120	S3-250360

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Proposed new KI on the MAC CE security

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

* * * First Change * * * *

# 2	Key issues

Editor’s Note: This clause contains all the key issues identified for the WID NRmobenh_sec_Ph1.

## 2.1	Key Issue #1: Security aspects of inter-CU LTM Handover

### 2.1.1	Key issue details

Layer1/Layer 2 Triggered Mobility (LTM) is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE serving cell by a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command.

Currently in Rel-18, LTM operation is only supported for mobility between cells within a gNB i.e., both intra-gNB-DU and intra-gNB-CU inter-gNB-DU (same CU) mobility [1]. In release-19, it is planned to enable it for between cells of different gNBs (inter-CU).

As specified in TS 38.321[x], the LTM cell switch command MAC CE includes parameters such as the Target Configuration ID, Timing Advance Command, TCI state ID, etc. To support the inter-CU LTM, the LTM cell switch command MAC CE may also include parameters such as NCC, Indication of chosen algorithm of target cell, and keySetChangeIndicator.

### 2.1.2	Security threats

An attacker may manipulate the timing advance information, and cause the desynchronization between the UE and the BS. The Target Configuration ID may be tampered with, leading to connection failure between UE and the target cell. Furthermore, if the NCC, keySetChangeIndicator or the chosen algorithms of the target gNB are carried by the LTM Cell Switch Command MAC CE, they could be tampered, if the UE has accepted the altered NCC value, this may result in out-of-sync, key mismatch or security negotiation failure between the UE and target gNB.

### 2.1.3	Potential security requirements

The 5GS shall support key handling for inter-CU LTM based cell switch scenario.

The 5GS shall support the synchronization of selected security algorithm for inter-CU LTM based cell switch scenario.

The 5GS shall support the synchronization of UE UP security activation status for inter-CU LTM based cell switch scenario.

The 5GS shall support security mechanism for the scenarios/features decided and specified by the RAN WGs for inter-CU LTM based cell switch scenario.

NOTE: There should be no impact to existing Layer 3 mobility.

NOTE: The above security threat(s) is not addressed in present document.

## 2.X	Key Issue #X: Security Aspects on MAC CE Protection

### 2.X.1	Key issue details

The L1/L2 Triggered Mobility (LTM) is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE serving cell by a cell switch command signalled via a MAC CE. As specified in TS 38.321 [x], the LTM cell switch command MAC CE may also include parameters such as NCC, Indication of chosen algorithm of target cell, and keySetChangeIndicator.

### 2.X.2	Security threats

An attacker may alter the MAC CE, which is causing the desynchronisation between the UE and the BS. The attacker may eavesdrop and thereafter resend the MAC CE with manipulated/altered content, with the aim to impact the key derivation. The attacker may replay the MAC CE frames, which is (again) causing a desynchronisation.

### 2.X.3	Potential security requirements

The 5GS shall support MAC CE security.

NOTE: There should be taken into consideration security measures for the support of confidentiality, integrity and anti-replay protection.

* * * End of Changes * * * *