3GPP TSG-SA3 Meeting #120	S3-250437

Athens, Greece, 17 - 21 February 2025

# Source:	Qualcomm Incorporated

# Title:	U2U relay discovery security

# Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

This contribution propose text for U2U relay discovery security.

# 2	References

[1]	TR 33.743

[2]	Skeleton for the draft CR

# 3	Rationale

This contribution proposes text for U2U relay discovery security based on the conclusions in the TR 33.743 [1].

# 4	Detailed proposal

It is proposed that the below changes to the draft CR [2] are approved.

# * * * * First change * * * *

#### 6.1.3.Y	5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery

##### 6.1.3.Y.1	General

This clause describes the security requirements and the security procedures that are specifically applied to 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery defined in 6.3.2.6 of TS 23.304 [2].

##### 6.1.3.Y.2	Security requirements for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery

The 5G system shall provide a means to securely provision the security materials for multi-hop UE-to-UE relay discovery.

The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of discovery messages in the multi-hop UE-to-UE Relay discovery.

The 5G System shall provide a means for mitigating trackability and linkability attacks on UEs in multi-hop UE-to-UE Relay discovery.

##### 6.1.3.Y.3	Security flows for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery

###### 6.1.3.Y.3.YY	Security of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery for IP PDU type

The 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery of IP PDU type consists of two types of relay discovery: one for Relay discovery among 5G ProSe UE-to-UE Relays and the other one for Relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay as specified in clause 6.3.2.6.2 of TS 23.304[2]. The discovery of the target 5G ProSe End UEs are performed via DNS queries after establishing a secure PC5 link with the 5G ProSe Multi-hop UE-to-UE Relay(s).

6.1.3.Y.3.YY.1	Relay discovery among 5G ProSe UE-to-UE Relays

For the provisioning of discovery security materials and discovery message protection based on the discovery security materials associated with an RSC for multi-hop UE-to-UE Relay, the security procedures for 5G ProSe UE-to-Network Relay discovery with Model A and Model B as specified in clause 6.1.3.2.2 are reused with the following change:

- 	One 5G ProSe UE-to-UE Relay plays the role of a 5G ProSe Remote UE and the other 5G ProSe UE-to-UE Relay plays the role of a 5G ProSe UE-to-Network Relay.

6.1.3.Y.3.YY.2	Relay discovery between a 5G ProSe End UE and 5G ProSe UE-to-UE Relay

For the provisioning of discovery security materials and discovery message protection based on the discovery security materials associated with an RSC for multi-hop UE-to-UE Relay, the security procedures for 5G ProSe UE-to-Network Relay discovery with Model A and Model B as specified in clause 6.1.3.2.2 are reused with the following change:

- 	A 5G ProSe End UE plays the role of a 5G ProSe Remote UE and a 5G ProSe UE-to-UE Relay plays the role of a 5G ProSe UE-to-Network Relay.

# * * * * End of change * * *