# 3GPP TSG-SA3 Meeting #120 	S3-250463

# February 17 –21, 2025, Athens, GR

# Source: 	Interdigital

# Title: 	New KI: Reader Authorization for 5G Ambient IoT Services

# Document for: 	Approval

Agenda Item: 	5.9

# 1	Decision/action requested

Approve the pCR to TR 33.713

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[3]	TS 22.369 Service requirements for Ambient power-enabled IoT

# 3	Rationale

This contribution proposes a new Key Issue #X: Reader Authorization for 5G Ambient IoT Services, to address and study potential attacks from adversarial AIoT Readers that initiate unauthorized inventory or command procedures. There is a companion contribution in S3-250464, “Conclusion to Key Issue #X “Reader Authorization for 5G Ambient IoT Services” proposing to consider Readers to be authenticated and authorised and not requiring standardization work in Rel-19.

# 4	Detailed proposal

SA3 is asked to approve the following pCR.

*** Start of 1st Change ***

## 5.X	Key Issue #X: Reader Authorization for 5G Ambient IoT Services

### 5.X.1	Key issue details

An adversary can spoof a legitimate AIoT Reader’s paging message, causing an AIoT device to respond with either an unauthorized inventory response or the execution of an unauthorized command.

The current Key Issue #2, “Authorization for 5G Ambient IoT Services,” states that:

“The 5GS shall be able to support the authorization of the AIoT-capable UE as an intermediate node in 5G Ambient IoT services.”

This requirement, however, only addresses the authorization of the AIoT-capable UE and does not ensure that the AIoT device can verify the authorization status of an AIoT Reader. Consequently, this oversight exposes AIoT devices to impersonation and spoofing attacks by unauthorized Readers.

To mitigate this vulnerability, it is essential to explore mechanisms that enable AIoT devices to distinguish between legitimate, authorized AIoT Readers and illegitimate ones.

### 5.X.2	Security threats

An adversary operating an illegitimate, unauthorized AIoT Reader can spoof a legitimate Reader paging message and cause the AIoT device to respond with either inventory response or execution of the invoked command.

A competitor or thief can perform an unauthorized store inventory by scanning AIoT devices with an unauthorized reader to determine the types and quantities of items. An unauthorized reader can query the AIoT devices e.g., UPC if AIoT devices used in the supply chain respond to unauthorized Readers.

An adversary can spoof a legitimate AIoT Reader’s paging message, causing an AIoT device to respond with either an inventory response or the execution of an unauthorized command.

To mitigate this vulnerability, it is essential to explore mechanisms that enable AIoT devices to distinguish between legitimate, authorized AIoT Readers and illegitimate unauthorized ones.

### 5.X.3	Potential security requirements

The 5GS shall support mechanisms to authorize AIoT Readers, ensuring that only legitimate Readers can communicate with AIoT devices.

The 5GS shall support a mechanism for AIoT devices to verify the authorization status of AIoT Readers before responding to their requests.

*** End of 1st Change ***