3GPP TSG-SA3 Meeting #120	S3-250469

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on Conclusion update for KI#2.1

Document for:	Approval

Agenda item:	5.14

Spec:	3GPP TR 33.749

Version:	V0.5.0

Work Item:	FS_EDGE_Ph3

Comments

This contribution updates the conclusion on KI#2.1 of TR 33.749 by introducing two changes:

1) The generic name of the UE Identifier API is replaced by the accurate name of the API (clause 8.6.5.4 of TS 23.558), i.e., Eees_UEIdentifier.

2) The third design principle is made extensible to the UE Identifier API. Here we use the generic term to be consistent with the third principle formulation.

* * * First Change * * * *

## 7.2	KI#2.1: Secure retrieval of 5G system UE Ids and privacy related information

The following solution design principles are to be followed in normative phase:

- The information used in Nnef_UEId API call invocation and Eees_UEIdentifier API call invocation needs to prevent the use of permanent or semi-permanent UE identifiers subject to be abused/compromised by malicious applications.

- The entity responsible for the generation of the information needs to be part of the 5G system in HPLMN, and needs to be verified by the 5G system in HPLMN. Alternatively, the information can be generated by both the UE and the 5G system.

- The information may be used by the AF invoking the NEF APIs, so that the NEF APIs can identify the UE by using the information.

- The information may be used by the EAS/EEC invoking the UE Identifier API, so that the EES APIs can identify the UE by using the information.

NOTE 1: The API invoker does not need to obtain the permanent 5G UE ID, which improves the privacy.

Alternative solutions having the design principles above will be captured in an informative annex in the normative work.

NOTE 2: The principles above will not prevent usage of existing mechanisms of Nnef_UEId  API and Eees_UEIdentifier API.

* * * End of Changes * * * *