3GPP TSG-SA3 Meeting #120	S3-250643

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0074 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TC RRC replay protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRC messsages typically create response messages and/or gNB reactions, but it could also be that these are more complete messages and/or these do not request and response from the gNB. Therefore, it is not sufficient to take a missing gNB response message as evidence for an existing replay protection mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In addition to any gNB response message, the tester should verify whether log-entries exist that document a discard of packets and the tester should capture N2 interface to see whether replayed RRC messages have initiated any reaction over the N2 interface.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing or wrong evidence and therefore wrong assessment about the existence of a replay protection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.2.2.1.9	Replay protection of RRC-signalling

Requirement Name: Replay protection of RRC-signalling.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of RRC-signalling as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.2 – Control plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-RRC-REPLAY_gNB

Purpose: To verify the replay protection of RRC-signalling between UE and gNB over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments.

-	Tester shall have knowledge of the integrity algorithm and the corresponding protection keys.

-	The tester shall have access to the NG RANs air interface.

-	The tester shall have access to the N2 interface.

-	The tester shall activate the integrity protection of RRC-signalling.

Execution Steps:

1.	The tester shall capture the data sent between UE and the gNB using any network analyser over the NG RAN air interface.

2.	Tester shall filter RRC signalling packets.

3.	Tester shall check for the PDCP COUNT of the filtered RRC signalling packets and shall use any packet crafting tool to create RRC signalling packets similar to the captured packets or the tester shall replay the captured RRC uplink packet to the gNB to perform the replay attack over gNB.

4.	 Tester shall check whether the replayed RRC signalling packets were processed by the gNB or not, by capturing over NG RAN air interface to see if any corresponding response message is received from the gNB or by verifying the gNB log files if there are entries about RRC packet discard or by capturing the N2 interface to see if any of the replayed user plane packets have been forwarded by the gNB..

5.	Tester shall confirm that gNB provides replay protection by dropping/ignoring the replayed packet if no corresponding response is sent by the gNB to the replayed packet or if the gNB logs include corresponding log entries or if no replayed user plane packets have been forwarded over the N2 interface.

Expected Results:

The RRC signalling over the NG RAN air interface is replay protected.

Expected format of evidence:

-	Evidence suitable for the interface, e.g. Screenshot containing the operational results.

-	Log files, e.g., containing corresponding log events.