3GPP TSG-SA3 Meeting #120	S3-250650

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Proposed baseline for conclusion on KI#4

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This contribution proposes updates on top of the latest revision of the merged document S3-250206 discussed during the last meeting. The changes are listed below:

- Clarified that the item list applies to the network layer security solution.

- Clarified that the keys used for the command protection are session key.

- Added a statement to reuse as much as possible the NAS confidentiality and integrity protection mechanisms of TS 33.501 while leaving the details (e.g., input parameter definitions) to the normative phase.

* * * First Change * * * *

## 7.X	Conclusion to Key Issue#4 on Protection of information during AIoT service communication

The following aspects are agreed for the protection of AIoT information carried in the command procedure:

-  The AIoTF acts as the security termination point for information protection in case of network layer security.

-  The protection of command request and response messages including confidentiality, integrity and replay protected is based on the session keys derived from pre-shared key and freshness parameter with a successful authentication run.

-  The integrity and ciphering mechanisms are using the same algorithm interfaces of Annex D from TS 33.501 [5].

NOTE: The details of input parameters for confidentiality, integrity and replay protection are to be resolved during normative phase.

-  The device security capabilities are known/preconfigured in the network side in advance. Therefore, there is no need for security capability negotiation.

* * * End of Changes * * * *