3GPP TSG-SA3 Meeting #120	S3-250671

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Clean up TR 33.743

Document for:	Approval

Agenda item:	5.12

Spec:	3GPP TR 33.743

Version:	1.0.0

Work Item:	FS_5G_ProSe_Ph3_Sec

Comments

This contribution tries to clean up TR 33.743 [1]. As part of completing the study, the template and corresponding Editor’s Notes for adding new key issues and new solutions were removed, and any remaining Editor’s Notes were converted to NOTEs.

* * * First Change * * * *

5	Key issues

* * * Second Change * * * *

5.1	Key issue #1: Security for multi-hop UE-to-Network Relay

5.1.1	Key issue details

Based on the information exchange between Remote UE and network via the UE-to-Network Relay in previous releases, the multi-hop UE-to-Network Relay scenario in TR 23.700-03 [1] further allows the Remote UE connecting to the network via one or more Intermediate UE-to-Network Relay(s) in proximity, by using either Layer-2 or Layer-3 connection methods. The Key Issue #1 (Support of multi-hop UE-to-Network Relays) in TR 23.700-03 [1] has the following note:

NOTE 3:	Security and privacy aspects will be handled by SA WG3.

The 5GS is supposed to be able to provide security (and privacy) protection of messages from the Remote UE, via Intermediate UE-to-Network Relay(s) and UE-to-Network Relay, to the network and vice versa. Failure to provide security (and privacy) protection of these messages may lead to various attacks, e.g. information manipulation or information leakage. Therefore, the security and privacy aspects of the discovery and communication messages in 5G ProSe multi-hop UE-to-Network Relay should be investigated.

This key issue focuses on the security (and privacy) issues for 5G ProSe multi-hop UE-to-Network Relay over NR PC5 reference point, including both discovery and communication scenarios.

5.1.2	Threats

If the exchanged messages are not confidentiality protected, integrity protected or replay protected, the parameters included can be obtained, modified or replayed by an attacker. Consequently, it may lead to various attacks such as information manipulation (e.g. Relay Service Code, hop count), privacy information (e.g. Relay Service Code) leakage or unable to discover each other for an intended service.

An attacker may impersonate the Remote UE, Intermediate UE-to-Network Relay or UE-to-Network Relay. If the authentication and authorisation of UEs cannot be verified, an attacker UE may impersonate the Remote UE, Intermediate UE-to-Network Relay or UE-to-Network Relay.

5.1.3	Potential security requirements

The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of discovery and communication messages in multi-hop UE-to-Network Relay discovery and communication scenarios.

The 5G System shall provide a means for mitigating trackability and linkability attacks on UEs in multi-hop UE-to-Network Relay discovery and communication scenarios.

The 5G System shall provide a means for authentication and authorisation of the UEs in multi-hop UE-to-Network Relay communication scenarios.

The 5G system shall provide a means to securely provision the security materials for multi-hop UE-to-Network Relay discovery.

NOTE: 	The specific E2E information, and support of E2E protection of the E2E information between the Remote UE and the UE-to-Network Relay will be determined at the normative stage, the alignment with architecture aspects in TR 23.700-03 [1] and TS 23.304 [4] need to be considered.

* * * Third Change * * * *

* * * Fourth Change * * * *

6	Solutions

* * * Fifth Change * * * *

6.4	Solution #4: Security of multi-hop UE-to-Network Relay communication

6.4.1	Introduction

This solution addresses key issue #1: Security for multi-hop UE-to-Network Relay.

Each hop of multi-hop UE-to-Network Relay communication (e.g. between Remote UE and intermediate UE-to-Network Relay, between target UE-to-Network Relay and intermediate UE-to-Network Relay, or between two intermediate UE-to-Network Relays) performs PC5 link security establishment procedure and sets up PC5 security context, reusing the methods as specified in clause 6.6.3 of TS 33.503[5].

Once hop-by-hop security for PC5 link establishment has been performed, the Remote UE and the target UE-to-Network Relay triggers e2e security between them, reusing the security methods (UP based or CP based solution) as specified in clause 6.3.3 of TS 33.503[5]. The signaling of the selected security method is overlay over the secured PC5 link of each hop.

NOTE: 	There could be one or more intermediate UE-to-Network Relays in the discovery message path. The maximum number of intermediate UE-to-Network Relays in the path is to be defined by SA2. This solution shows only one intermediate UE-to-Network Relay as example.

Additionally, to support authorization between the Remote UE and the target U2N, this solution proposes to reuse the existing UP or CP based security procedure between the Remote UE and the target U2N as specified in TS 33.503 so that the Remote UE and the target U2N can authorize each other after security context is established between the two.

6.4.2	Solution details

The security procedure for multi-hop 5G ProSe UE-to-Network Relay communication is described as follows.

Figure 6.4.2-1: Security for multi-hop UE-to-Network Relay communication

0.	The 5G ProSe Remote UE, the intermediate UE-to-Network Relay(s) and the target 5G ProSe UE-to-Network Relay sets up hop by hop PC5 link security, reusing the methods as specified in clause 6.6.3 of TS 33.503[5].

After this procedure, secured PC5 transport messages can be exchanged between PC5 links of each hop and protected by the PC5 link security context of each hop.

NOTE: 	It is assumed existing methods e.g. security for Direct Communication as specified in clause 6.2 of TS 33.503 [5], or security for UE-to-Network Relay Communication (UP or CP based solution) as specified in clause 6.6.3 of TS 33.503 [5] can be used to set up per hop PC5 security.

For example, in case 5G ProSe Intermediate UE-to-Network Relay(s) are out of coverage, the security procedure specified in clause 6.2 is used to establish a secure PC5 link for each hop. If 5G ProSe Intermediate UE-to-Network Relay(s) are in-coverage, the CP-based/UP-based procedures as specified in clauses 6.3.3 of TS 33.503 [5] can be used.

Whether there is any need of enhancement of the existing the security methods to set up per hop PC5 security, it is not addressed in this solution.

- The 5G ProSe Remote UE is to set up e2e security with the target 5G ProSe UE-to-Network Relay and chooses a security method to be used (i.e. UP based or CP based solution). The 5G ProSe Remote UE forms U2N security container to be sent to the target 5G ProSe UE-to-Network Relay over intermediate UE-to-Network Relay(s) , which contains Direct Communication Request (DCR) messages that contains the UP-PRUK ID/CP-PRUK ID or SUCI, RSC and freshness parameter Nonce_1, as specified in clause 6.3.3.2.2 or clause 6.3.3.2.3  of TS 33.503[5]. The 5G ProSe Remote UE protects U2N security container using the relay discovery security material associated with the target 5G ProSe UE-to-Network Relay, by reusing the protection method as specified in clause 6.3.5 of TS 33.503[5].

The 5G ProSe Remote UE sends the U2N security container and possible other additional information (e.g. hop counter) required for multi-hop UE-to-Network Relay over a secured PC5 transport message to the intermediate UE-to-Network Relay.

NOTE: The content of possible additional information (e.g. hop count) that is required for multi-hop UE-to-Network relay is to be defined by SA2.

- The intermediate UE-to-Network Relay forwards the secured transport PC5 message to the target 5G ProSe UE-to-Network Relay.

  - The target UE-to-Network Relay performs UP based solution (step 4 of clause 6.3.3.2.2 TS 33.503[5]) or CP based solution (step 3-13 of clause 6.3.3.3.2 TS 33.503[5]). The messages, if available, exchanged between the Remote UE and the target 5G ProSe UE-to-Network Relay are sent within the U2N security container over secured PC5 transport messages via PC5 link of each hop. The UE-to-Network Relay also receives the Remote User ID of the 5G ProSe Remote UE (CP-PRUK ID or UP-PRUK ID) from the Network during the UP or CP based security procedure as specified in clause 6.3.3.2.2 and clause 6.3.3.3.2 of TS 33.503[5].

- The 5G ProSe Remote UE and the target 5G ProSe UE-to-Network Relay performs Direct Security Mode Command procedure as specified in UP based solution (step 5 of clause 6.3.3.2.2 TS 33.503[5]) or CP based solution (step 14-17 of clause 6.3.3.3.2 TS 33.503[5]). The messages exchanged between the Remote UE and the target 5G ProSe UE-to-Network Relay are sent within the U2N security container over secured PC5 transport messages via PC5 link of each hop.

Successful verification of the Direct Security Mode Command assures the 5G ProSe Remote UE that the 5G ProSe UE-to-Network Relay is authorized to provide the relay service. Successful verification of the Direct Security Mode Complete message assures the 5G ProSe UE-to-Network Relay that the 5G ProSe Remote UE is authorized to get the relay service.

  - After successful verification, PC5 security context is also set up between the Remote UE and the target 5G ProSe UE-to-Network Relay. The 5G ProSe UE-to-Network Relay responds a Direct Communication Accept message within U2N security container over secured PC5 transport messages via PC5 link of each hop. The U2N security container is protected by the PC5 security context between the Remote UE and the target 5G ProSe UE-to-Network Relay. Whether communication protection between Remote UE and U2N is activated is determined by the negotiated PC5 signalling security policy and the PC5 user plane security policies during UP/CP based security procedure as specified in clause 6.3.3.2.2 and clause 6.3.3.3.2 of TS 33.503[5].

9.	When the conditions to send a Remote UE Report reach as specified in TS 23.304 [2], the target 5G ProSe UE-to-Network Relay shall send a Remote UE Report (Remote User ID, Remote UE info) message to the SMF of the 5G ProSe UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay shall include Remote User ID (i.e. UP-/CP-PRUK ID received in step 5 in the message.

10.	If the mapping of the Remote User ID and the 5G ProSe Remote UE's SUPI is not available in the SMF of the 5G ProSe UE-to-Network Relay, the SMF of the 5G ProSe UE-to-Network Relay performs the procedure as specified in clause 6.3.3.2.2 and clause 6.3.3.3.2 of TS 33.503[5] to resolve Remote User ID procedure.

11. The Remote UE and the target 5G ProSe UE-to-Network Relay perform the rest of the UE-to-Network relay procedure.

The conclusion to KI#1 "Support of multi-hop UE-to-Network Relays" in TR 23.700-03 [1], related to communication and link establishment states the following principle: "Standalone discovery & link setup/management is considered and both Model A and Model B are supported". The 5G ProSe Direct Communication procedure defined in TS 23.304 [4] is reused for the Layer-2 link establishment between each pair of hops in the path. Step 0a, 0b in this solution aligns with the above principles therefore the solution is aligned to TR 23.700-03[1] conclusions.

6.4.3	Evaluation

The solution addresses key issue #1 and provides methods for security protection (confidentiality protection, integrity protection and replay protection, and mitigating trackability and linkability attacks) of multi-hop UE-to-Network Relay communication.

The solution proposes to perform PC5 link security establishment and sets up PC5 security context for each hop of multi-hop UE-to-Network Relay communication (e.g. between Remote UE and intermediate UE-to-Network Relay, between target UE-to-Network Relay and intermediate UE-to-Network Relay, or between two intermediate UE-to-Network Relays), reusing the methods as specified in TS 33.503[5].  For example, in case 5G ProSe Intermediate UE-to-Network Relay(s) are out of coverage, the security procedure specified in clause 6.2 is used to establish a secure PC5 link for each hop. If 5G ProSe Intermediate UE-to-Network Relay(s) are in-coverage, the CP-based/UP-based procedures as specified in clauses 6.3.3 of TS 33.503 [5] can be used.

Once hop-by-hop security for PC5 link establishment is established, the Remote UE and the target UE-to-Network Relay triggers authentication and authorization as well as e2e security context establishment between them, reusing the security methods (UP based or CP based solution) as specified in clause 6.3.3 of TS 33.503[5]. The signaling of the security context establishment between the Remote UE and the target U2N is overlay over the established PC5 security link of each hop.

The security context establishment between the Remote UE and the target U2N enables authorization between the two UEs. It can also enable signaling and traffic protection between the Remote UE and the target U2N if required by their PC5 security policy.

Furthermore, this solution also support security for Remote UE Report procedure. As output of UP or CP based security procedure between the Remote UE and the target U2N, the target U2N also obtains the Remote User ID (i.e. UP-PRUK ID or CP-PRUK ID) of the remote UE from the Network. When the conditions to send a Remote UE Report reach as specified in TS 23.304 [2], the 5G ProSe UE-to-Network Relay sends a Remote UE Report message with the received (UP-/CP-PRUK ID) to the SMF of the 5G ProSe UE-to-Network Relay. Thus, the existing Remote UE Report procedure can be reused.

This solution assumes hop-by-hop security establishment for the Multi-hop U2NW Relay path reuses the methods as specified in clause 6.6.3 of TS 33.503[5]. When mechanism without network assistance in 6.6.3.2 of TS 33.503 is used, the Remote UE is required to be provisioned with two sets of security materials (i.e. the Long-Term Credential and the CP/UP-PRUK). When mechanism with network assistance in 6.6.3.1 of TS 33.503 is used, the solution only works when the Intermediate Relay in network coverage.

* * * Sixth Change * * * *

6.6	Solution #6: Security for multi-hop UE-to-Network Relay Communication

6.6.1	Introduction

This solution addresses security requirements for communication scenarios (as show in Figure 6-6) in Key Issue #1 (as defined in clause 5.1) .

Figure 6.6.1-1: Example scenario of multi-hop UE-to-Network Relay

Figure 6.6.1-2: High level security procedure for multi-hop UE-to-Network Relay Communication

1. Security procedure for each hop PC5 Link of multi-hop 5G ProSe UE-to-Network Relay Communication as described in clause 6.6.2.

2a. For the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication with N3IWF support, the 5G ProSe Remote UE establish security protection with 5G network as described in clause 6.6.3.

2b. For the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication without N3IWF support, as described in clause 6.6.4, the 5G ProSe Remote UE establish security protection with the UE-to-Network Relay to prevent eavesdropping attacks on the traffic of the 5G ProSe Remote UE.

2c. For the 5G ProSe Multi-hop Layer-2 UE-to-Network Relay Communication, the 5G ProSe Remote UE establish AS security with 5G network as described in clause 6.6.5.

6.6.2	Security procedure for each hop PC5 Link

The security procedure for 5G ProSe UE-to-Network Relay Communication as defined as defined in clause 6.3 of TS 33.503 [5] is used to establish a secure PC5 link between the Remote UE and the intermediate Relay, the intermediate Relay and the intermediate Relay, the intermediate Relay and the UE-to-Network Relay.

6.6.3	Security procedure for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication with N3IWF support

The 5G ProSe Layer-3 Remote UE selects N3IWF and performs the security procedures as specified in clause 7.2.1 of TS 33.501 [z].

6.6.4	Security procedure for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication without N3IWF support

Figure 6.6.4-1: Security procedure for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication without N3IWF support

- The hop-by-hop PC5 security has been established as described in clause 6.6.2.

1. The Remote UE sends a Multi-hop Communication Request to the UE-to-Network Relay via one or more Intermediate Relay, which includes information about the selected path, the SUCI or UP-/CP-PRUK ID of Remote UE, RSC and freshness_parameter_1.

2. Upon receiving the Multi-hop Communication Request from the remote UE via one or more Intermediate Relay, the UE-to-Network Relay perform the security procedure with network assistance as specified in step 4 of clause 6.3.3.2.2 of TS 33.503 [y] or step 3 to step 13 in clause 6.3.3.3.2 of TS 33.503 [5].

NOTE 1: 	Whether UP based and CP based PC5 security procedures as specified in clause 6.3.3 of TS 33.503 [5] can be reused will be determined at the normative stage.

3. The 5G ProSe UE-to-Network Relay shall derive the session key from KNRP/KNR_ProSe and then derive the confidentiality key (if applicable) and integrity key. The 5G ProSe UE-to-Network Relay shall sends a Authorization Check Request message to the 5G ProSe Remote UE via one or more Intermediate Relay. This message shall also include the freshness Parameter 2 and shall be protected by integrity key.

4. Upon receiving the Authorization Check Request message, the Remote UE shall derive the KNRP/KNR_ProSe from UP-/CP-PRUK. It shall then derive the session key and the confidentiality key (if applicable) and integrity key and process the Authorization Check Request message. Successful verification of the Authorization Check Request message assures the Remote UE that the UE-to-Network Relay is authorized to provide the Multi-hop U2N Relay service.

5. The Remote UE responds with a Authorization Check Response message to 5G ProSe UE-to-Network Relay via one or more Intermediate Relay if successfully verified the Authorization Check Request message. The Authorization Check Response message shall be protected by integrity key and confidentiality key (if applicable).

6. Upon receiving the Authorization Check Response message, the UE-to-Network Relay shall verify the this message. Successful verification of the Authorization Check Response message assures the 5G ProSe UE-to-Network Relay that the 5G ProSe Remote UE is authorized to get the Multi-hop U2N Relay service.

7. After the successful verification of the Authorization Check Response message, the 5G ProSe UE-to-Network Relay responds a Multi-hop Communication Accept message to the 5G ProSe Remote UE via one or more Intermediate Relay to finish the Multi-hop Communication establishment procedures. The Multi-hop Communication Accept message shall be protected by integrity key and confidentiality key (if applicable).

NOTE 2: 	Whether the session key between the remote UE and U2N relay is used for security protection is left for normative phase.

NOTE 3: 	The Multi-hop Communication Request/Accept message and Authorization Check Request/Response message are transmitted between the Remote UE and the UE-to-Network Relay over hop-by-hop PC5 link.

6.6.5	Security procedure for 5G ProSe Multi-hop Layer-2 UE-to-Network Relay Communication

The 5G ProSe Remote UE and NG-RAN node shall establish AS security as specified in TS 33.501 [z].

6.6.6	Evaluation

For the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication with N3IWF support, this solution propose to reuse the security procedure as defined in clause 7.2.1 of TS 33.501 [z] to establish E2E security protection between the 5G ProSe Remote UE and 5G network to prevent eavesdropping attacks on the traffic of the 5G ProSe Remote UE.

For the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication without N3IWF support, this solution propose to reuse the security procedure for 5G ProSe UE-to-Network Relay Communication as described in clause 6.3 of TS 33.503 to establish E2E security protection between the 5G ProSe Remote UE and the UE-to-Network Relay to prevent eavesdropping attacks on the traffic of the 5G ProSe Remote UE.

For the 5G ProSe Multi-hop Layer-2 UE-to-Network Relay Communication, this solution propose to reuse the AS security procedure as defined in TS 33.501 to establish E2E security protection between the 5G ProSe Remote UE and NG-RAN to prevent eavesdropping attacks on the traffic of the 5G ProSe Remote UE.

This solution assumes hop-by-hop security establishment for the Multi-hop U2NW Relay path reuses the methods as specified in clause 6.6.3 of TS 33.503[5]. When mechanism without network assistance in 6.6.3.2 of TS 33.503 is used, the Remote UE is required to be provisioned with two sets of security materials (i.e. the Long-Term Credential and the CP/UP-PRUK). When mechanism with network assistance in 6.6.3.1 of TS 33.503 is used, the solution only works when the Intermediate Relay in network coverage.

NOTE: Further evaluation is for normative phase.

* * * Seventh Change * * * *

6.8	Solution #8: Multi-hop UE-to-network Relay security establishment procedure

6.8.1	Introduction

This solution is proposed to address Key Issue #1, which provides a method by which the Remote UE can securely establish the communication with the UE-to-network Relay via one or multiple Intermediate Relay(s).

In this solution, it is assumed that the security of each hop (i.e. the link between the Intermediate Relay1 and Intermediate Relay2, the link between the Intermediate Relay2 and UE-to-network Relay) has been established during the multi-hop UE-to-network Relay discovery procedure.

For establishing the security for multi-hop UE-to-network Relay communication, the DCR message is sent by the Remote UE and is transmitted over the secure link. Once receiving the DCR message, the UE-to-network Relay can perform the Remote UE report procedure as defined in TS 33.503 [5].

6.8.2	Solution details

Figure 6.8.2-1: Security establishment procedure for multi-hop UE-to-network Relay communication

- The Remote UE can securely discover the UE-to-network Relay by performing the multi-hop UE-to-network Relay discovery procedure.

It is assumed the final path selected by the Remote UE is Remote UE – Intermediate Relay1 – Intermediate Relay2 – UE-to-network Relay, and the security of each hop (i.e. the link between the Intermediate Relay1 and Intermediate Relay2, the link between the Intermediate Relay2 and UE-to-network Relay) has been established by reusing the existing procedure defined in TS 33.503 [5]. By reusing the existing link establishment procedure, the authorization of Intermediate Relay and UE-to-network Relay can be checked.

- The Remote UE sends the DCR message to the Intermediate Relay1, which may include RSC, user info ID of Remote UE, etc.

- Once receiving the DCR message, the Remote UE and Intermediate Relay 1 establish the PC5 link security connection by reusing existing link security establishment procedure defined in TS 33.503 [5].

- The Intermediate Relay1 forwards the DCR message, which may include RSC, user info ID of Remote UE, etc. The DCR message is transmitted over the secure link between the Intermediate Relay1 and Intermediate Relay2.

- The Intermediate Relay2 forwards the DCR message, which may include RSC, user info ID of Remote UE, etc. The DCR message is transmitted over the secure link between the Intermediate Relay2 and UE-to-Network Relay.

- The UE-to-network Relay returns the DCA message to the Remote UE via the Intermediate Relay(s).

- The UE-to-network Relay sends the Remote UE report to the SMF of Relay UE.

NOTE: 	Whether steps 3-6 are aligned with TR 23.700-03 [1] and TS 23.304 [4] will be determined at the normative stage.

6.8.3	Evaluation

This solution addresses the first security requirement of Key Issue #1 for the multi-hop UE-to-Network Relay communication.

In this solution, it is assumed that the security of each hop (i.e. the link between the Intermediate Relay1 and Intermediate Relay2, the link between the Intermediate Relay2 and UE-to-network Relay) has been established during the multi-hop UE-to-Network Relay discovery procedure.

This solution is for establishing the security between the Remote UE and Intermediate UE-to-Network Relay. The existing link establishment procedure is reused to support the multi-hop UE-to-Network service.

The Intermediate Relay can be assumed as a trusted node for forwarding the traffic between the Remote and UE-to-network Relay upon authentication and authorization are finished. Therefore, the E2E security between the Remote UE and UE-to-network Relay is not supported in this solution. The DCR message is for initiating the mutli-hop UE-to-Network Relay security establishment procedure with the selected Intermediate Relay(s) and UE-to-Network Relay, which is not aligned with TS 23.304 [4].

* * * Eighth Change * * * *

6.13	Solution #13: Solution of multi-hop UE-to-UE Relays Discovery Model B

6.13.1	Introduction

This Solution based on the solution #5 in TR 23.700-03 [1] and addressed the requirement of KI#2,

The 5G ProSe End UE are provisioned direct discovery security material associated with ProSe Code. The 5G ProSe End UE and U2U Relay are provisioned a set of discovery security material associated with RSC. The RSC related discovery security material is obtained from the HPLMN of discoveree 5G ProSe End UE.

The RSC related discovery security materials provisioning for Relay UE is between Relay UE and Discoveree End UE. The path information is protected by U2U RSC related discovery material.

6.13.2	Solution details

Figure 6.13.2-1 Security procedure for multi-hop UE-to-UE Relay Discovery Model B

1.	The discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are provisioned with the discovery security materials associated with a 5G ProSe Direct Discovery service based on the discovery security materials provisioning procedure for Restricted 5G ProSe Direct Discovery, as specified defined in clause 6.1.3.2.2.2.

The discoverer 5G ProSe End UE, discoveree 5G ProSe End UE and 5G ProSe UE-to-UE Relay are provisioned with the discovery security materials associated with a RSC based on the discovery security materials provisioning procedure for UE-to-Network Relay Discovery from the HPLMN of discoveree DDNMF, as specified in clause 6.1.3.2.2.2.

- The discoverer 5G ProSe End UE shall construct a direct discovery set that contains two End UE discovery infos. Each End UE discovery info is protected using the discovery security materials associated with the 5G ProSe Direct Discovery service as specified in clause 6.1.3.2.3. The first protected End UE discovery info shall include User Info ID of the discoverer 5G ProSe End UE, the UTC-based counter LSB parameter, and a MIC IE. The second protected End UE discovery info shall include the and User Info ID of the discoveree 5G ProSe End UE, the UTC-based counter LSB parameter, and a MIC IE. Then, the discoverer 5G ProSe End UE shall include the above protected End UE discovery set and HPLMN id of the discoveree 5G ProSe End UE in the Solicitation message and protect the Solicitation message using the discovery security materials associated with RSC obtained from the HPLMN of discvoeree 5G ProSe End UE as specified in clause 6.1.3.2.3. The solicitation message is sent to the 5G ProSe UE-to-UE Relay 1.

NOTE 1: 	How can the discoverer End UE knows the HPLMN ID of discoveree End UE is left to the normative phase.

3.	On receiving the 5G ProSe UE-to-UE Relay Discovery Solicitation message from the discoverer 5G ProSe End UE, the 5G ProSe UE-to-UE Relay 1 shall extract the PLMN id from the Discovery Solicitation message and process the received UE-to-UE Relay Discovery Solicitation message using the discovery security materials identified by extracted PLMN id. The discovery security materials is obtained from the HPLMN of discoveree 5G ProSe End UE.

If the verification is successful, the 5G ProSe UE-to-UE Relay 1 shall modify the UE-to-UE Relay Discovery Solicitation message to include User Info ID of the 5G ProSe UE-to-UE Relay 1.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message is protected using the security materials associated with the RSC based on the extracted PLMN id as specified in clause 6.1.3.2.3.

Then, 5G ProSe UE-to-UE Relay 1 sends the message to the 5G ProSe UE-to-UE Relay 2 including the extracted PLMN id which is the HPLMN of discoveree DDNMF.

4.5. 5G ProSe UE-to-UE Relay 2…N repeat the step 3.

6.	The discoveree 5G ProSe End UE shall extract the PLMN id and process the received UE-to-UE Relay Discovery Solicitation message using the discovery security materials associated with the RSC based on the extracted PLMN id which is the HPLMN of discoveree DDNMF,.

If the verification is successful, the discoveree 5G ProSe End UE shall extract the protected direct discovery set from the message and process the direct discovery set using the discovery security materials associated with the 5G ProSe Direct Discovery service as specified in clause 6.1.3.2.3.

The discoveree 5G ProSe End UE shall select a multi-hop relay path.

NOTE 2: 	Whether path selection procedure is aligned with TR 23.700-03 [1] and TS 23.304 [4] will be determined at the normative stage.

7.	The discoveree 5G ProSe End UE shall construct a direct discovery set that contains two End UE discovery infos. Each End UE discovery info is protected using the discovery security materials associated with the 5G ProSe Direct Discovery service as specified in clause 6.1.3.2.3. The first protected End UE discovery info shall include User Info ID of the discoverer 5G ProSe End UE, the UTC-based counter LSB parameter, and a MIC IE. The second protected End UE discovery info shall include the and User Info ID of the discoveree 5G ProSe End UE, the UTC-based counter LSB parameter, and a MIC IE. Then, the discoveree 5G ProSe End UE shall include the above protected direct discovery sets and HPLMN id of the discoveree 5G ProSe End UE in the Solicitation message and protect the Solicitation message using the discovery security materials associated with the RSC based on the extracted PLMN id which is the HPLMN of discoveree 5G ProSe End UE’s DDNMF as specified in clause 6.1.3.2.3. The discoveree 5G ProSe End UE replies to the 5G ProSe UE-to-UE Relay N with the UE-to-UE Relay Discovery Response message.

8.  On receiving the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE, the 5G ProSe UE-to-UE Relay N shall extract the PLMN id from the Discovery Response message and process the received UE-to-UE Relay Discovery Response message using the discovery security materials associated with the RSC based on the extracted PLMN id which is the HPLMN of discoveree 5G ProSe End UE’s DDNMF.

If the verification is successful, the 5G ProSe UE-to-UE Relay N shall modify the UE-to-UE Relay Discovery Response message to include User Info ID of 5G ProSe UE-to-UE Relay N.

The UE-to-UE Relay Discovery Response message is protected using the security materials associated with the RSC based on the extracted PLMN id which is the HPLMN of discoveree 5G ProSe End UE’s DDNMF. Then, 5G ProSe UE-to-UE Relay N sends the UE-to-UE Relay Discovery Response message to UE-to-UE Relay 2.

Then, 5G ProSe UE-to-UE Relay N sends the message to the 5G ProSe UE-to-UE Relay 2 including the extracted PLMN id which is the HPLMN of discoveree DDNMF.

9. The 5G U2U Relay 2 repeat the step 8.

10. On receiving the UE-to-UE Relay Discovery Response message, the discoverer 5G ProSe End UE shall extract the PLMN id from the Discovery Response message and process the UE-to-UE Relay Discovery Response message using the discovery security materials associated with the RSC based on the extracted PLMN id which is the HPLMN of discoveree 5G ProSe End UE’s DDNMF.

If the verification is successful, the discoverer 5G ProSe End UE shall extract the protected direct discovery set from the UE-to-UE Relay Discovery Response message and process the protected End UE discovery infos using the discovery security materials associated with the 5G ProSe Direct Discovery service as specified in clause 6.1.3.2.3. If the verification of the first End UE discovery info is successful and the User Info ID of the discoverer matches, the discoverer 5G ProSe End UE processes the second End UE discovery info.

6.13.3	Evaluation

This solution addresses the provision of security material in discovery procedure of KI#2. This solution provides a method that the U2U discovery message between each pair of UEs in the path are protected by the security material provided from HPLMN of Discoveree End UE with the assumption that the U2U relays are in the same HPLMN with the Discoveree End UE.

NOTE: Further evaluation is for normative phase.

* * * Ninth Change * * * *

* * * Tenth Change * * * *

7	Conclusions

* * * Eleventh Change * * * *

7.2	Key issue #1: Security for multi-hop UE-to-Network Relay

The following statements are agreed as a basis for normative work:

For 5G ProSe Multi-hop UE-to-Network Relay discovery:

- The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay’s HPLMN respectively, based on the procedure specified in clause 6.1.3.2 of TS 33.503 [5].

- The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

- The Intermediate UE-to-Network Relay processes the received discovery message using the provisioned discovery security materials associated with the RSC as specified in clause 6.1.3.2 of TS 33.503[5]. If the processing is successful, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count, Relay Info.) and protects the updated message using the same security materials associated with the RSC as specified in clause 6.1.3.2 of TS 33.503 [5].

NOTE 1: Since the Intermediate UE-to-Network Relay and 5G ProSe UE-to-Network Relay are provisioned with a single set of discovery security materials from the 5G PKMF/5G DDNMF in their own HPLMN, it is assumed Intermediate UE-to-Network Relays and 5G ProSe UE-to-Network Relay belong to the same HPLMN during 5G ProSe multi-hop UE-to-Network Relay discovery.

- Both discovery with Model A and discovery with Model B are supported.

- For discovery with Model A, the Intermediate UE-to-Network Relay may (as specified in step 6 in clause 6.3.2.5.2 of TS 23.304 [4]) protect and send the Announcement message after a secure PC5 link establishment with the upstream Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay.

NOTE 1: 	How the PC5 link establishment is performed based on the optional procedure (step 6 in clause 6.3.2.5.2 of TS 23.304 [4]) will be determined at the normative stage.

- For discovery with Model B, the 5G ProSe Remote UE protects the discovery message (i.e., Relay Discovery Solicitation) reusing the procedures as specified in clause 6.1.3.2.3 of TS 33.503[5], i.e., using the discovery security materials associated with the RSC and HPLMN ID of the potential 5G ProSe UE-to-Network Relay.

For Key issue #1 on the support Security for multi-hop UE-to-Network Relay communication, the following principles apply:

1)	For UE-to-Network Relay communication security following Model A discovery:

- The security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay is based on CP-based/UP-based procedures as specified in clause 6.3.3 of TS 33.503 [5].

2)	For UE-to-Network Relay communication security following Model B discovery, the existing CP-based/UP-based procedures as specified in the in clauses 6.3.3 of TS 33.503 [5] are used as baseline for the normative work with the following principles:

- To trigger the multi-hop U2NW Relay link security establishment, the Remote UE sends a DCR message as specified in the in clauses 6.3.3 of TS 33.503 [5] to the Intermeditate UE-to-Network Relay.

- The Intermediate UE-to-Network Relay plays the role of the Remote UE to establish secured link with the UE-to-Network Relay or with the next Intermediate UE-to-Network Relay, reusing the procedure as specified in the in clauses 6.3.3 of TS 33.503 [5], if the Intermediate UE-to-Network Relay does not have an existing PC5 link with the UE-to-Network Relay or the next Intermediate UE-to-Network Relay, and the next Intermediate UE-to-Network Relay is in-coverage.

NOTE 2: The next Intermediate UE-to-Network Relay are considered in-coverage if they have connection to the network.

NOTE 3: 	Whether a separate RSC for multi-hop or the same RSC as for single hop is provisioned is to be determined during normative phase in coordination with SA2.

3)	For authentication and authorization, the existing CP-based/UP-based procedures as specified in clause 6.3.3 of TS 33.503 [5] can be used as baseline for each hop during the normative work.

NOTE 4: 	Further work on authentication and authorization between the 5G ProSe Remote UE and potential 5G ProSe UE-to-Network Relay via the network will be determined at the normative stage.

4)	For Remote UE report procedure, the existing procedures for Remote UE Report as specified in clause 6.3.3 of TS 33.503 [5] can be used as baseline for the normative work.

NOTE 5: 	Further work on Remote UE report procedure will be determined at the normative stage.

NOTE 6: How the Remote User ID of the Remote UE is obtained by the 5G ProSe UE-to-Network Relay for the Remote UE Report procedure is to be determined during normative phase in coordination with SA2.

NOTE 7: For the security procedure over User Plane, it is assumed that an Intermediate UE-to-Network Relay is able to access to the 5G PKMF in its HPLMN (i.e., PC8 establishment via Intermediate UE-to-Network Relay(s) or UE-to-Network Relay).

* * * End of Changes * * * *