3GPP TSG-SA3 Meeting #120	S3-250729

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | 0103 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add threat description about the implications of the AUSF confirming an authentication with the SUPI while the authentication was started with the SUCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI, Montsecure |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Allow testing of confirmation attacks while authenticating with the AUSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a threat reference for the security implications of flawed AUSF implementation that accepts the SUPI as authCtxId. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AUSF might be left vulnerable to a confirmation attack by a rogue SEAF (especially in a roaming scenario). The confirmation attack can be used to link a SUPI to a SUCI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 33.516 draftCR (S3-250730) |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

# F.2.2.X Usage of SUCI or SUPI as authCtxId

-	Thread name: SUCI or SUPI as authCtxId

-	Thread Category: Information retrieval

-	Thread description: As specified in TS 29.509, the SEAF can initiate a UE authentication by sending either a SUPI or a SUCI to the AUSF. The AUSF shall then generate an authCtxId that can be used by the SEAF to inform the AUSF of the authentication result. If the implementation of the AUSF network function saves the authentication context by SUPI (because both the SUPI and SUCI of the initial authentication request can be mapped to the SUPI) and the AUSF accepts a SUPI as authCtxId it might be vulnerable to a confirmation attack by the SEAF, especially in a roaming scenario, where the SEAF is part of the visiting network. A rogue SEAF could then start a UE authentication with a SUCI, compute a random RES*, and confirm the authentication by using the SUPI as ‘authCtxId’. If the AUSF responds to this request in any other way than a 404, the rogue SEAF can use the information from the response to determine if the SUPI was used to generate the SUCI.

-	Threatened Asset: Linkability between SUCI and SUPI

**** END OF CHANGES ****