3GPP TSG-SA3 Meeting #120	S3-250772

# Athens, Greece, 17 - 21 February 2025

Source:	Samsung

Title:	Security procedures for CAPIF interconnection

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS 33.122

Version:	18.4.0

Work Item:	CAPIF_Ph3_sec

Comments

This pCR proposes normative texts for security procedures for CAPIF interconnection based on the conclusions made in TR 33.700-22.

It is assumed that the API invoker is onboarded to CCF-A and the target AEF is registered to a different CCF-B, which will be followed throughout the document for CAPIF interconnection scenario.

* * * First Change * * * *

## 4.X	Security requirements on the CAPIF-6/6e reference points

The security requirements for CAPIF-6/6e reference points are:

-  	[CAPIF-SEC-4.7-a] The transport of messages over the CAPIF-6 and CAPIF-6e reference points shall be integrity protected.

-	[CAPIF-SEC-4.7-b] The transport of messages over the CAPIF-6 and CAPIF-6e reference points shall be protected from replay attacks.

-	[CAPIF-SEC-4.7-c] The transport of messages over the CAPIF-6 and CAPIF-6e reference points shall be confidentiality protected.

-	[CAPIF-SEC-4.7-d] The designated CAPIF core function of the CAPIF provider A and the CAPIF core function of the CAPIF provider B shall be able to mutually authenticate each other over CAPIF-6/6e reference points.

## 4.8	Security requirements on the CAPIF-7/7e reference points

The security requirements for CAPIF-7/7e reference points are:

-	[CAPIF-SEC-4.8-a] The transport of messages over the CAPIF-7 and CAPIF-7e reference points shall be integrity protected.

-	[CAPIF-SEC-4.8-b] The transport of messages over the CAPIF-7 and CAPIF-7e reference points shall be protected from replay attacks.

-	[CAPIF-SEC-4.8-c] The transport of messages over the CAPIF-7 and CAPIF-7e reference points shall be confidentiality protected.

-	[CAPIF-SEC-4.8-d] Privacy of the 3GPP user over the CAPIF-7 and CAPIF-7e reference points shall be protected.

-	[CAPIF-SEC-4.8-e] The API exposing function (destination AEF handling service API) shall determine whether AEF that is topology hiding entity, is authorized to access service API.

## 4.9	Security requirements on the CAPIF-8 reference points

CAPIF-8 interface is not in scope of the present document. Nevertheless, integrity and confidentiality protection, protection against replay attacks, privacy of the resource owner, authentication between the resource owner and the CCF need to be addressed by mechanism(s) which are out of 3GPP scope.

* * * Next Change * * * *

## 5.1	General functional security model

Figure 5.1-1 shows the functional security model for the CAPIF architecture. The interfaces CAPIF-1, CAPIF-1e, CAPIF-2, CAPIF-2e, CAPIF-3, CAPIF-4, CAPIF-5, CAPIF-6, CAPIF-3e, CAPIF-4e, CAPIF-5e, CAPIF-6e, CAPIF-7 and CAPIF-7e are defined in 3GPP TS 23.222 [3] and support the CAPIF functionality defined in 3GPP TS 23.222 [3]. CAPIF-1, CAPIF-2, CAPIF-3, CAPIF-4, CAPIF-5, CAPIF-6 and CAPIF-7 are interfaces that lie within the PLMN trust domain while the CAPIF-1e, CAPIF-2e, CAPIF-3e, CAPIF-4e, CAPIF-5e, CAPIF-6e and CAPIF-7e interfaces are CAPIF core(s) and AEF access points for API Invokers outside of the PLMN trust domain.

Security for the CAPIF-1, CAPIF-2, CAPIF-3, CAPIF-4, CAPIF-5, CAPIF-6 and CAPIF-7 interfaces support TLS and are defined in subclauses 6.2, 6.4 and 6.6 of the present document. Security for the CAPIF-1e, CAPIF-2e and CAPIF-7e interfaces support TLS, and are defined in subclause 6.3, subclause 6.5, and subclause 6.9 respectively.

Security for the CAPIF-3e, CAPIF-4e, CAPIF-5e and CAPIF-6e interfaces support NDS/IP security to secure communication between different IP security domains. This avoids multiple secure connections between API provider domain and CAPIF core domain by leveraging the NDS/IP security procedures specified in TS 33.210 [2].

* * * Next Change * * * *

## 6.X	Security procedures for CAPIF interconnection

### 6.X.1	General

The CAPIF provider A and CAPIF provider B host the CAPIF in their trust domains as specified in clause 6.2.2 in TS 23.222 [xx]. The designated CAPIF core function of the CAPIF provider A interconnects with the designated CAPIF core function of the CAPIF provider B over CAPIF-6/6e interface.

It is assumed that the API invoker is onboarded to CCF-A of the CAPIF provider A and the target AEF is registered to CCF-B of CAPIF provider B.

### 6.X.2	Security method negotiation

For security method negotiation procedure in CAPIF interconnection, clause 6.3.1.2 shall be followed with the following enhancement:

- API invoker shall send the security method request to the CCF-A. The API invoker shall optionally include the CCF ID information in the security method negotiation request to the CCF-A.

- Based on the received CCF ID information the CCF-A shall forward the security method negotiation request to the target CCF.

- CCF-A shall share the supported security method(s) by the AEF to the API invoker in the security method response message.

### 6.X.3	Authentication and authorization procedure

For the mutual authentication between the API invoker onboarded to a designated CAPIF core function (CCF-A) in CAPIF provider A and AEF registered to the CAPIF core function in CAPIF provider B, the procedures as defined in clause 6.5.2 shall be followed with the enhancements as specified in clause 6.X.3.1 and 6.X.3.2.

#### 6.X.3.1	Method 1 or 2: TLS-PSK or PKI

The API invoker shall include the designated CCF-ID information in the authentication initiation request message sent to the target AEF in CAPIF provider domain B.

When the CCF-B receives authentication initiation request message from the AEF, the CCF-B requests the security information (AEFPSK/root CA) from CCF-A over CAPIF-6/6e reference point based on the received CCF-ID.

#### 6.X.3.2		Method 3: TLS with OAuth Token

When API invoker sends the access token request, the CCF-A determines that the service API requested is not provided by the AEFs in its domain, then the CCF-A forwards the request to the CCF-B. The CCF-B provides an access token to the API invoker via CCF-A as specified in clause 6.5.2.3. The access token is specific to the AEF, which is providing services in domain B and signed by the CCF-B as per IETF RFC 7515 [zz]. The AEF validates the access token using CCF-B’s certificate.

* * * End of Changes * * * *