3GPP TSG-SA3 Meeting #120	S3-250835

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi Technology Spain S.L

Title:	Pseudo-CR on Conclusion for KI#4 in TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This pCR proposes to conclude KI#4 for FS_AIOT_SEC in TR 33.713.

* * * First Change * * * *

7.X	Conclusion to Key Issue#4 on Protection of information during AIoT service communication

The following aspects are agreed for the protection AIoT information carried in the command procedure:

-  The AIoTF acts as the security termination point for information protection in case of network layer security.

-  The information transfer for an AIoT device or each AIoT device among a group of AIoT device can be confidentiality, integrity and replay protected based on the pre-shared key and freshness parameter or session key derived from them.

NOTE 1: Details of input parameters for confidentiality, integrity and replay protection additional are to be resolved during normative phase.

-  The device security capabilities are stored in the AIoT device profile data. Therefore, there is no need for security capability negotiation.

* * * End of Changes * * * *