3GPP TSG-SA3 Meeting #120	S3-250841

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi Technology Spain S.L

Title:	Security flows for 5G ProSe Multi-hop UE-to-Network Relay Discovery

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Ph3_Sec

Comments

Based on the conclusion in TR 33.743, it is proposed to define the security procedure for multi-hop UE-to-Network Relay discovery in TS 33.503.

* * * First Change * * * *

6.1.3.X.3	Security flows for 5G ProSe Multi-hop UE-to-Network Relay Discovery

6.1.3.X.3.1	5G ProSe multi-hop UE-to-Network Relay Discovery with Model A

Figure 6.1.3.X.3.1-1: Security procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model A

- The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay’s HPLMN respectively, based on the procedure specified in clause 6.1.3.2 of the present document.

The 5G ProSe Remote UE is provisioned with multiple sets of discovery security materials and the associated HPLMN IDs of the potential Intermediate UE-to-Network Relays.

The Intermediate UE-to-Network Relay is provisioned with the one set of discovery security materials associated with its own HPLMN and multiple sets of discovery security materials and the associated HPLMN IDs of the potential Intermediate UE-to-Network Relays/5G ProSe UE-to-Network Relays.

The 5G ProSe UE-to-Network Relay is provisioned with one set of discovery security materials associated with its own HPLMN.

The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of Multi-hop UE-to-Network Relay Discovery Announcement.

- The 5G ProSe UE-to-Network Relay shall protect the Multi-hop UE-to-Network Relay Discovery Announcement using the provisioned discovery security materials as specified in clause 6.1.3.X.Y of the present document. The Multi-hop UE-to-Network Relay Discovery Announcement shall include the HPLMN ID of 5G ProSe UE-to-Network Relay in cleartext to identify the discovery security materials.

3a.	 On receiving the Multi-hop UE-to-Network Relay Discovery Announcement, the Intermediate UE-to-Network Relay shall decide the discovery security materials based on the received HPLMN ID and process the received message as specified in clause 6.1.3.X.Y of the present document.

3b.	 If the verification is successful and the Intermediate UE-to-Network Relay does not have a PC5 link with the 5G ProSe UE-to-Network Relay, based on RSC configuration, the Intermediate UE-to-Network Relay may establish a secure PC5 link with the 5G ProSe UE-to-Network Relay before sending the updated Multi-hop UE-to-Network Relay Discovery Announcement. The CP-based/UP-based link establishment procedure specified in clause 6.3.3 of present document can be reused.

- The Intermediate UE-to-Network Relay shall protect the Multi-hop UE-to-Network Relay Discovery Announcement using the discovery security materials associated with its own HPLMN as specified in clause 6.1.3.X.Y of the present document. The Multi-hop UE-to-Network Relay Discovery Announcement shall include the HPLMN ID of Intermediate UE-to-Network Relay in cleartext to identify the discovery security materials.

On receiving the Multi-hop UE-to-Network Relay Discovery Announcement, the 5G ProSe Remote UE shall decide the discovery security materials based on the received HPLMN ID and process the received message as specified in clause 6.1.3.X.Y of the present document.

6.1.3.X.3.2	5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

Figure 6.1.3.X.3.2-1: Security procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

- The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay’s HPLMN respectively, based on the procedure specified in clause 6.1.3.2 of the present document.

The 5G ProSe Remote UE is provisioned with multiple sets of discovery security materials and the associated HPLMN IDs of the potential Intermediate UE-to-Network Relays.

The Intermediate UE-to-Network Relay is provisioned with the one set of discovery security materials associated with its own HPLMN and multiple sets of discovery security materials and the associated HPLMN IDs of the potential Intermediate UE-to-Network Relays/5G ProSe UE-to-Network Relays.

The 5G ProSe UE-to-Network Relay is provisioned with one set of discovery security materials associated with its own HPLMN.

The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of Multi-hop UE-to-Network Relay Discovery Announcement.

- The 5G ProSe Remote UE shall protect the Multi-hop UE-to-Network Relay Discovery Solicitation using the discovery security materials of potential Intermediate UE-to-Network Relay as specified in clause 6.1.3.X.Y of the present document.

- On receiving the Multi-hop UE-to-Network Relay Discovery Solicitation, the Intermediate UE-to-Network Relay shall process the received message using the discovery security materials associated with its own HPLMN as specified in clause 6.1.3.X.Y of the present document.

If the verification is successful, the Intermediate UE-to-Network Relay shall protect the updated Multi-hop UE-to-Network Relay Discovery Solicitation using the discovery security materials of potential Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay as specified in clause 6.1.3.X.Y of the present document.

- On receiving the Multi-hop UE-to-Network Relay Discovery Solicitation, the 5G ProSe UE-to-Network Relay shall process the received message using the provisioned discovery security materials as specified in clause 6.1.3.X.Y of the present document.

If the verification is successful, the 5G ProSe UE-to-Network Relay shall protect the Multi-hop UE-to-Network Relay Discovery Response using the provisioned discovery security materials as specified in clause 6.1.3.X.Y of the present document. The Multi-hop UE-to-Network Relay Discovery Response shall include the HPLMN ID of 5G ProSe UE-to-Network Relay in cleartext to identify the discovery security materials.

- On receiving the Multi-hop UE-to-Network Relay Discovery Response, the Intermediate UE-to-Network Relay shall decide the discovery security materials based on the received HPLMN ID and process the received message as specified in clause 6.1.3.X.Y of the present document.

If the verification is successful, the Intermediate UE-to-Network Relay shall protect the updated Multi-hop UE-to-Network Relay Discovery Response using the discovery security materials associated with its own HPLMN as specified in clause 6.1.3.X.Y of the present document. The Multi-hop UE-to-Network Relay Discovery Response shall include the HPLMN ID of Intermediate UE-to-Network Relay in cleartext to identify the discovery security materials.

- On receiving the Multi-hop UE-to-Network Relay Discovery Response, the 5G ProSe Remote UE shall decide the discovery security materials based on the received HPLMN ID and process the received message as specified in clause 6.1.3.X.Y of the present document.

* * * End of Changes * * * *