3GPP TSG-SA3 Meeting #120	S3-250857

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi

Title:	Pseudo-CR on TR 33.700-22 KI#1.1 conclusion update

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TR 33.700-22

Version:	0.5.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This contribution proposes to update TR 33.700-22 KI#1.1 conclusion.

Since CAPIF-8 is used to enable resource owner (i.e., human) to communicate with the CCF, the CCF needs to authenticate the resource owner rather than the ROF. Therefore, the following EN is removed.

Editor’s Note: The conclusion for authentication of the ROF is ffs.

* * * First Change * * * *

7.1.1	Conclusions for KI#1.1 CAPIF-8 reference point

Normative work is recommended to protect the CAPIF-8 reference point based on the following principles:

The authentication of the CCF is based on TLS using the CCF’s certificate.

The authentication of ROF is left to implementation.

Editor’s Note: The conclusion for authentication of the ROF is ffs.

The TLS secure channel is used to provide messages exchanged between the ROF and the CCF with integrity protection, confidentiality protection and replay protection.

* * * End of Change * * * *