3GPP TSG-SA3 Meeting #120	S3-250884

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Adding Temporary ID based mechanism draftCR of MEC ph3

Document for:	Approval

Agenda item:	4.17

Spec:	3GPP TS 33.558

Version:	18.2.0

Work Item:	EDGE_Ph3

Comments

This contribution proposes to add the Temporary ID based mechanism on the ID verification concluded in the clause 7.2 of TR 33.749.

*** Start of The First Change ***

Z.x	Temporary ID based mechanism

Z.x.1	Introduction

This mechanism proposes to use temporary ID to addresses the security requirements illustrated in clause 6.Y.

The main design principle of the mechanism is based on the utilization of a temporary identifier (ID) as UE identity to fetch the Application Function (AF) specific UE Id from Nnef_UEId service. The temporary ID is generated by the 5G Core (e.g., UDM), it includes a randomly generated value, and it is associated to the PDU session (eventually to the AF ID).

Z.x.2	Mechanism details

Figure Z.3.2-1 illustrates the procedure to fetch the 5G system UE Id and privacy related information.

Figure Z.3.2-1: Procedure to fetch the 5G system UE Id and privacy related information

Precondition:

0. The EEC requests a Temporary ID to UE through interface provided by UE module.

1. PDU session establishment / modification request is sent from the UE to the 5G system.

2. The 5G Core (e.g., UDM) generates (per UE) an internal temporary ID (with a randomly generated part) for PDU session.

NOTE 1: The UE could request multiple temporary IDs during the same PDU session (PDU Session modification). The UE can use this to invalidate previous IDs which might have been compromised and/or at fix intervals without initializing another PDU session establishment/modification.

NOTE 2: The Temporary ID consists of a random part and an identifier of the UDM (UDM group ID) that generated the Temporary ID.

NOTE 3: The Temporary ID could also be generated from SMF and sent to UDM/UDR for storage.

3. The 5G Core returns the internal temporary ID to the UE as part of the NAS PDU session establishment/modification response.

4. The EEC uses the allocated temporary ID, received from the UE, inside the Edge application layer communication with the application function AF (e.g., EAS).

5. The AF uses the received temporary ID to invoke the Nnef_UEId API intended to fetch the 5G system UE identifier, i.e., AF specific UE Id, as per the procedure described in TS 23.502 figure 4.15.10-1 [8].

6. The NEF verifies the temporary ID, authorizes the request coming from the AF and accordingly retrieves from the 5G Core the AF specific UE Id corresponding to the temporary ID.

NOTE 4: NEF retrieves the correct UDM instance from the UDM identifier in the Temporary ID, and NEF queries UDM to obtain the UE ID from the temporary ID.

7. The NEF replies to the AF with the corresponding AF specific UE Id.

8-9 (Optional). Assuming the application logic requires to fetch the UE location, a location request is sent to the NEF using the AF specific UE Id, and the corresponding information is provided by the 5G system through the NEF.

10. The AF responds to the UE (EEC).

*** End of The First Change ***