3GPP TSG-SA3 Meeting #120	S3-250892

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Update conclusion for KI3 Security and privacy aspects of IMS DC capability exposure

Document for:	Approval

Agenda item:	5.2

Spec:	3GPP TR 33.790

Version:	1.1.0

Work Item:	 FS_NG_RTC_SEC_Ph2

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

This pCR proposes updating conclusion of KI#3 on Security and privacy aspects of IMS DC capability exposure.

SA2 endorsed a CR (S2-2501099) for detailed call flows of adding application data channel(s) to an existing IMS session for A2P, P2P, and P2A2P DC. A note is added which mentioned Privacy and security aspects regarding adding, removing, updating data channels by the network are under SA3 responsibility.

According to workflows in Annex AG.2.1 of the CR, especially AG.2.1.Y	(Adding P2P application data channel to an existing IMS session), the subscriber of the UE cannot be aware of that the DC adding is initiated by the DC application server (DC AS) instead of the opposite UE, hence may be misled to accept the DC adding/establishment request but the user may opt-out of connection if connection request is come from DC AS. Similarly, the user may accept the DC terminating/update request from the DC AS without aware of originator of the request. This can lead to privacy violation without explicit user consent, ethical concern due to lack of transparency, then legal consequence issue and loss of user trust, probably security risks with potential misusing also.

It’s essential to consider transparency, consent, compliance, user autonomy and control, and security when expose IMS DC capability to DC AS and authorize the DC session control request from the DC AS.

This contribution proposes consideration of transparency, user consent and control when adding, removing, updating data channels by the network.

Proposed Changes

* * * Next Change * * * *

<Proposed change in revision marks>

## 7.3		Conclusions for Key Issue #3: Security and privacy aspects of IMS DC capability exposure

It is concluded to reuse authentication and authorization procedure specified in clause 12 of TS 33.501 [11].

In addition, to avoid privacy violation, ethical concern, legal and security risks, transparency, user consent and control shall be considered when adding, removing, updating data channels by the network. For example, the user shall be aware that the DC adding, removing, updating request is initiated by the DC AS instead of the opposite UE, and explicit user consent shall be obtained.

Editor's Note: Whether more granular authorization is required is FFS.

* * * End of Changes * * * *